3GPP RAN WG2 Meeting #129	R2-2500655

# Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.6

Source:	HONOR

Title:	Discussion on XR rate control

Document for:	Discussion and Decision

1. Introduction

In RAN#105 meeting, the following objective of Rel-19 XR work item was agreed [1]:

Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In RAN2#128 meeting, the following agreements on XR rate control have been achieved [2].

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE. - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

In this contribution, we discuss the detail of the rate indication, assistance from UE/CN to gNB, and whether to support rate query MAC CE.

2. Discussion

2.1. Detail of the rate indication

Rate indication based on QoS flow level has been agreed in last RAN2 meeting. Two options have been summarised for the detail of the rate indication.

Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

The difference between the two options can be seen by the following example. Given that three QoS flow (e.g., QFI1, QF12 and QFI3), all of which can be throttled, are mapping into one DRB. When congestion occurs, the behaviours of the two options are as follows.

For the option1, based on congestion level, gNB can indicate some, not all, of the QoS flows that need to be throttled, with an appropriate recommended rate for each flow. For example, if the congestion can be relieved only by decreasing rate of QFI2 and QFI3, gNB does not need to indicate QFI1 in the MAC CE.

For the option2, all the adjustable QoS flows (QFI1, QF12 and QFI3) configured by RRC need to adapt to the new rate indicated by gNB, which seems not flexible. The rate of each QoS flow is up to UE implementation. In addition, UE itself knows which QoS flows can be throttled, making the configuration by RRC seems unnecessary.

Thus, the option1 is more flexible and accurate to indicate a new rate per QoS flow when congestion status changes. We slightly prefer option1.

Proposal 1: Use MAC CE to indicate QoS flows that need to be throttled.

2.2. Assistance from UE/CN to gNB

Since SA2 is now working on available data rate exposure from gNB to AF based on QoS flow level, which may mean that gNB needs to be indicated which QoS flows can be throttled. We can ask SA2 that if it has already provided or intends to provide such assistance information to gNB. If the reply is negative, gNB gets the assistance information from UE.

Proposal 2: Ask SA2 if it has already provided or intends to provide gNB with the assistance information about which QoS flows can be throttled.

Furthermore, if what gNB knows is merely whether one QoS flow can be throttled, it may send useless rate indication under certain conditions. Let’s give an example to illustrate the issue.   For simplicity, assume only one service is included in the XR application.

AF usually triggers CN to set up QoS flow after UE and AF negotiate encoding parameters. Assume the encoding rate set after UE and AF negotiated are {40Mbit/s, 30Mbit/s, 16Mbit/s}, which means the corresponding QoS flow has three adjustable rates {40Mbit/s, 30Mbit/s, 16Mbit/s}.

Take the using encoding rate is 16Mbit/s as an example, when congestion occurs, since gNB does not know the rate set that both supported by UE and AF, it may indicate a rate that lower than 16Mbit/s. As UE does not have the corresponding encoding rate to fit the indicated rate, it may do nothing but discard this rate indication. On the other hand, if the rate indicated by gNB is between 30Mbit/s and 16Mbit/s when congestion is relieved, UE may discard the rate indication for the same reason.

In conclusion, if the assistance information indicated by UE/CN includes the rate set that both UE and AF support, gNB can appropriately send the rate indication.

Observation1: If what gNB knows is merely whether one QoS flow can be throttled, it may improperly send rate indication under certain conditions.

Proposal 3: Rate set that both UE and AF support should be included in the assistance information indicated by UE/CN.

2.3. Whether to support rate query MAC CE

Some companies argue that ANBRQ (Access Network Bitrate Recommendation Query, i.e. Recommended bit rate query MAC CE) should also be introduced into XR rate control mechanism. However, we believe that there is no such rate query scenario for XR rate control. Two scenarios for the usage of ANBRQ are described in section 10.7.3.2 of TS 26.114, as copied below.

Figure 1 Uplink bitrate increase based on ANBR and ANBRQ

In step 4 in the figure 1, AN-1(Access Network) indicates UE-1 to increase UL rate to R2. UE-1 needs to know whether peer AN-2 supports to increase DL rate to R2. UE-2 sends ANBRQ (with value R2) to AN-2 and notifies UE-1 of the Recommended UL bit rate R3(<R2) based on the query result.

Figure 2 Downlink bitrate increase based on ANBR and ANBRQ

In step 4 in the figure 2, AN-1(Access Network) indicates UE-1 to increase DL rate to R1. UE-1 requests peer UE-2 to increase UL rate to R1. UE-2 sends ANBRQ (with value R1) to AN-2 and adjusts the encoding rate to R2(<R1) based on the query result.

Thus, when AN indicates UE to increase DL/UL rate, ANBRQ is used to confirm whether the peer AN support the new recommended UL/DL rate, which is a kind of end-to-end rate control. However, both parties involved in XR service are UE and AF. When AN indicates UE to increase UL rate, UE can directly increase the encoding rate without confirmation from AF. Therefore, we have not observed scenario involving rate query in XR rate control.

Proposal 4: Do not support UL MAC CE rate query.

3. Conclusions

This contribution provides the following proposals.

Proposal 1: Use MAC CE to indicate QoS flows that need to be throttled.

Proposal 2: Ask SA2 if it has already provided or intends to provide gNB with the assistance information about which QoS flows can be throttled.

Observation1: If what gNB knows is merely whether one QoS flow can be throttled, it may improperly send rate indication under certain conditions.

Proposal 3: Rate set that both UE and AF support should be included in the assistance information indicated by UE/CN.

Proposal 4: Do not support UL MAC CE rate query.

4. References

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Report of 3GPP TSG RAN WG2 meeting #128