3GPP TSG-RAN WG2#129											      R2-2500662

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.18

Source: 	NEC

Title:	        SDT enhancement for DL bandwidth restriction

Document for:	 Discussion and Decision

- Introduction

In RAN2 #128 [1], DL bandwidth restriction issue for SDT was discussed as below.

- RAN2 will address this issue.  FFS the exact solution

In this contribution, we will discuss this issue and provide some possible solving schemes.

- Discussion

During SDT procedure, DCI 1_0 can be used for scheduling PDSCH receptions. The indicated FDRA field size in DCI 1_0 is calculated based on the RB number of CORESET 0 if CORESET0 is configured as below [2].

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats - 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment - bits where is given by Clause 7.3.1.0 Step 0: - Determine DCI format 0_0 monitored in a common search space according to clause 7.3.1.1.1 where is the size of the initial UL bandwidth part. - Determine DCI format 1_0 monitored in a common search space according to clause 7.3.1.2.1 where is given by - the size of CORESET 0 if CORESET 0 is configured for the cell; and - the size of initial DL bandwidth part if CORESET 0 is not configured for the cell. Step 1: - Determine DCI format 0_0 monitored in a UE-specific search space according to clause 7.3.1.1.1 where is the size of the active UL bandwidth part. - Determine DCI format 1_0 monitored in a UE-specific search space according to clause 7.3.1.2.1 where is the size of the active DL bandwidth part.

When the larger DL data arrives during SDT procedure, transmission of DL data will consume a longer time due to CORESET 0 restriction, which is not efficient. Hence, the scheduled bandwidth size for DL transmission is restricted for SDT procedure.

In the last meeting, this issue was discussed and agreed to be addressed. Some possible options are listed below based on the last discussion:

- Option 1: removing CORESET 0 restriction for initial DL BWP

- Option 2: introducing CQI reporting in Msg3 or MSGA

- Option 3: introducing CFR or separate initial DL BWP

For option 1, if CORESET 0 restriction for initial DL BWP is removed, up to the whole initial DL BWP bandwidth may be used for scheduling of DL data during SDT procedure. Of course, removing CORESET 0 restriction for initial DL BWP can have some specification impacts, such as DCI size misalignment.

For option 2, since the restriction of COREST 0 for initial DL BWP still exists, CQI reporting only achieves a limited gain and cannot solve the original issue. Besides, this option also brings some specification impacts, for example, how to request CQI report and how to report CQI.

For option 3, a new frequency domain resource such as CFR/separate initial DL BWP needs to be configured. If a new resource is introduced, some corresponding issues also need to be considered, for example, whether this new resource may be used for transmitting SIB, paging, performing random access procedure, and so on. Besides, DCI size misalignment issue also needs to be considered due to the introduced larger frequency domain resource when CSS is used.

According to the analysis of the three options above, we can see each option has its pros and cons. From the perspective of scheme effectiveness and specification impacts, we prefer option 1, i.e. removing CORESET 0 restriction for initial DL BWP during SDT procedure. Then if CORESET 0 restriction is removed, DCI size misalignment needs to be considered. Hence, RAN2 needs to send an LS to ask RAN1 whether removing this restriction is feasible.

Proposal 1: Remove CORESET 0 restriction for initial DL BWP during SDT procedure for MT-SDT.

Proposal 2: Send an LS to ask RAN1 whether removing CORESET 0 restriction for initial DL BWP during SDT procedure is feasible.

- Conclusion

This contribution provides our analysis and views for the SDT enhancement, the following proposals are listed below:

Proposal 1: Remove CORESET 0 restriction for initial DL BWP during SDT procedure for MT-SDT.

Proposal 2: Send an LS to ask RAN1 whether removing CORESET 0 restriction for initial DL BWP during SDT procedure is feasible.

- References

[1] Draft_RAN2_128_Meeting_Report_v1.

[2] TS 38.212 i50.