3GPP TSG-RAN WG2 #129	R2-2500668

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.6.3

Source: 	Ericsson

Title:  	Discussions on event-based L1 measurements

Document for:	Discussion and Decision

# Introduction

Since last meeting the first draft for the running CR for MAC has been shared by the rapporteur which triggers some discussions around design alternatives. This contribution brings up our views on these.

# Discussion

## Aspect on the truncated MAC CE

During RAN2#128 the following agreement was taken:

RAN2#128 Agreement: Support the truncated measurement report MAC CE

What the agreement means is not totally clear since truncated can be interpreted in different ways. However, from the post-meeting discussions on the running CR it seems like the general understanding is that the UE can choose to not include measurements from all beams. In the current MAC spec for other truncated MAC CEs this means that the remaining information will be discarded if the grant available is not big enough to fit the entire measurement report. However, this may not be a good practice for the event-triggered L1 measurement report MAC CE, as this MAC CE is used for something that is a bit different from the existing MAC CEs which have a truncated format.

The existing agreement “Support the truncated measurement report MAC CE” is not clear about what is the use of the truncated measurement report MAC CE.

A first aspect which should be clarified is when the UE would send the truncated measurement report MAC CE. A straightforward solution for this issue is that the UE may decide to use a truncated measurement report MAC CE only in case the available grant is not able to fit the overall measurements available. In this case, if the grant is too small, the UE decides to either send an SR to request a bigger grant or to use to send the truncated measurement report MAC CE. Therefore, we propose:

The UE sends a truncated measurement report MAC CE only if the available grant is too small for a normal measurement report MAC CE and no SR (to ask for a bigger grant) is sent.

Another aspect to clarify is what the UE should do with the available measurements in case the truncated measurement report MAC CE is used. Given that the UE may have available much more measurements than the one reported within the truncated measurement report MAC CE, the UE eventually could send multiple reports to the network as far as the network is aware that more measurement report may be coming. According to this, we propose:

If a truncated measurement report MAC CE is sent, the UE can send the remaining measurements in subsequent measurement report MAC CE. Whether multiple measurement report MAC CEs are sent to the network, this is indicated by the UE within the (truncated) measurement report MAC CE.

## Fulfilled conditions and event reporting

During RAN2#128 there was a discussion on how to include beams that does and does not fulfill an event condition, and how they should be reported in a measurement report.

RAN2#128 agreement: For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE. For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

According to what has been agreed, the network indicate to the UE which can include beams which had not fulfilled an event condition in the measurement report MAC CE, but at the moment there are no means for the network to identify those beams. With the current agreement, this means that the network may take wrong mobility decision by handing over a UE to a beam which may not be particularly good.

In we want to solve this, two possible solutions can be adopted to indicate those beams in the measurement report MAC CE that have not fulfilled an event condition:

- Report in the measurement report MAC CE how many beams have fulfilled the event condition, and UE indicates which is the number of this beams.

- Each reported beam has a flag which indicate whether this beam has fulfilled the event condition or not.

Generally speaking, both solutions work and RAN2 can simply decide which one to adopt. Therefore, we propose:

RAN2 agrees to either one of the following approaches to indicate in the measurement report MAC CE the beams which have not fulfilled the event condition:

  - The UE reports in the measurement report MAC CE how many beams (among those included in the report) have fulfilled the event condition. The beams which have fulfilled the event conditions are included at the beginning of the MAC CE.

  - The UE, for each reported beam. includes a flag which indicates whether this beam has fulfilled the event condition or not.

Finally, a further aspect that needs to be considered for the case when the UE report beams which have not fulfilled the event condition, is that those beams may have a really bad RSRP quality. In this case reporting this kind of beams will just increase the size unnecessarily of the measurement report and also will provide not useful information to the network. In this case, it would be good to limit which beams that have not fulfilled the event condition and, to do this, a threshold may be used. In this case, the UE would report the beams which have not fulfilled the event condition only if the measured RSRP on those beams is above a configured threshold. Therefore, we propose:

The UE includes (if configured by the network) in the measurement report MAC CE beams which have not fulfilled the event condition, only if the measured RSRP on those beam is above a threshold (which is also configured by the network).

## SSBRI and differential reporting

RAN2#128 agreement: SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

With the agreement in the last meeting to use SSBRI/CRI as the beam identifier, an issue which we earlier tried to highlight become apparent. To maintain one MAC CE format for both FR1 and FR2 the format has to address 8 candidate cells with 64 beams each, which adds up to 9 bits to represent the total number of 512 combinations of cell and beam. On top of this we need 7 bits to report the full measurement quantity (RSRP). For this reason, the sum is 16 bits for a beam which leaves no room for spare bits to report e.g. fulfillment of event conditions, leaving conditions or any other information, as can be seen in Figure 1, and also visible in the running CR.

| SSBRI (9 bits) |  |
| --- | --- |
|  | RSRP (7 bits) |

Figure 1; SSBRI and full RSRP

It is possible to enhance the MAC CE with the differential encoding, similar as has been done in R18, this comes however with some drawbacks since the reporting pattern is restricted and the granularity is reduced. Differential encoding is however considered necessary for the needs of additional beam information required as indication bits (leaving conditions, fulfillment of event criteria etc.).

Differential RSRP reporting seems necessary to make room for “per beam” indications in the MR MAC CE.

RAN2 considers the differential RSRP encoding as baseline to enable “per beam” indication bits.

By enabling a “per beam” reporting, according to our earlier proposals to identify the beam by the candidate configuration, a number of spare bits would be available for the octets describing a beam, see example in Figure 2.

| SSBRI (9 bits) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | R | R | R | Diff RSRP (4 bits) |

Figure 2; SSBRI and differential RSRP

However, regardless on the availability of spare bits “per beam”, it would be good to report additional information in the MAC CE, which now are missing, like the following:

- Beam has triggered the measurement report

- Beam has fulfilled the event condition

- Beam has fulfilled the leaving condition

To deliver this information, we propose to deliver each of these by different information. Given that differential RSRP requires the highest RSRP to be reported first, the beam with the highest RSRP needs to be first in the order, regardless of if it triggered the event or not.

Differential RSRP requires the highest RSRP to be reported first.

If differential RSRP is used, the first beam in the measurement report MAC CE is the one with the highest measurement quantity.

To save on information bits, it could be noticed that the most likely scenario is that exactly one beam fulfils the measurement conditions and time to trigger. A case where two or more beams achieve this at the exact time seems rare or unlikely. One exception is if we choose to include beams that fulfils the event condition but have not yet fulfilled the time to trigger. Regardless of which way the discussions in RAN2 goes, we propose to let one spare bit indicate that the specific beam triggered the report. We realize that for all the other beams, this will most likely mean that the bit is waste, but to not complicate the MAC CE, we still think it is the best solution.

The beam that triggered the measurement report MAC CE is indicated explicitly.

RAN2 has agreed to also deliver information to the network if a specific beam fulfilled the measurement conditions and if it has fulfilled the leaving condition.

If a beam included in the measurement report MAC CE has met the leaving condition of the event, this is indicated explicitly.

According to what is propose above, a MAC CE could look like below in Figure 3, where T would indicate if this beam triggered the measurement report, F indicating if the beam fulfilled or not the event condition and L would indicate if the beam fulfilled the leaving condition.

| Report ID |  |  |  |  |  | T | F |  | Oct 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSBRI (9 bits) |  |  |  |  |  |  |  |  |  |
|  |  | RSRPstrongest (7 bits) |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| SSBRIN (9 bits) |  |  |  |  |  |  |  |  | Beam N |
|  | TN |  | FN | LN | RSRPN (4 bits) |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| SSBRIN+1 (9 bits) |  |  |  |  |  |  |  |  | Beam N+1 |
|  | TN+1 |  | FN+1 | LN+1 | RSRPN+1 (4 bits) |  |  |  |  |

Figure 3; Example of per beam reporting

# Conclusion

In the previous sections we made the following observations:

Observation 1	The existing agreement “Support the truncated measurement report MAC CE” is not clear about what is the use of the truncated measurement report MAC CE.

Observation 2	Differential RSRP reporting seems necessary to make room for “per beam” indications in the MR MAC CE.

Observation 3	Differential RSRP requires the highest RSRP to be reported first.

Observation 4	RAN2 has agreed to also deliver information to the network if a specific beam fulfilled the measurement conditions and if it has fulfilled the leaving condition.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The UE sends a truncated measurement report MAC CE only if the available grant is too small for a normal measurement report MAC CE and no SR (to ask for a bigger grant) is sent.

Proposal 2	If a truncated measurement report MAC CE is sent, the UE can send the remaining measurements in subsequent measurement report MAC CE. Whether multiple measurement report MAC CEs are sent to the network, this is indicated by the UE within the (truncated) measurement report MAC CE.

Proposal 3	RAN2 agrees to either one of the following approaches to indicate in the measurement report MAC CE the beams which have not fulfilled the event condition:

a.	The UE reports in the measurement report MAC CE how many beams (among those included in the report) have fulfilled the event condition. The beams which have fulfilled the event conditions are included at the beginning of the MAC CE.

b.	The UE, for each reported beam. includes a flag which indicates whether this beam has fulfilled the event condition or not.

Proposal 4	The UE includes (if configured by the network) in the measurement report MAC CE beams which have not fulfilled the event condition, only if the measured RSRP on those beam is above a threshold (which is also configured by the network).

Proposal 5	RAN2 considers the differential RSRP encoding as baseline to enable “per beam” indication bits.

Proposal 6	If differential RSRP is used, the first beam in the measurement report MAC CE is the one with the highest measurement quantity.

Proposal 7	The beam that triggered the measurement report MAC CE is indicated explicitly.

Proposal 8	If a beam included in the measurement report MAC CE has met the leaving condition of the event, this is indicated explicitly.