3GPP TSG-RAN WG2#129												R2-2500686

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.9.2 Support of Store & Forward

Source: 	NEC

Title:	        Radio Interface Aspect of S&F

Document for:	 Discussion

# Introduction

This is a list of main agreements on S&F topic, regarding scope ,scenario , architecture and so on:

- RAN2 will adopt SA2 agreed full CN and spit-MME payload options as the baseline for further discussion.

- S&F feature is common for NB-IOT and eMTC,

- S&F feature is applicable to both CP and UP solutions

- consider both single satellite pass and multiple satellite pass scenarios

- consider both MO and MT data within scope

- include a S&F operation indication bit in SIB1

- a Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode

In this contribution, we share our view on remaining issues

# Discussion

## Access Control

A dynamic “S&F operation” indication was agreed and sent in SIB1. The setting of this indication was agreed as below:

- When not present:

  - the NTN cell is operating in normal mode (i.e. default/”not-S&F” mode)

- When present, the “S&F operation” indication has two possible settings:

  - ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

  - ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

This means NW will modify system information upon feeder link become available or unavailable as following:

- Upon feeder link becomes unavailable and NTN cell is operating in “S&F” mode : add the indication

- upon feeder link becomes available and NTN cell is operating in “normal” mode: delete the indication

At meantime, we agreed to have time information: indicate when the current satellite operation mode will transit from “S&F operation” mode to normal mode. both time information and deleting/adding “S&F” indication are tightly relevant to feeder link availability, this is duplicated. To avoid SI modification, We propose to change the dynamic “S&F operation” indication into static “S&F cell” indication and use the time information to indicate when the feeder link is resumed and the cell could be temporarily/exceptionally in normal operation mode.

Change the dynamic “S&F operation” indication into a static “S&F cell” indication(i.e., no need to update the value when feeder link is resumed), and use Time/Time window information, to indicate when feeder link becomes available and the cell is in normal operation mode temporarily/exceptionally.

There are few FFS relevant to the time information:

- FFS what we specify in RAN2 specs for this and if we differentiate the behavior for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer).

- FFS which SIB is used (SIB1 or SIB31).

- FFS if absolute or relative time is used

- FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

We slightly prefer the time information could be placed in SIB1 together with “S&F” indication.  And absolute time can be used, because it could be long time in future in case there is only few ground stations. If P1 is agreeable, the time information could be a time duration when feeder link is available

the time information including an absolute time and a duration, is provided in SIB1

About UE behaviour or specification for time information, RAN2 assumed that a R19 UE could use this time information at least to delay some NAS procedures until the feeder link is resumed. However, as shown in following figure, in a very common scenario, e.g. a UE located at left side cannot delay NAS procedure, because the satellite will fly away at the time feeder link becomes available. There is no other choice than initiating NAS procedure in S&F mode.  Only in the very rare case where a UE is in a specific location e.g. not far from NTN GW, so feeder link is not available at first but become available shortly after when the UE is still in the satellite coverage, this UE could delay the NAS procedure, but how long will the feeder link be available and how long will the UE be still under its coverage? Will the overlapping time be long enough for the UE to finish its NAS procedure? It might be questionable due to fast moving satellite coverage.

Figure 4.13.x.1-1 of TS23.401 CR: Store and Forward (S&F) Satellite operation

Instead, we think the time information could be used for UE to know when its MO signal/data will be forwarded to CN and new MT signal/data will be available in the same satellite/configured satellite set. it can be used for UE to delay to re-attempt to the same satellite/cell. This is similar but different to the Wait timer provided by MME. This AS time information is only relevant to current satellite’s feeder link resuming time. But the NAS wait timer provided by MME may be longer after taking into account other factors e.g., UE location, power consumption requirement etc.

The UE behavior related to the time information could be clarified:

- if the static “S&F cell “ indication is agreeable, UE will use this information to understand when feeder link is available hence the “S&F cell” is operating temporarily in normal mode

- After a UE initiates NAS procedure/data transmission, before feeder link becomes available as indicated by the time information, the data will be stored in satellite without forwarding to CN, and UE is not supposed to receive any follow-up signaling/data for the same NAS procedure or data session. Consequently, UE shall not re-attempt the NAS procedure/data session,  and should not expect paging for receiving following up signaling/data for the same session. This may overlap with “wait time” provided by NAS, but it is not necessary same, because NAS consider the whole satellite sets and other factors. UE behavior could be defined by taking both time information into account.

The time information together with “wait time” is used by UE to refrain from expecting follow-up signalling/data from network.

## MME-configured satellite ID list

There were RAN2 discussion whether a new SIB shall be introduced to provide satellite information of the MME-configured satellite/satellite set . Below a copy from CR to 23.401 is relevant to MME-configured satellites:

For a UE which indicates support of Store and Forward Satellite operation and when an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation. - If the UE is rejected with a reject cause indicating it is due to S&F operation, the UE’s EMM state shall remain unchanged. - The MME may provide to the UE a S&F Wait Timer, a S&F Monitoring List or both when accepting or rejecting a NAS procedure. NOTE 3: How the MME determines the S&F Wait Timer and S&F Monitoring List is up to MME implementation, e.g. based on feeder link (un)availability period, service link (un)availability period, UE power saving requirements, Communication Pattern parameters, UE location, UE mobility, etc. When the S&F Wait Timer expires, the UE may perform a NAS procedure, which can be a subsequent NAS procedure or a reattempt of a NAS procedure previously rejected with a S&F reject cause. The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. NOTE A: The S&F Wait Timer or S&F Monitoring List doesn’t affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE B: When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. If the MME does not send a S&F Monitoring List or S&F Wait Timer in a NAS procedure, the UE deletes any stored S&F Monitoring List or S&F Wait Timer. NOTE Y: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. - The MME may indicate to the UE the Uplink S&F estimated delivery time in a NAS accept messages (i.e. Attach Accept, TAU Accept or Service Accept messages). NOTE 4: The Uplink S&F estimated Delivery time is the estimated to deliver the data to the ground from the time the data is sent by the UE. How UE uses this information is left for UE implementation.

UE is supposed to re-attempt Attach/TAU or other signalling/data exchange over the configured satellites according above text. This means, after UE scans and camps successfully on a S&F satellite, it should check if this satellite is one of the MME configured satellites, if yes, the satellite/cell potentially holds the UE’s context and its DL data/signalling relevant to the Attach/TAU/other signalling/data sent during last connection. In this case, the UE could re-attempt the connection to continue the NAS procedure or data exchange session. Otherwise UE may not re-attempt the connection. This would only require UE to check if the serving satellite ID is one of MME provided satellite ID

Legacy serving satellite assistance information and ID could be used for UE to check against to MME-configured satellite ID(s)

MMC-provided satellite/satellite set may be used by the UE assist with saving power too. From AS point of view, it may be used for cell reselection procedure. Firstly it is clearly mentioned that UE may select/camp on “normal” satellite other than the configured ones especially during wait time, but these satellites likely belong to another PLMN. Hence legacy neighbouring satellite information should be the kept for cell reselection purpose. Secondly, the MME provided satellite/satellite set in our understanding are UE-specific rather than system/cell specific, UE may check if a MME provided satellite is in the neighbouring satellite list, consequently to refrain cell reselection measurement or encourage cell reselection measurement, however SA2 seems have intention to leave this to UE implementation. In summary, we did not see the need to provide these satellite information separately via another new SIB, other small enhancement on cell reselection procedure could be discussed after we receive replay LS from SA2.

Legacy neighbouring satellite assistance information and ID could be reused for UE to check against to MME-configured ID for cell reselection enhancement purpose, no new SI is needed

Moreover, S&F is tightly linked to discontinuous coverage feature, as stated in WID justification:

- S&F operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure (e.g.mid-sea, remote areas)

- Along with support for discontinuous coverage, support for S&F operation is key to facilitate cost-effective deployment of IoT NTN services and enable an immediate operational service with (1) sparse LEO constellations and (2) reduced ground segment infrastructure.

Considering the cases when there is no neighbour cell, for UE in RRC_IDLE, the network may provide information for operating during discontinuous coverage (e.g., ephemeris parameters, the start-time and/or carrier frequency of upcoming NTN payload coverage). And UE shall use these discontinuous coverage information to predict its in and out of coverage time window. If UE is out of coverage, it could suspend all AS idle procedure for power saving.

For UE in RRC_CONNECTED, we expect NW will release the UE connection before it is out of coverage. Otherwise, upon declaring RLF, UE may determine not to reestablishment the RRC connection by implementation (e.g., based on ephemeris parameters).

UE(s) who support S&F feature, support discontinuous coverage-like feature upon service link becoming unavailable and there is no neighbor cell:

- For UE in RRC_IDLE, RAN2 assume the UE suspend all AS idle mode operations related to NTN.

- For UE in RRC_CONNECTED mode, either NW will release the UE or the UE will declare RLF and enter idle mode directly.

## PUR

Transmission using PUR applicable to NB-IoT UEs currently allows pre-configured resources with periodicity ranging from 8 H-SFN to 8192 H-SFN

:PUR-PeriodicityAndOffset-r16 ::= CHOICE {

periodicity8		INTEGER (1..7),

periodicity16		INTEGER (1..15),

periodicity32		INTEGER (1..31),

periodicity64		INTEGER (1..63),

periodicity128		INTEGER (1..127),

periodicity256		INTEGER (1..255),

periodicity512		INTEGER (1..511),

periodicity1024		INTEGER (1..1023),

periodicity2048		INTEGER (1..2047),

periodicity4096		INTEGER (1..4095),

periodicity8192		INTEGER (1..8191)

}

And the TA validity, which controls by pur-TimeAlignmentTimer, can has a largest length of 8*PUR periodicity.

pur-TimeAlignmentTimer-r16		INTEGER (1..8)				OPTIONAL,	-- Need OR

Base on above information, the overall validity of PUR resources could be configured for a relative long duration however PUR validity is limited to the same cell and within pur-TimeAlignmentTimer. However in IoT-NTN scenario changing satellites may cause the release of the resources. In this case, UE needs to repeatedly release and request PUR resources, which reduces the benefits of using PUR.

In one aspect, the condition of running pur-TimeAlignmentTimer can be lifted due to self  TA compensation. In another aspect, if UE is provided by MME with satellite/satellite sets, potentially all these cells/satellite will hold UE’s context (potentially including PUR configuration) even if the UE goes to idle mode and anther cell/satellite. It means during the time gap before the same satellite or satellite from the satellite sets serves the UE again, UE can store/suspend the configured PUR resources, then resume/reuse it after reconnecting to the same satellite/satellite set.

If UE is configured with MME provided satellite/satellite sets, consider PUR resources could be resumed when the coverage of the same satellite/satellites set comes back.

# Conclusion

In summary, we would like to share our view on S&F as follows:

Proposal 1:	Change the dynamic “S&F operation” indication into a static “S&F cell” indication(i.e., no need to update the value when feeder link is resumed), and use Time/Time window information, to indicate when feeder link becomes available and the cell is in normal operation mode temporarily/exceptionally.

Proposal 2:	the time information including an absolute time and a duration, is provided in SIB1

Proposal 3:	The time information together with “wait time” is used by UE to refrain from expecting follow-up signalling/data from network.

Proposal 4:	Legacy serving satellite assistance information and ID could be used for UE to check against to MME-configured satellite ID(s)

Proposal 5:	Legacy neighbouring satellite assistance information and ID could be reused for UE to check against to MME-configured ID for cell reselection enhancement purpose, no new SI is needed

Proposal 6:	UE(s) who support S&F feature, support discontinuous coverage-like feature upon service link becoming unavailable and there is no neighbor cell:

-	For UE in RRC_IDLE, RAN2 assume the UE suspend all AS idle mode operations related to NTN.

-	For UE in RRC_CONNECTED mode, either NW will release the UE or the UE will declare RLF and enter idle mode directly.

Proposal 7:	If UE is configured with MME provided satellite/satellite sets, consider PUR resources could be resumed when the coverage of the same satellite/satellites set comes back.