3GPP TSG-RAN WG2 Meeting #129	R2-2500692

Athens, Greece, 17 - 21 Feb, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on adaptation of common signal/channel transmissions

Agenda Item:	8.5.4

Document for:	Discussion and decision

Introduction

In this contribution, we provide our considerations on adaptation of common signal/channel transmissions.

Discussion

## 2.1 Adaptation of paging

In RAN2#128, RAN2 has agreed to introduce Ns=8 and N=T/32, FFS on other values. RAN2 also discussed whether enhancement to PEI is needed, and identified several possible directions, but no conclusion was made:

Discussion: [Apple]: Separate PEI configuration for NES UEs is quite aligned with what RAN2 previously agreed on separate PO configurations for NES UEs. [ZTE]: Think it can be handled by NW implementation by using the existing PEI configuration. Do not see a real need to introduce separate PEI configuration for NES UEs. [Xiaomi]: Prefer having no agreement at all this meeting. [Huawei]: Ok with proposal 3 in general, however some wording gives confusion, we need to clearly distinguishes three case. i) single PEI configuration with separate PEI transmission, ii) single PEI configuration with single PEI transmission and iii) separate PEI configuration. [OPPO]: The intention of offline discussion was to exclude the option ii). [Nokia]: It is early to exclude case ii) now. Proposal-4: R2 assume to leave it to network implementation to set po-NumPerPEI and maxPEI-perPF properly, in order to be compatible with Ns=8 without further spec effort [Huawei]: Think proposal 4 is also dependent on the conclusion of the issue indicated by proposal 3. [ZTE]: Share the same understanding as Huawei. Introduce value for Ns=8. FFS on 16. Introduce value for N= T/32. FFS on T/64, T/128, T/256. RAN2 will revisit PEI configuration and PEI transmission aspects with the consideration of offline discussion in the next meeting.

In this contribution, we further discuss the remaining issues: 1) PEI impacts; 2) Additional Ns/N values; 3) UE capability.

- PEI impacts

PEI was introduced in R17 for UE power saving. Even though it cannot increase network energy saving gain, we think it is beneficial to make the R19 NES paging adaptation compatible with PEI, to avoid unnecessary power consumption at the UE side.

Figure 1: PEI mechanism in the existing spec

In the offline discussion, three cases have been identified:

Option 1 has fewer spec impacts, but the drawback for Option 1 is that, the PEI for legacy UEs cannot span two PFs, as follows:

The issue can be solved by Option 3, which is the most flexible option, as the NW can configure both paging parameters and PEI parameters separately for legacy UEs and R19 NES UEs.

The advantage of Option 2 is that it can save more NW energy, because fewer PEIs need to be sent compared with Option 1 and Option 3.

Observation 1: Option 3 is most flexible, the NW configures separate paging/PEI parameters for legacy UEs and R19 NES UEs, the mapping method between PO and PEI follows existing mechanism. Option 2 saves the most NW energy as it enables the NW to send fewer PEI indications. The drawback of Option 1 is that the PEI cannot be mapped to 2 PFs (which is possible in the existing spec).

Proposal 1: RAN2 to down-select from the following options:

- Option 1: single PEI configuration with separate PEI transmission;

- Option 2: single PEI configuration with single PEI transmission;

- Option 3: separate PEI configuration.

- Extended values of N and Ns

In the last meeting, it has been agreed to introduce Ns=8 and N=T/32, FFS on other values:

Introduce value for Ns=8. FFS on 16.

Introduce value for N= T/32. FFS on T/64, T/128, T/256.

In our understanding, the enhancement of paging adaptation is not at the expense of decreasing paging capacity. To maintain the same paging capacity, one additional N value comes with one additional Ns value.

Observation 2: Paging enhancements is not at the expense of reducing paging capacity, i.e. the introduction of one additional N value always comes with one additional Ns value.

Then, we consider whether the new pair (N=T/32, Ns=16) is feasible/necessary. One reason for not wanting Ns = 16 is the compatibility with PEI. If Ns is increased to 16, there would be 16 POs at most in one single PF. Considering the payload size of PEI is no larger than 41 bits and 43 bits for licensed and unlicensed spectrums respectively, it is difficult to further increase the associated POs for one PEI. For instance, if we still follow the principle of associating at most 2 PFs, 16 POs per PF means that the PEI will be associated with 32 POs altogether, then there is only one bit for each PO considering the PEI payload size, and one bit can only enable one subgroup for the UEs, and the UE power saving gain is largely decreased.

Observation 3: Considering the payload size of PEI is no larger than 41 bits and 43 bits for licensed and unlicensed spectrums respectively, supporting (N=T/32, Ns=16) and maintaining the current principle of associated at most 2 PFs for the PEI means that there could only be one subgroup for the UEs in PEI indication.

Proposal 2: Apart from Ns=8 and N=T/32, other values are not considered in this release.

- UE capability

Since the introduction of new N/Ns values affects the PO to be monitored by the UE, a new UE capability should be introduced to ensure alignment between the UE and the network. The new UE capability will be reported in UE-NR-Capability, and will be decoded by the gNB and included in the UERadioPagingInformation inter-node message.

It is preferred that the new UE capability is added in the UE-RadioPagingInfo container, because this IE was designed in R17 to incorporate all the paging related capabilities that need to be aware to the gNB.

Proposal 3: A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo.

In LP-WUS discussion, SA2 identified the issue of lost UE paging capability due to reporting to a legacy gNB, and asked whether this issue applies to other features [2]:

SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

In our understanding, the issue exists for R19 NES as well. If the UE reports the new capability to a legacy gNB who fails to generate the capability in inter-node message, the UE would assume the capability has been reported and monitors the additional POs based on extended N/Ns while the camped cell cannot receive the UE capability from the core network. However, the detailed solution can wait until conclusion is made in LP-WUS.

Proposal 4: Confirm the issue of lost paging capability (explained in S2-2412876) is valid for R19 NES as well. The detailed solution is postponed until conclusion is made in LP-WUS.

## 2.2 Adaptation of SSB

The discussion on SSB adaptation mainly took place in RAN1, and considerable progress has been made in RAN1.

Apart from the open issues identified by RAN1, we think RAN2 should discuss the impacts on L3 measurements due to dynamic adaptation of SSB transmission. It is also requested by RAN4 to discuss SMTC impacts in RAN2 [3]:

RAN4#112bis extensively discussed impact on RRM requirements from SSB adaptation. The following agreements were reached after discussion: Agreements: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.

The “SSB adaptation” mentioned in the RAN4 LS refers to the adaptation of SS burst periodicity [4]:

Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism)

For serving cell measurements, in the legacy mechanism the UE measures serving cell SSB based on SMTC configuration. In R19 SSB adaptation, the UE should adjust the SMTC periodicity based on the latest SSB periodicity.

For neighbour cell measurements, it is not feasible to ask UE to adapt SMTC periodicities based on dynamic change of neighbour cell SSB measurements, a proper way is to configure a separate SMTC for neighbour cells supporting SSB adaptation. For instance, if a neighbour cell supports dynamic change of SSB periodicity among {20ms, 40ms, 80ms}, the network can configure a dedicated SMTC of 80ms periodicity for such NES neighbour cells.

Proposal 5: A dedicated SMTC with larger periodicity can be configured to measure neighbour cells supporting dynamic SSB adaptation.

The above applies only to SSB adaptation, not to on-demand SSB, as there are differences between the two features:

- For OD-SSB, there could be on-demand SSB and always-on SSB simultaneously (it is still FFS in RAN1 whether there is time/frequency overlap between on-demand SSB and always-on SSB), while for SSB adaptation only the periodicity is changed.

- For OD-SSB, the SSB pattern is much more complex than legacy SSB pattern, as RAN1 has defined a time instance A to indicate the beginning of the fist transmitted SSB, and gNB can flexibly configure SSB time domain positions of on-demand SSB burst (which could be more compact compared with existing SSB positions in an SS burst). Still, for SSB adaptation, only the periodicity is changed.

- For neighbour cell measurements, in case of SSB adaptation, the largest periodicity of neighbour cell SSB can be predicted, therefore a SMTC window of large periodicity can be used for neighbour cell measurements. However, for on-demand SSB, it is not possible or efficient for the gNB to frequently exchange the dynamic pattern for neighbour cell OD-SSB transmission and configure the SMTC accordingly.

Proposal 6: Discuss L3 measurements for OD-SSB and SSB adaptation separately. No need to pursue a unified solution.

## 2.3 Adaptation of RACH

In RAN#104 meeting, the revised WID has already removed the PRACH adaptation in spatial domain [5].

Regarding RACH adaptation in time domain, RAN1 has agreed to introduce additional PRACH resources for NES-capable UEs in additional to PRACH resources for legacy UEs. The PRACH preamble format is the same for the legacy PRACH resources and the additional resources. Hence, only the time/frequency domains are left for discussion. The PRACH configuration index and the frequency domain resources can be same or different. It is still FFS whether additional parameters are needed if the PRACH configuration index is the same, and whether additional mechanism is needed if the PRACH configuration index is different.

Agreement For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources. Agreement For adaptation of PRACH in time-domain, support both of the following - Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources - Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources - FFS: Additional details Agreement For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different - FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement). - Discuss further following options for signaling - Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources. - msg1-FrequencyStart at least for 4-step RACH - FFS: other applicable legacy frequency domain parameter(s) - Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources - Note: Offset to legacy frequency domain parameter(s) is a new parameter - FFS: applicable legacy frequency domain parameter(s) Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16] - FFS: whether the subset of the additional PRACH resources is in - Alt 2-1: RO level per SSB - Alt 2-2: SSB-to-RO mapping cycle level - Alt 2-3: PRACH association period level - Alt 2-4: PRACH association pattern period level - Alt 2-5: SFN level - Alt 2-6: Network configured time period

There are other features introducing separate RACH resources, and can be categorized as follows:

- Using RACH partitioning framework, including (e)Redcap, SDT, Coverage enhancement, RAN slicing.

The configuration of RACH resource sets is achieved by separate ROs (via additionalRACH-ConfigList in BWP-UplinkCommon), or separate preamble partitioning with shared ROs (via featureCombinationPreamblesList in RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA).

- Not using RACH partitioning framework, including LTM and twoTA.

The additional RACH configuration for LTM is provided via EarlyUL-SyncConfig in dedicated RRC signalling. The early sync in LTM is RACH towards LTM candidate cell, not serving cell.

The additional RACH configuration for twoTA is provided via additionalRACH-perPCI-ToAddModList and additionalRACH-perPCI-ToReleaseList in BWP-UplinkCommon.

Since RAN1 has agreed in 116bis meeting that the PRACH adaptation is applicable for UEs all RRC states, the additional RACH resources should be provided in SIB1. Additionally, the P-RNTI is assumed to be used to indicate the PRACH adaptation. After receiving the DCI, UE in all RRC states could use the additional PRACH resources based on the configuration in SIB1.

Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - [DCI payload size = 1 bit] - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources - FFS: details

Proposal 7: The additional PRACH resources for NES-capable UEs are provided in SIB1.

On the detailed signalling, one open issue is whether we incorporate the additional PRACH resources of NES into the RACH partitioning framework. There are two things to be considered: 1) whether there is a need to enable the early identification of NES feature on the network side; 2) whether other RACH-related features like Redcap/SDT/CE can be used jointly with NES.

Take Redcap as an instance, we think it is possible for NES cells to support the access of Redcap UEs, there is no need to exclude the combination of NES and Redcap. RAN2 has discussed whether to prevent legacy UEs from accessing R19 NES cell, and the “legacy UEs” usually refer to the UEs from previous releases not supporting the R19 NES feature. RAN2 can also consider whether UEs of other features (e.g. Redcap UEs) can implement the R19 NES feature and whether these kinds of UEs should be considered when designing the R19 framework.

Proposal 8: RAN2 to discuss whether to incorporate the additional PRACH resources of NES into the RACH partitioning framework, based on two factors: 1) whether there is a need to enable the early identification of NES feature on the network side; 2) whether other RACH-related features like Redcap/SDT/CE can be used jointly with NES.

On CBRA vs CFRA, we think RAN2 should start from CBRA, because UEs in Idle and Inactive states need to be considered and the objective is restricted to common signals. The discussion on CFRA can wait until the mechanism on CBRA is finalized. On 2-step RA vs 4-step RA, we do not see the need to preclude 2-step RA at the current stage.

Proposal 9: CBRA is prioritized for RACH adaptation in time domain.

Proposal 10: Both 4-step RA and 2-step RA are considered for RACH adaptation in time domain.

Conclusion

In this contribution, we discussed the adaptation of common signal/channel transmissions, and made the following proposals:

Observation 1: Option 3 is most flexible, the NW configures separate paging/PEI parameters for legacy UEs and R19 NES UEs, the mapping method between PO and PEI follows existing mechanism. Option 2 saves the most NW energy as it enables the NW to send fewer PEI indications. The drawback of Option 1 is that the PEI cannot be mapped to 2 PFs (which is possible in the existing spec).

Proposal 1: RAN2 to down-select from the following options:

- Option 1: single PEI configuration with separate PEI transmission;

- Option 2: single PEI configuration with single PEI transmission;

- Option 3: separate PEI configuration.

Observation 2: Paging enhancements is not at the expense of reducing paging capacity, i.e. the introduction of one additional N value always comes with one additional Ns value.

Observation 3: Considering the payload size of PEI is no larger than 41 bits and 43 bits for licensed and unlicensed spectrums respectively, supporting (N=T/32, Ns=16) and maintaining the current principle of associated at most 2 PFs for the PEI means that there could only be one subgroup for the UEs in PEI indication.

Proposal 2: Apart from Ns=8 and N=T/32, other values are not considered in this release.

Proposal 3: A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo.

Proposal 4: Confirm the issue of lost paging capability (explained in S2-2412876) is valid for R19 NES as well. The detailed solution is postponed until conclusion is made in LP-WUS.

Proposal 5: A dedicated SMTC with larger periodicity can be configured to measure neighbour cells supporting dynamic SSB adaptation.

Proposal 6: Discuss L3 measurements for OD-SSB and SSB adaptation separately. No need to pursue a unified solution.

Proposal 7: The additional PRACH resources for NES-capable UEs are provided in SIB1.

Proposal 8: RAN2 to discuss whether to incorporate the additional PRACH resources of NES into the RACH partitioning framework, based on two factors: 1) whether there is a need to enable the early identification of NES feature on the network side; 2) whether other RACH-related features like Redcap/SDT/CE can be used jointly with NES.

Proposal 9: CBRA is prioritized for RACH adaptation in time domain.

Proposal 10: Both 4-step RA and 2-step RA are considered for RACH adaptation in time domain.

References

- R2-2410911, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung)

- S2-2412876, LS Reply on LP-WUS subgrouping, SA2 (contact company: Huawei)

- R4-2416911, LS on SSB adaptation, RAN4 (contact company: Apple)

- R4-2416860, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei

- RP-241650, Revised WID: Enhancements of network energy savings for NR, Ericsson