3GPP TSG-RAN WG2 Meeting #129	R2-2500708

Athens, GR, 17th – 21st Feb, 2025

Agenda Item:	8.10.4

Source: 	China Unicom

Title:  	Discussion on SON/MDT for NTN mobility

Document for:	Agreement

Introduction

RAN3#126 meeting made some progress on SON/MDT for NTN and sent and LS to RAN2 in [1]. The main issues that involved with RAN2 are listed as follows:

- Regarding SON/MDT for NTN: On MRO for NTN, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not. - On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP.

This document starts the discussion on SON/MDT for NTN based on RAN3’s LS.

Discussion

- 2.1	MRO for NTN

For the combination of CHO and NTN scenarios, RAN3 agreed to report the historical fulfilled conditions for NTN before RLF occurs. This approach differs from the current specification, where fulfilled CHO conditions are logged in RLF reports only at the moment of HOF or RLF. Therefore, a new IE is required to log the historical fulfilled conditions for NTN before RLF occurs.

Proposal 1: A new IE should be introduced in RLF reports to log the historical fulfilled CHO conditions for NTN before RLF occurs.

- 2.2	MDT for NTN

Considering the large coverage of NTN cells, it is inefficient to log all MDT results across the cell when the UE is in the IDLE/INACTIVE state. Thus, a new definition that enables MDT logging within a smaller and more flexible area scope is needed, such as a geographical area (a reference location plus a radius).

Proposal 2: The geographical area (a reference location plus a radius) can be reused for area scope of logged MDT over Uu for NTN.

Conclusion

RAN2 is kindly asked to discuss the following proposals:

Proposal 1: A new IE should be introduced in RLF reports to log the historical fulfilled CHO conditions for NTN before RLF occurs.

Proposal 2: The geographical area (a reference location plus a radius) can be reused for area scope of logged MDT over Uu for NTN.

References

- R3-247908, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT.