3GPP TSG-RAN WG2 Meeting #129	R2-2500766

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.7.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	DSR enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#128:

Thresholds configuration Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. Non-delay critical data ahead in the queue The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In this contribution we further discuss remaining issues on DSR.

# 2	Discussion

## 2.1	Thresholds Configuration

Multiple reporting thresholds has been agreed for Rel-19, but it has not been decided is whether the Rel-19 reporting threshold parameter could also be used in case of single reporting threshold, . In our views, allowing the Rel-19 parameter to be appliable for both single and multiple reporting thresholds enables a more flexible configuration where the threshold for reporting delay critical data and the threshold for triggering DSR reporting can be configured to be different also in case of single reporting threshold.

Proposal 1: the Rel-19 DSR reporting threshold parameter could be used to configure single or multiple reporting thresholds.

If the R bit in RT octet is used to indicate whether there is a further BS/RT pair following for the LCG, the new DSR format could also be used even if there is only one entry for each LCG, or only the LCGs configured with legacy single threshold are to be reported. Always being able to use the new DSR format as long as there is any LCG configured with multiple reporting thresholds may also simplify the DSR selection without extra overhead.

Proposal 2: for each reported BS/RT entry, an extension bit (the R bit in RT octet) is used to indicate whether there is a further BS/RT pair following for the LCG.

Proposal 3: the Rel-19 DSR format is used when there is any LCG configured with multiple reporting thresholds since there is no overhead difference from legacy DSR even if all the LCGs are with single entry.

## 2.2	Impact on BSR

With current design of allowing independent configuration of the DSR triggering threshold and reporting threshold, the NW has the flexibility of configuring DSR to report buffer status of non-delay critical data as well which was covered by BSR in Rel-18. It would make sense to allow the NW to disable BSR for the LCH if DSR already serve the purpose or the UE should take care of cancellation of the BSR if DSR already includes the full buffer status to avoid unnecessary overhead with duplicated information.

Proposal 4: To avoid double reporting with BSR and DSR, one of the following options could be considered:

Option 1: The signalling design should allow to disable BSR and enable DSR only for an LCH/LCG;

Option 2: the UE cancels BSR if DSR already includes full buffer status for all the LCGs.

## 2.3	Non-delay critical data ahead in the queue

Regarding non-delay critical data ahead of delay critical data in the queue, it was agreed in RAN2#128 that the UE may support (UE capability) including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

However, the Rel-19 DSR may also report non-delay critical data since the triggering threshold, which is also used for determining if the data is delay critical, may be lower than at least one of the reporting thresholds.

Therefore, the RAN#128 agreement on non-delay critical data ahead of delay critical data should be generalized to non-delay critical data ahead of non-delay critical. E.g., the buffer size calculation for a given remaining time range should also include data that is ahead of any PDU whose associated remaining time is within the reported remaining time range.

To better illustrate this, let’s consider an example where data belonging to different PDU sets (PDU set#0, #1, and #2 in Figure2.2-1) arrive in the UE buffer interlaced. We assume that PDU set based discard (pdu-SetDiscard-r18) is configured.

Figure 2.2-1: DSR with low-importance and high-importance data

In the example, the UE is configured with 3 reporting thresholds: RTT0, RTT1 and RTT2. The triggering threshold is assumed to be equal to RTT0. When the DSR is triggered, there is data of PDU set #1 (i.e. PDU #4), whose associated remaining time RT2 is in the range [RTT0, RTT1), which is ahead of data of PDU set#0, whose associated remaining time 
RT0 is in the range [0, RTT0). If the data ahead of PDU #4 with remaining time larger than the remaining time of PDU #4 (i.e. PDUs #2 and #3) is not included in the BS calculation of the range [RTT0, RTT1), then the DSR with multiple reporting thresholds would look like the one depicted in Table 2.2-1.

Table 2.2-1: Buffer size in multiple reporting thresholds

| Remaining Time range | Reported Remaining Time | Reported Buffer Size |
| --- | --- | --- |
| [0,RTT0) | RT0 | S0+S1+S4 |
| [RTT0,RTT1) | RT2 | S2+S3+S5+S6 |

Then, the gNB may think that allocating a transport block size of S0+S1+S4 would be enough for the UE to transmit data with associated remaining time RT0. However, this is not the case. Therefore, a better option for the DSR with multiple reporting thresholds would be the one shown in Table 2.2-2.

Table 2.2-2: Buffer size in multiple reporting thresholds

| Remaining Time range | Reported Remaining Time | Reported Buffer Size |
| --- | --- | --- |
| [0,RTT0) | RT0 | S0+S1+S2+S3+S4 |
| [RTT0,RTT1) | RT2 | S5+S6 |

Proposal 5: Include in the BS calculation for DSR corresponding to a remaining time range all PDUs until the last PDU belonging to any PDU set whose associated remaining time is within the remaining time range.

# 3	Conclusion

Remaining issues on DSR are discussed in this contribution with the following proposals proposed:

Proposal 1: the Rel-19 DSR reporting threshold parameter could be used to configure single or multiple reporting thresholds.

Proposal 2: for each reported BS/RT entry, an extension bit (the R bit in RT octet) is used to indicate whether there is a further BS/RT pair following for the LCG.

Proposal 3: the Rel-19 DSR format is used when there is any LCG configured with multiple reporting thresholds since there is no overhead difference from legacy DSR even if all the LCGs are with single entry.

Proposal 4: To avoid double reporting with BSR and DSR, one of the following options could be considered:

Option 1: The signalling design should allow to disable BSR and enable DSR only for an LCH/LCG;

Option 2: the UE cancels BSR if DSR already includes full buffer status for all the LCGs.

Proposal 5: Include in the BS calculation for DSR corresponding to a remaining time range all PDUs until the last PDU belonging to any PDU set whose associated remaining time is within the remaining time range.