3GPP TSG-RAN WG2 Meeting #129							    R2-2500780

Athens, Greece, Feb 17 – 21 2025

Source: 	ZTE Corporation, Sanechips

Title: 	SDT on separate BWP

Agenda item:	8.18

Document for: 	Discussion and Decision

# Introduction

At RAN2#128, there was a discussion regarding the BWP restriction in SDT [1]. It was agreed that this issue will be addressed but the exact solution is left FFS. In this contribution we look at various aspects of configuring SDT on a separate BWP and propose a way forward.

# Discussion

Currently, SDT resources can only be configured in the initial BWP. In Rel-17 there was some discussion configuring SDT on non-initial BWP, however this was not agreed at that time. In [1], it was noted that the SDT operation over initial BWP also requires the usage of conservative MCS schemes because of lack of CQI feedback. This means that the resource usage for SDT will be high, contributing to congestion over initial BWP. One option to solve this issue is to introduce a CQI feedback to be provided by the UE to gNB. In [1], it was proposed that the CQI feedback is included in MSG3/MSGA. However, the size of MSG3/MSGA is quite restricted and including additional information in these messages is not practical as this will restrict the coverage of MSG3/MSGA. Furthermore, the network has a good idea of the channel quality for initial access anyway because it can estimate the quality based on the uplink messages such as RACH and MSG3/MSGA. In case of SDT, the situation is even better because the network also configures an RSRP threshold for SDT and hence if SDT is initiated, then it means that the DL signal is above the RSRP threshold and this also gives a good indication of which MCS the network may use for the SDT session. If inclusion of CQI in MSG3/MSGA is expected to provide some additional benefit, then it would be good at least to check if this is the case with RAN1.

Observation 1: Inclusion of additional information in MSG3/MSGA will reduce the coverage of these messages and is not preferable.

Observation 2: Network has a reasonable idea of the MCS to be used for SDT data based on the quality of RACH/MSG3/MSGA and also based on the fact that SDT is only initiated if the path loss reference of the cell is above the configured RSRP threshold

Based on the above, the following proposal is made:

Proposal 1: If RAN2 intends to consider inclusion of CQI in MSG3/MSGA, we should check with RAN1 if this is indeed beneficial

One general issue with restricting SDT to initial BWP is that the SDT traffic will be on the same BWP as the initial access signalling from other IDLE/INACTIVE mode UEs. This will restrict the capacity of the network to support a larger population of SDT UEs in the cell. To avoid this issue, one option is to remove the restriction of initial BWP for SDT UEs.

Observation 3: Restricting SDT operation on initial BWP results in congestion over initial BWP and may unnecessarily restrict the network capacity.

Thus, one option that can be considered is to remove this restriction of initial BWP for SDT. This means that SDT may be configured over non-initial BWP.

Proposal 2: Discuss whether configuration of SDT on non-initial BWP can be specified as part of TEI19

If RA-SDT operation is to be allowed on separate (non-initial) BWP, then the UE has to select the separate BWP for RA-SDT based on feature priorities. So, in order to implement this, the UE has to first check whether RA-SDT resources exist on the separate BWP (considering all the feature priorities etc) and select this BWP if RA-SDT resources exist on this BWP (according to the current feature prioritization mechanism). If not, the UE shall perform the resume procedure on the normal initial BWP.

Proposal 3: For RA-SDT, the UE shall select the new BWP for SDT if RA-SDT resources exist on the non-initial BWP considering all the feature priorities etc and if this is not the case, then the UE shall select the normal initial BWP.

In case of CG-SDT on the other hand, there is no issue with RACH partitioning and feature prioritization. So, if SDT is triggered and valid CG-SDT resources exist on the separate BWP, then the UE shall select the CG-SDT resources on the separate BWP.

Proposal 4: If a valid CG-SDT resource exists on the non-initial BWP then the UE shall select the CG-SDT resource one the non-initial BWP

# Conclusion and proposals

The following observations and proposals are made in this paper:

Observation 1: Inclusion of additional information in MSG3/MSGA will reduce the coverage of these messages and is not preferable.

Observation 2: Network has a reasonable idea of the MCS to be used for SDT data based on the quality of RACH/MSG3/MSGA and also based on the fact that SDT is only initiated if the path loss reference of the cell is above the configured RSRP threshold

Observation 3: Restricting SDT operation on initial BWP results in congestion over initial BWP and may unnecessarily restrict the network capacity.

Proposal 1: If RAN2 intends to consider inclusion of CQI in MSG3/MSGA, we should check with RAN1 if this is indeed beneficial

Proposal 2: Discuss whether configuration of SDT on non-initial BWP can be specified as part of TEI19

Proposal 3: For RA-SDT, the UE shall select the new BWP for SDT if RA-SDT resources exist on the non-initial BWP considering all the feature priorities etc and if this is not the case, then the UE shall select the normal initial BWP.

Proposal 4: If a valid CG-SDT resource exists on the non-initial BWP then the UE shall select the CG-SDT resource on the non-initial BWP

# References

- R2-2410542	Quality Indication in Msg3 for SDT	Ericsson	discussion	Rel-19	TEI19