3GPP TSG-RAN WG2 Meeting #129	                                          R2-2500800

Athens, Greece, 17 – 21, Feb, 2025

Source: 	Xiaomi

Title:  	Discussion on generalization performance over UE speed of GC1

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

Introduction

In last meeting, it’s agreed to study the generalization over speed. Following generalization evaluation methodology is agreed

Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

In this contribution, we provide simulation results of generalization performance over UE speed. Some observations are given.

Discussion

Simulation assumption

In the simulation, sliding L1/L3 filtering option is used for both FR1 and FR2.

In FR1 case B, example 1 of temporal domain prediction pattern is used depicted following fig.

Example 1 of intra-frequency temporal domain case B

FR1 generalization performance

In last meeting, following simulation combinations are agreed for FR1 generalization study,

|  | Training @Dataset: 30km/h | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Inference @30km/h | Inference @60km/h | Inference @90km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#1 | Yes |  |  |  | Yes | Yes |
| Baseline |  | Yes |  |  | Yes |  |
| GC#1 |  | Yes |  | Yes |  | Yes |
| Baseline |  |  | Yes |  |  | Yes |
| GC#1 |  |  | Yes | Yes | Yes |  |

It’s up to company to select the prediction pattern, i.e. PW an OW. We select multiple patterns to check the generalization performance.

Following table shows the generalization performance between baseline and GC1 when MRRT=1, i.e PW=OW.

| OW/PW (ms) | 80/80 | 120/120 |
| --- | --- | --- |
| Baseline (30/30) average RSRP difference (dB) NOTE1 | 0.771 | 1.030 |
| GC#1 (60/30) average RSRP difference (dB) NOTE2 | 0.791 | 1.065 |
| GC#1 (90/30) average RSRP difference (dB) | 0.815 | 1.093 |
| Baseline (60/60) average RSRP difference (dB) | 0.751 | 0.957 |
| GC#1 (30/60) average RSRP difference (dB) | 0.767 | 0.983 |
| GC#1 (90/60) average RSRP difference (dB) | 0.749 | 0.958 |
| Baseline (90/90) average RSRP difference (dB) | 0.764 | 1.001 |
| GC#1 (30/90) average RSRP difference (dB) | 0.810 | 1.077 |
| GC#1 (60/90) average RSRP difference (dB) | 0.780 | 1.022 |

NOTE1: Baseline (30/30) refers to the scenario that AI model is trained and tested with the 30 km/h data set.

NOTE2: GC1#1 (60/30) refers to the scenario that AI model is trained with 60 km/h data set and tested with 30 km/h data set.

When baseline is trained with 30 km/h data set, GC1 (60/30 km/h) performs better than GC1 (90/30 km/h), i.e. average RSRP difference is smaller. We understand it’s because the 60km/h data set has more similarity with the 30 km/h data set than 90 km/h data set. Similarly, when baseline is trained with 90km/h data set, GC1 (60/90) performs better than GC1 (30/90). The performance degradation is less when speed difference is smaller between baseline and GC1.

Observation 1: The larger speed difference between GC1 and baseline results in larger performance degradation between baseline and GC1.

If the OW and PW length increases from 80ms to 120ms, we can observe the average RSRP difference increases. Even though the average RSRP difference of baseline also increases, generalization degradation between GC1 and baseline still increases a bit, i.e from 2% to 3%.

Observation 2: The larger PW and OW length results in larger performance degradation between GC1 and baseline if MRRT is fixed.

Following table further shows generalization performance between baseline and GC1 when MRRT=2, i.e PW=2*OW.

| OW/PW (ms) | 80/80 | 80/160 |
| --- | --- | --- |
| Baseline (30/30) RSRP difference (dB | 0.771 | 1.202 |
| GC#1 (60/30) RSRP difference (dB) | 0.791 | 1.276 |
| GC#1 (90/30) RSRP difference (dB) | 0.815 | 1.320 |
| Baseline (60/60) RSRP difference (dB) | 0.751 | 1.187 |
| GC#1 (30/60) RSRP difference (dB) | 0.767 | 1.220 |
| GC#1 (90/60) RSRP difference (dB) | 0.749 | 1.195 |
| Baseline (90/90) RSRP difference (dB) | 0.764 | 1.251 |
| GC#1 (30/90) RSRP difference (dB) | 0.810 | 1.355 |
| GC#1 (60/90) RSRP difference (dB) | 0.780 | 1.282 |

With the same OW and PW increases from 80ms to 160ms, we can observe the average RSRP difference increases. Even though the average RSRP difference of baseline also increases, generalization degradation between GC1 and baseline still increases, i.e from 2% to 6%.

Observation 3: The larger MRRT would result in larger performance degradation between GC1 and baseline.

Proposal 1: Capture observation 1-3 in TR for FR1.

FR2 generalization performance

In last meeting, following simulation combinations are agreed for FR2 generalization study,

|  | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Training @Dataset: 120km/h | Inference @60km/h | Inference @90km/h | Inference @120km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#1 | Yes |  |  |  | Yes | Yes |
| Baseline |  | Yes |  |  | Yes |  |
| GC#1 |  | Yes |  | Yes |  | Yes |
| Baseline |  |  | Yes |  |  | Yes |
| GC#1 |  |  | Yes | Yes | Yes |  |

Following table shows generalization performance between baseline and GC1 with different prediction patterns,

| OW/PW (ms) | 160/160 | 160/80 | 320/160 | 240/80 |
| --- | --- | --- | --- | --- |
| Baseline (60/60) RSRP difference (dB | 1.280 | 0.760 | 1.080 | 0.689 |
| GC#1 (90/60) RSRP difference (dB) | 1.288 | 0.768 | 1.098 | 0.694 |
| GC#1 (120/60) RSRP difference (dB) | 1.296 | 0.772 | 1.150 | 0.704 |
| Baseline (90/90) RSRP difference (dB) | 1.363 | 0.822 | 1.199 | 0.701 |
| GC#1 (60/90) RSRP difference (dB) | 1.366 | 0.825 | 1.218 | 0.739 |
| GC#1 (120/90) RSRP difference (dB) | 1.364 | 0.824 | 1.219 | 0.741 |
| Baseline (120/120) RSRP difference (dB) | 1.436 | 0.861 | 1.306 | 0.813 |
| GC#1 (60/120) RSRP difference (dB) | 1.449 | 0.885 | 1.404 | 0.827 |
| GC#1 (90/120) RSRP difference (dB) | 1.440 | 0.881 | 1.354 | 0.820 |

Similar as FR1, the larger speed difference between GC1 and baseline results in larger performance degradation.

Observation 4: The larger speed difference between GC1 and baseline results in larger performance degradation between baseline and GC1.

Proposal 2: Capture observation 4 in TR for FR2.

Conclusions

According to the analysis given above, we have the following observations and proposals:

FR1

Observation 1: The larger speed difference between GC1 and baseline results in larger performance degradation between baseline and GC1.

Observation 2: The larger PW and OW length results in larger performance degradation between GC1 and baseline if MRRT is fixed.

Observation 3: The larger MRRT would result in larger performance degradation between GC1 and baseline.

Proposal 1: Capture observation 1-3 in TR for FR1.

FR2

Observation 4: The larger speed difference between GC1 and baseline results in larger performance degradation between baseline and GC1.

Proposal 2: Capture observation 4 in TR for FR2.

Reference

[1] TR 36.839