3GPP TSG-RAN WG2 Meeting #129	                                          R2-2500801

Athens, Greece, 17 – 21, Feb, 2025

Source: 	Xiaomi

Title:  	Discussion on generalization performance over UE speed of GC2

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

Introduction

In last meeting, it’s agreed to study the generalization over speed. Following generalization evaluation methodology is agreed

Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

In this contribution, we provide simulation results of generalization performance over UE speed. Some observations are given.

Discussion

Simulation assumption

In the simulation, sliding L1/L3 filtering option is used for both FR1 and FR2.

In FR1 case B, example 1 of temporal domain prediction pattern is used depicted following fig.

Example 1 of intra-frequency temporal domain case B

FR1 generalization performance

In last meeting, following simulation combinations are agreed for FR1 generalization study,

|  | Training @Dataset: 30km/h | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Inference @30km/h | Inference @60km/h | Inference @90km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#2 | Yes | Yes | Yes | Yes |  |  |
| Baseline |  | Yes |  |  | Yes |  |
| GC#2 | Yes | Yes | Yes |  | Yes |  |
| Baseline |  |  | Yes |  |  | Yes |
| GC#2 | Yes | Yes | Yes |  |  | Yes |

It’s up to company to select the prediction pattern, i.e. PW an OW. We select multiple patterns to check the generalization performance. Following table shows the generalization performance between baseline and GC2 with different patterns.

| OW/PW (ms) | 80/80 | 80/160 | 120/120 | 200/400 |
| --- | --- | --- | --- | --- |
| Baseline (30/30) NOTE1 | 0.771 | 1.202 | 1.030 | 1.885 |
| GC#2 (30) NOTE2 | 0.839 | 1.270 | 1.074 | 2.043 |
| Baseline (60/60) | 0.751 | 1.187 | 0.957 | 2.088 |
| GC#2 (60) | 0.758 | 1.189 | 0.982 | 2.122 |
| Baseline (90/90) | 0.764 | 1.251 | 1.001 | 2.429 |
| GC#2 (90) | 0.774 | 1.268 | 1.017 | 2.639 |

NOTE1: Baseline (30/30) refers to the scenario that AI model is trained and tested with the 30 km/h data set.

NOTE2: GC#2 (30) refers to the scenario that AI model is trained with mixed data set and tested with 30 km/h data set.

We can observe the GC2(60) shows the least performance degradation compared with baseline, while GC2(30) and GC2(90) shows larger performance degradation. According to observation 1 in [2], the smaller speed difference results in less performance degradation. We understand the reason is the speed difference between 60 km/h data set and mixed data set is the smallest.

Observation 1: GC2 can reach best performance tested by the middle UE speed data set, i.e. 60 km/h.

Proposal 1: Capture observation 1 in TR for FR1.

FR2 generalization performance

In last meeting, following simulation combinations are agreed for FR2 generalization study,

|  | Training @Dataset: 60km/h | Training @Dataset: 90km/h | Training @Dataset: 120km/h | Inference @60km/h | Inference @90km/h | Inference @120km/h |
| --- | --- | --- | --- | --- | --- | --- |
| Baseline | Yes |  |  | Yes |  |  |
| GC#2 | Yes | Yes | Yes | Yes |  |  |
| Baseline |  | Yes |  |  | Yes |  |
| GC#2 | Yes | Yes | Yes |  | Yes |  |
| Baseline |  |  | Yes |  |  | Yes |
| GC#2 | Yes | Yes | Yes |  |  | Yes |

Following table shows the generalization performance between baseline and GC2 with different patterns.

| OW/PW (ms) | 160/160 | 160/80 | 320/160 | 240/80 |
| --- | --- | --- | --- | --- |
| Baseline (60/60) | 1.280 | 0.760 | 1.080 | 0.689 |
| GC#2 (60) | 1.296 | 0.776 | 1.106 | 0.702 |
| Baseline (90/90) | 1.363 | 0.822 | 1.199 | 0.701 |
| GC#2 (90) | 1.373 | 0.832 | 1.251 | 0.750 |
| Baseline (120/120) | 1.436 | 0.861 | 1.306 | 0.813 |
| GC#2 (120) | 1.448 | 0.889 | 1.368 | 0.828 |

Similar as FR1, we can observe the GC2(90) shows the least performance degradation compared with baseline, while GC2(60) and GC2(120) shows larger performance degradation. The reason is the speed difference between 90 km/h data set and mixed data set is the smallest. According to observation 1, the smaller speed difference results in less performance degradation.

Observation 2: GC2 can reach best performance tested by the middle UE speed data set, i.e. 90 km/h.

Proposal 2: Capture observation 2 in TR for FR2.

Conclusions

According to the analysis given above, we have the following observations and proposals:

FR1

Observation 1: GC2 can reach best performance tested by the middle UE speed data set, i.e. 60 km/h.

Proposal 1: Capture observation 1 in TR for FR1.

FR2

Observation 2: GC2 can reach best performance tested by the middle UE speed data set, i.e. 90 km/h.

Proposal 2: Capture observation 2 in TR for FR2.

Reference

[1] TR 36.839