3GPP TSG-RAN WG2 Meeting #129								R2-2500810

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.15

Source: 	Huawei, HiSilicon

Title: 	Discussion on the remaining issues for BDS B2b

Document for:	Discussion and Decision

1. Introduction

Regarding the introduction of BDS B2b signal, there was a post meeting email discussion for LPP spec.

[Post128][402][POS] BDS B2b stage 3 CR check (CATT)

Scope: Check and update the CR in R2-2409627.

Intended outcome: Agreeable CR

Deadline: Long

In this contribution, we continue the discussion on the remaining issues.

2. Discussion

During the email discussion, QC has raised the issue about two different interpretations about the IOD field for BDS, signals, one for the model defined by bds-ClockModel/bds-KeplerianSet  used for B1I/B3I; another defined by bds-ClockModel2/bds-KeplerianSet2 used for B1C/B2a/B2b in the current running CR, as follows.

From our understanding:

IOD is just a version of the navigation message and has nothing to do with the unit or scale for the field toe. Hence, the description for the scale factor should be removed

The interpretation of the IOD field for B2b signal should be the same as BDS B1I and B3I. Hence, they can be added together regardless of the difference in the navigation model.

Based on the discussion above, we propose the following:

Proposal1: Remove the scale factor and unit for the field IOD for BDS B2b signal and apply the same change for the legacy fields description for B3I.

Proposal2: Add BDS B2b to the row for BDS B1I/B3I for the mapping between GNSS to iod bit string.

3. Conclusion

In this paper, we discuss the current issues within the LPP spec for the introduction of BDS B2b signal. We propose the following:

Proposal1: Remove the scale factor and unit for the field IOD for BDS B2b signal and apply the same change for the legacy fields description for B3I .

Proposal2: Add BDS B2b to the row for BDS B1I/B3I for the mapping between GNSS to iod bit string.

4. References

[1] BDS B2b signal ICD, http://www.beidou.gov.cn/xt/gfxz/202008/P020230516558683155109.pdf