3GPP TSG-RAN WG2 #129	R2-2500863

# Athens, Greece, Feb 17th– 21st, 2025

Agenda Item:	8.2.3

Source:	Ericsson

Title:	UL multiple access for Ambient IoT

Document for:	Discussion/Decision

# 1	Introduction

A new work item for A-IoT has been recently agreed in RP#106 [1]. The following objectives are defined for RAN2 scope:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

We focus on the objective related to random access in this paper.

# 2	Discussion

## 2.1 Contention Based Random Access

Both 2-step and 3-step CBRA have pros and cons which are summarised in the below table.

Table 1

Pros and cons of 2- and 3-step CBRA

|  | 2-step CBRA | 3-step CBRA |
| --- | --- | --- |
| Pros | - Faster access or low latency as data can be transmitted in 1st UL message, i.e., Msg1. | - Works well in high load scenarios as Msg1 is much smaller than 2-step CBRA’s Msg1, and collision rate is reduced. - Works as well in low load scenario. The performance can be either same or slightly better than 2-step given both 2 and 3-step CBRA will have nearly same collision success probaility due to sparse load. |
| Cons | - Works well in low load scenario as Msg1 can be extremely big which may increase contention failures. | - Slightly longer latency as data is transmited in Msg3 or later. |

The clear benefit of 2-step is slightly short latency and applicability in low load scenario. However, for A-IoT, the latency is not critical requirement. Further, on the contrary, the load can be extremely high with thousands of devices contending for access and thus 2-step CBRA.

Hence, we believe, only 3-step CBRA should be considered as primary method for access. In other words, RAN2 doesn’t pursue any unified option to also support 2-step CBRA.

2-step CBRA benefits with low latency and low load scenarios.

3-step CBRA benefits with high load scenario. The performance in low load scenario is nearly same as low load scenario as collisions rate might be negligible.

Adopt only 3-step CBRA for A-IoT random access, i.e., not pursue any unified option to also support 2-step CBRA.

## 2.2	Paging and Access Round

The random-access procedure for inventory or command use case is initiated by a paging message. During this random access, there is probability that some devices may fail to access, and thus re-access is needed. Currently, there are couple of options where re-access can be triggered by a paging message or an access trigger message. But before getting into details regarding these two options, let us define following terminologies.

Access occasion: It is defined as resource opportunity for transmitting Msg1 and possibly successive messages if reader successfully echoes back the Msg1 to the device in the same occasion.

Access round: A round consists of N random-access occasions allocated by the reader to group of devices where a device selects one of the occasions randomly for its access.

Paging round: Depending on the option, a paging round can be defined as an equivalent to single access round or consisting of multiple access rounds.

The transmissions in contention based random-access procedure may be prone to collisions when there is high number of devices contending the channel. In addition, in general, the channel error probability for A-IoT transmissions would be higher than of typical NR transmission due to device’s limited capabilities and most likely its inability to do channel measurements. As a consequence, A-IoT transmissions can be unreliable and thus, a suitable feedback mechanism can be useful to indicate device’s UL message success or failure. However, the feedback has a cost on device energy and processing resources which are already scarce and limited.

Subject to feedback design, reliability requirements, there can be multiple ways re-access for the failed devices can be done. For instance, re-access can be done in same or next inventory/access round, therefore, first, it is important if we agree on terminologies related to paging, access, inventory round and the interactions which RAN2 has not defined properly. See below figure which depicts an interaction between paging message, access round, occasions, etc.

Figure 1: A paging message triggers an access round composed of N occasions serving group of devices. Each occasion can be used for CBRA based access. If the paging targets a single device and only single occasion is allocated in the access round, then the allocation behaviour reduces to CFRA.

RAN2 has captured two options: 1- and 2-tier in TR where the re-access allowed with access trigger or paging. The options are described as below.

Option A (1-tier): Paging round is equivalent to access round.

In this option, the access round is same as paging round. The paging message triggers the paging/access round consisting of N random-access occasions. In order to do re-access of failed devices, the reader can send new message to perform re-access with new resource allocation, e.g., more or less random-access occasions, updated MCS, etc. Below figure portrays a paging/access round consisting of N random-access occasions.

Figure 2: A paging round is equivalent to an access round.

Option B (2-tier): In this option, a paging round consists of multiple access rounds. Apart from 1st round, the remaining access rounds are primarily of re-access for the devices which unable to access in first round within a paging round. Given, multiple rounds are assumed within a paging round, essentially there can be two trigger message types:

- Paging trigger: Initiates first round or initial access round.

- Access trigger: Initiates successive rounds within a paging round. It differs from paging trigger as it may have reduced information. For instance, access trigger may not have information pertinent to transaction ID or devices IDs, etc. depending on what scenarios we foresee and define this trigger message accordingly with reduced overhead. If access trigger is defined like paging trigger, then this option reduces to Option A.

Figure 3: A paging round consists of multiple access rounds.

In the below table, we capture pros and cons of both options.

Table 2

Pros and cons of both options with paging round consisting of single or multiple access rounds.

|  | Option A: Paging round equal to access round | Option B: Paging round consists of multiple access rounds |
| --- | --- | --- |
| Benefits | - Reader and device complexity is lower as there is single message type is needed (i.e., paging trigger). - The device which has missed initial paging message (unable to decode due to channel error or lack of energy) may not particpate in correponding access round, however, it can particpate in next round if network delivers paging message again for re-access or new access for devices for the same request (e.g., same transaction ID). | - Signaling overhead may be lower slightly when it comes to re-acess given the device will be sent access trigger message which may contain only relevant or reduced infiormation, say updated resource allocation, such as new number of occasions in the round. - Given, the acces trigger may have relatively smaller size, the energy cost for processing such message will be lower. But there can be downside (see drawbacks) |
| Drawbacks | - Signaling overhead may increase when it comes to re-acess given the device will be sent paging message containing full or redundant information (as the failed user may already know some of the information when it had attempted failed initial access in previous round. - In contrast to acces trigger in Option B, the failed device may need to decode whole paging message everytime for re-access, and thus may have slightly increased energy cost. | - Reader and device complexity will be slightly higher as there will be multiple messages or triggers, for instance, paging trigger, access trigger, etc. - The access trigger messge may have reduced information which means both reader and device (with failed attempt) needs to remember paging message/trigger which adds cost to memory or buffering reuirement of old/paging information which is extremely critical for devices of Type 1 or passive nature. - The device which has missed initial paging message (unable to decode due to channel error or lack of energy) may not particpate in correponding access round, however, it may happen it still may not able particpate in next round initiated by access trigger if the message is defined in such as manner that some of the information in paging messag is excluded from access trigger message. In this round, only device can particpate which have successfully decoded paging message inittially but failed with uplink access, e.g., failure in Msg1 transmission or Msg3 transmission. - Due to large number of formats, the monitoring or blind decoding cost of number of formats may result in increased energy cost. |

Based on above texts, we believe tier-1 is most suited option as it results simple device behaviour.

- Adopt only 1-tier option where one paging round is equivalent to one access round.

## 2.3	Re-access

In RAN#127bis, it was agreed to support optional explicit R2D failure/success feedback indication for at least Msg3 for re-access purpose, and  FFS for following D2R data. However, in order to feedback design and various options, it is important to understand and agree on termninologies surrounding retransmission and re-access. Note, we TDMA as baseline for feedback. We will update he discussion once concerete behavior from RAN1 and RAN2 on FDMA basline is specified.

Retransmission: In an occasion, if a device has done contention resolution successfully, i.e., Msg1 is transmitted successfully and network has echoed back the selected RN16 in Msg2, then the device has a right to transmit its data/Msg3. If the device encounters Msg3 transmission failure, then the reader can send Msg2 again to trigger Msg3 retransmission. Summarising, the transmission is possible in the occasion where device has gotten the access. See Fig. 4.

Figure 4: Retransmission of failed Msg3 triggered by Msg2 retransmission.

Device performs re-access or retransmission in case of data transmission failure.

Device performs re-access in case of contention resolution failure.

Re-access: In an occasion, if a device has failed to get an access where it does not receive a Msg2 echoing back its selected RN16 from the reader, then device does not have the right to transmit its Msg3 in the current occasion and has to look for new occasion either in the same or next round or not, subject to re-access policies defined by the reader. Here, the re-access can be triggered based on defined policies and possibly no explicit indication is needed to trigger re-access. See Fig. 5.

Figure 5: Re-access after failed access due Msg1 transmission failure.

In another scenario, the re-access can also be permitted or defined, in case device has gotten the access in the given occasion but constantly failing with its Msg3/data transmission, then reader may allow re-access in a newer occasion where the device competes again for the access by transmitting Msg1 again. In this scenario, and explicit AS signalling possibly needed to trigger re-access as its intention from reader to stop the access in the current occasion and trigger a re-access. See Fig. 6. Most likely, this could happen due to poor coverage issues. The re-access in later time allows the device to get out of poor-coverage region or the obstacle to move away if the coverage issues are temporary.

Figure 6: Re-access is triggered after repeated failed Msg3 retransmission attempts.

Summarising above, as we see for retransmission, there is no ned for additional AS signalling other than Msg2 retransmission triggering Msg3 retransmission, however, for re-access, we may need additional explicit feedback signalling which indicates NACK and thus stopping the device in current occasion and triggering re-access. As indicated above, RAN2 already agreed to have such feedback message, but it was left optional without any details. Consider extremely simple device, to the least we may need AS signalling

In our option, the option based on explicit AS signaling (e.g., 1 NACK bit) is most suitable. Given majority devices are expected to have transmission success, and thus requiring fewer devices re-access, then sending NACK would be beneficial as it amounts to fewer transmissions.

Based on above discussion, we propose following observation and proposals.

For data transmission failure, the device may be triggered to perform re-access by the reader after a number of times of (re-)transmissions failed attempts.

Reader to send an explicit AS signalling (e.g., 1 bit) to devices for triggering re-accesses due to data transmission failures (regardless of if data transmission is segmented or not).

Further, this AS signalling is equally applicable for the case where if a transmission is divided in multiple segments. The reader doesn’t need to distinguish between segment and non-segment case regarding re-access trigger.

## 2.4 FDMA and Msg2

In the WID, FDMA with small frequency shift is included and we elaborate solutions for the above two issues focusing small frequency shift and wait for RAN1 progress on issue with large frequency shifts. Given the device can perform small frequency shift we believe, thus it can desire FDMA occasion from the pool of occasion within a TDMA occasion randomly. This is bit similar the same device selects a TDMA occasion in a random manner.

Device randomly selects among FDMA occasions as the baseline. FFS if other conditions/rules are needed for the selection of FDMA occasions.

On issue 2, one solution can be based on RAR functionality where Msg2 can be addressed to multiple devices with their RN16s as their sub-headers and corresponding resource allocation information for their Msg3 and Msg4 transmissions as MAC sub-payloads. The major issue is the message size would increase as it would include all the successfully transmitted RN16s which a device may not be interested in the message.

Other solution can be based on L1 mapping where a preceding DL control information is sent before individual Msg2 transmissions. Each Msg2 is addressed to single device and the preceding DL control information provides mapping information of Msg2 resource allocation, see below figure. The individual message size is decreased compare RAR inspired option, however, there is a need of additional DL control information prior to Msg2 transmissions which provides the resource mapping.

In our opinion, RAR functionality based Msg2 is more suitable as this behaviour already implemented in legacy solutions, and thus, for A-IoT, a simplified version for RAR message for Msg2 is very appropriate, and accordingly we capture following proposal.

The Msg2 echoes the list RN16s for the devices which have successfully accessed the FDMA occasions within a TDMA occasion limited by grant size.

For each echoed RN16, there should have some mapping between the echoed RN16 and FDMA resource. The mapping can be either

- Static, or

- Dynamic.

In case of static mapping, the Msg2 contains the same amount of placeholders as umber of available FDMA access occasions. The FDMA access occasion which is access successfully, then the mapped placeholder in Msg2 will indicate RN16. Other placeholders may indicate dummy RN16 where no device is identified. The drawback is the Msg2 length is long even the number of accessed devices is small.

On the other hand, the Msg2 can include FDMA occasion index as a header and the corresponding payload contains the accessed device’s RN16 in that particular FDMA occasion. Compared to static scenario, the message length is small as its size is limited by number of access devices, not by total number of FDMA occasions configured. Further, the order of indication of FDMA occasion indices need to be defined. For instance, FDMA access occasion at lower frequency channel can be assigned smaller index and the occasion at relatively higher frequency channel ca be assigned higher index. Further, a rule is needed on indicating index (MAC sub-header) and the corresponding RN16 (MAC sub-payload). In Msg2, the echoed RN16s for the corresponding FDMA occasion indices can be arranged in, e.g., increased sort order. We believe dynamic mapping is more suitable as static mapping may incur lot of wastage of resource as it necessitates dummy indications, resulting bigger Msg2 size.

In case there are multiple FDMA occasions available, reader signals devices a mapping between FDMA occasions and received random IDs in Msg2. FFS if the mapping is conveyed via L1 control info or MAC header.

In case there are multiple FDMA occasions available, Msg2 format allows inclusion of multiple random IDs limited by the size of the R2D resources for Msg2 transmission.

## 2.5 Contention-Free Access

To provide CFRA, the reader needs to allocate dedicated (collision-free) resources to the devices. We have named two methods, where in one method, the device IDs are know to reader and in another, they are kept hidden.

One of the method is to allocate resources by mapping them to their device IDs. It means the paging message includes non-transparent device ID (IDs are not part of NAS container) and the corresponding resources. It is not yet agreed what are these device IDs based on, e.g., AS ID, original device ID (CN allocated), or temporary ID (CN allocated) and whether it will be transparent or non-transparent to reader. This discussion we leave it to SA2 first except the part with AS ID which is a focus of agenda item dealing with protocol stack. In essence, in this method, the reader needs to know the IDs and provide mapping of resources.

In another method, the device IDs are part of NAS container or kept hidden from reader in paging message. In order to allocate CFRA resource, the

- the reader needs to know the group size or number of devices from CN and his information can be retrieved by reader from CN via NGAP signaling, and based on this information, the reader provides group of resources to group of devices without any explicit mapping;

- A implicit mapping rule is defined by the reader, for instance, the devices can arrange themselves to group of resources in increasing sort manner. For instance, device with lower ID have a right o transmit on resource with lower resource index;

- In order of implicit mapping to work, a device must able to other device IDs in paging message so the device knows where it should position itself on resource w.r.t. other devices

Below is summary of two methods.

| CFRA Allocation |  |
| --- | --- |
| Method 1: Device IDs are non-transparent to reader | Method 2: Device IDs are transparent to reader |
| There is direct/explcit mapping between IDs and resource | There is implicit mapping between IDs and resource as IDs are not known to reader The resources are ptovided by reader based on group size information which reader can retrive from CN via NGAP message The devices must be able to know other devices indicatd paging message in order to implcit map to pool of rsources |

Observation: For CFRA based allocation, two allocation methods are observed where in one method device IDs are known to reader, and in another method, it is transparent to reader.

For CFRA, it is assumed that the reader knows at least the number of devices (e.g., provided by CN) to allocate contention free resources corresponding to the number of devices.

For CFRA, reader indicates a mapping rule between the set of allocated resources and the corresponding devices explicitly or implicitly to devices.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	2-step CBRA benefits with low latency and low load scenarios.

Observation 2	3-step CBRA benefits with high load scenario. The performance in low load scenario is nearly same as low load scenario as collisions rate might be negligible.

Observation 3	Device performs re-access or retransmission in case of data transmission failure.

Observation 4	Device performs re-access in case of contention resolution failure.

Observation 5	For data transmission failure, the device may be triggered to perform re-access by the reader after a number of times of (re-)transmissions failed attempts.

Observation 6	The Msg2 echoes the list RN16s for the devices which have successfully accessed the FDMA occasions within a TDMA occasion limited by grant size.

Observation 7	For CFRA, it is assumed that the reader knows at least the number of devices (e.g., provided by CN) to allocate contention free resources corresponding to the number of devices.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt only 3-step CBRA for A-IoT random access, i.e., not pursue any unified option to also support 2-step CBRA.

Proposal 2	Adopt only 1-tier option where one paging round is equivalent to one access round.

Proposal 3	Reader to send an explicit AS signalling (e.g., 1 bit) to devices for triggering re-accesses due to data transmission failures (regardless of if data transmission is segmented or not).

Proposal 4	Device randomly selects among FDMA occasions as the baseline. FFS if other conditions/rules are needed for the selection of FDMA occasions.

Proposal 5	In case there are multiple FDMA occasions available, reader signals devices a mapping between FDMA occasions and received random IDs in Msg2. FFS if the mapping is conveyed via L1 control info or MAC header.

Proposal 6	In case there are multiple FDMA occasions available, Msg2 format allows inclusion of multiple random IDs limited by the size of the R2D resources for Msg2 transmission.

Proposal 7	For CFRA, reader indicates a mapping rule between the set of allocated resources and the corresponding devices explicitly or implicitly to devices.

# 4	References

RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.