3GPP TSG-RAN WG2 Meeting #129	R2-2500910

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.1.5

Source:	Samsung

Title:	Discussion on signalling feasibility of dataset and parameter sharing for CSI compression

Document for:	Discussion & Decision

# Introduction

RAN2 has received from RAN1 an LS (in R2-2500015) on signalling feasibility of dataset and parameter sharing. In it, RAN1 has outlined the most recent agreements on two-sided AIML CSI compression discussions, with specific focus on the dataset and/or parameter sharing solutions under consideration in RAN1. RAN1 has further asked RAN2 to evaluate the feasibility of signaling to support RAN1-identified solutions for the two-sided model case. In this submission, we discuss the key issues raised in the LS – which in our view lacks some key information which RAN2 would require in order to provide answers to questions raised. We provide a way forward for RAN2 comprising a number of proposals.

# Discussion

The focus of the LS is on the so-called ‘Direction A’ of RAN1’s discussion, whereby the NW-side shares model parameters and/or dataset with the UE/UE-side via standardized signaling. RAN1 concluded that the standardized signaling may be

- over-the-air (OTA), or

- other approaches

We have a general observation, and a related proposal:

Observation 1. For “other approaches” (non-OTA) referred to by RAN1 in their LS, RAN1 did not clarify the dataset and/or parameter sharing path. It is assumed the path is NW server to UE server (and without any 3GPP-standardised involvement of the UE), and is therefore beyond the scope of RAN2.

Proposal 1. RAN2 to ask RAN1 to confirm that the path for dataset and/or parameter sharing for 
non-OTA approaches is NW server to UE server (and without any 3GPP-standardised involvement of the UE), and to ask RAN1 to clarify what RAN1 believe RAN2’s remit here would be (if any).

For the OTA approach, one general observation worth noting is:

Observation 2a. For the OTA approach referred to by RAN1 in their LS, the dataset and/or parameter sharing path is NW  UE (via the 3GPP network), and then UE  UE server (in part via the 3GPP network, possibly in a way transparent to the 3GPP network).

For the NW UE portion of the path in the OTA approach, the following holds:

Observation 2b. For the OTA approach and the NW  UE portion of it, RAN2 can discuss CP and UP-based solutions.

Observation 2c. For the OTA approach and the NW  UE portion of it, use of downlink UP data transmission procedures is feasible, meaning no significant impact on RAN2 specs is expected in this Release.

For the UE UE server portion of the path in the OTA approach, we would like the following to be noted:

Observation 2d. For the OTA approach and the UE  UE server portion of it, UE-side data collection considerations apply.

Based on these initial obsevrations on OTA, we propose:

Proposal 2. For the OTA approach, RAN2 to focus on the NW  UE portion of it, and discuss CP and UP-based solutions, bearing in mind that no significant impact is expected on RAN2 specs if the UP-based approach is adopted. For the OTA approach and the UE  UE server portion of it, UE-side data collection considerations apply.

The data set transfer for the OTA approach could be managed by the NW server and/or the gNB. However the RAN1 LS does not make it clear if the gNB should be involved in managing the data set transfer (which is the case where RAN2 would have some remit):

Observation 3a. If the data set transfer for the OTA approach is managed by the NW server, question of feasibility of controllability of data set transfer procedure is beyond the scope of RAN2.

Observation 3b. If the data set transfer for the OTA approach is managed by the gNB, question of feasibility of controllability of data set transfer procedure is within the scope of RAN2.

Observation 3c. It is unclear at present (based on the RAN1 LS) whether the gNB should be involved in managing the data set transfer.

Proposal 3. RAN2 to ask RAN1 whether they envisage gNB being involved in managing the data set transfer.

For the various Options under consideration for dataset and/or parameter exchange, RAN1 have shared the total estimated dataset sizes, and on this topic we would like RAN2 to consider the following:

Observation 4a. In order to be able to work out whether changes (such as RRC segmentation for the CP case) are needed, RAN2 would need to understand the minimum data size which needs to be transmitted within a time window. This information appears lacking in the RAN1 LS.

Proposal 4a. RAN2 to ask RAN1 about their assumption on the minimum transmission time for the data set sizes quoted in the LS (the time range given in the LS “e.g. days/weeks” is too broad for meaningful calculations to be done), i.e. the upper limit on how many bits we need to move across the radio interface, within a given amount of time.

Based on our understanding of the RAN1 discussions, segmentation/concatenation of messages within a single data set may need to be done:

Observation 4b-1. From the RAN1 LS, it appears that segmentation/concatenation of messages within a single data set may need to be done in NW server/UE server, and/or at the gNB/UE.

Observation 4b-2. It is unclear at present whether the 3GPP network is meant to facilitated such segmentation/concatenation e.g. via an indexing scheme, in case when it happens at the gNB/UE.

Observation 4b-3. Whether any new segmentation/concatenation mechanism will be needed also depends on the minimum message size within the data set to be transmitted.

Proposal 4b. RAN2 to ask RAN1 about their assumption on how the segmentation/concatenation of messages within a single data set is to be done in UE server and whether assistance from the 3GPP network is needed for this, and to further ask for the minimum message size anticipated.

Based on the analysis above, our final proposal is as follows:

Proposal 5. RAN2 to send a reply-LS based on previous Proposals (1, 3, 4a & 4b) asking for clarifications outlined therein, and wait for clarifications from RAN1 detailed in previous Proposals (1, 3, 4a & 4b), before continuing this work along Proposal 2.

# Conclusion

RAN2 is kindly asked to discuss and agree the following proposals:

Proposal 1. RAN2 to ask RAN1 to confirm that the path for dataset and/or parameter sharing for 
non-OTA approaches is NW server to UE server (and without any 3GPP-standardised involvement of the UE), and to ask RAN1 to clarify what RAN1 believe RAN2’s remit here would be (if any).

Proposal 2. For the OTA approach, RAN2 to focus on the NW  UE portion of it, and discuss CP and UP-based solutions, bearing in mind that no significant impact is expected on RAN2 specs if the UP-based approach is adopted. For the OTA approach and the UE  UE server portion of it, UE-side data collection considerations apply.

Proposal 3. RAN2 to ask RAN1 whether they envisage gNB being involved in managing the data set transfer.

Proposal 4a. RAN2 to ask RAN1 about their assumption on the minimum transmission time for the data set sizes quoted in the LS (the time range given in the LS “e.g. days/weeks” is too broad for meaningful calculations to be done), i.e. the upper limit on how many bits we need to move across the radio interface, within a given amount of time.

Proposal 4b. RAN2 to ask RAN1 about their assumption on how the segmentation/concatenation of messages within a single data set is to be done in UE server and whether assistance from the 3GPP network is needed for this, and to further ask for the minimum message size anticipated.

Proposal 5. RAN2 to send a reply-LS based on previous Proposals (1, 3, 4a & 4b) asking for clarifications outlined therein, and wait for clarifications from RAN1 detailed in previous Proposals (1, 3, 4a & 4b), before continuing this work along Proposal 2.