3GPP TSG-RAN WG2 #129	Tdoc R2-2500931

Athens, Greece, February 17 – 21, 2025

Agenda Item:	5.1.3.2

Source:	Ericsson

Title:	Further aspects to consider for UE capability segmentation in LTE

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses another issue found with UE capability segmentation in LTE and proposes a way forward.

# 2	Discussion

The N26 interface is an inter-CN interface between MME in EPC and AMF in 5GC and supports inter-system (inter-RAT) handover procedures. The interface is based on GTPv2 and UDP transport protocol. The maximum UDP datagram size limitation of 65535 bytes inherent in UDP will limit the messages on N26 interface to max ~64 kB. In particular, the Forward Relocation Request message containing UE-NR-Capability and/or UE-EUTRA-Capability containers will be limited to ~64 kB.

When acquiring UE capabilities with segmentation in LTE, the UE can send up to 16 segments of UE capability. However, upon handover from LTE to NR, the eNB must forward the NR capabilities to the gNB, which will require the use of N26 interface between MME and AMF and will thus limit the overall size of the UE capabilities that can be transferred.

N26 interface is limited to ~64 kB and will impact the overall size of the UE capability that can be transferred from LTE to NR upon handover.

Therefore, if the UE capability has more than ~63 kB, such capability could not be transferred between LTE and NR. But as stated in 38.331, for such handover case the NR capabilities must be forwarded to the gNB:

| Source RAT | NR capabilities | E-UTRA capabilities | MR-DC capabilities | UTRA capabilities |
| --- | --- | --- | --- | --- |
| NR | May be included if UE Radio Capability ID as specified in 23.502 [43] is used for the UE. Included otherwise. | May be included | May be included | May be included, ignored by gNB if received |
| E-UTRAN | May be included if UE Radio Capability ID as specified in 23.502 [43] is used for the UE. Included otherwise. | May be included | May be included | May be included, ignored by gNB if received |

Note that the same issue does not happen in the opposite handover direction (from NR to LTE), since the NR capabilities are not mandatory to be transferred in this case (which should be a large part of the UE capability size), as stated in 36.331 below, and thus the LTE capabilities should be able to fit in the N26 interface.

| Source RAT | E-UTRA capabilites | UTRA capabilities | GERAN capabilities | MR DC capabilities | NR capabilities |
| --- | --- | --- | --- | --- | --- |
| UTRAN | Included | May be included, ignored by eNB if received | May be included | Excluded | Excluded |
| GERAN CS | Excluded | May be included, ignored by eNB if received | Included | Excluded | Excluded |
| GERAN PS | Excluded | May be included, ignored by eNB if received | Included | Excluded | Excluded |
| E-UTRAN | May be included if UE Radio Capability ID as specified in 23.502 [102] is used for the UE. Included otherwise. | May be included | May be included | May be included | May be included |
| NR | May be included if UE Radio Capability ID as specified in 23.502 [102] is used for the UE. Included otherwise. | Excluded | Excluded | May be included | May be included |

Upon handover from NR to LTE, it is not mandatory to forward NR capabilities and thus the LTE capabilities should be able to fit in the N26 interface.

To address this limitation, one can consider using the Rel-17 feature of maximum number of segments allowed for UE capability segmentation. However, it should be noted that even in that case it should be clarified that the number of segments to be allowed cannot be greater than 7, which would limit all enquiries that the NW performs with the same filters (i.e. if the NW requests MR-DC capabilities, those would also be limited to 7 segments). Moreover, the Rel-17 feature for maximum number of segments and the Rel-16 feature of UE capability segmentation were introduced independently in the specifications (i.e. a UE may support only the Rel-16 feature or only the Rel-17 feature), hence a solution should be found that can work in both features. Another possibility would be to rely on network implementation to re-enquire the UE capabilities without segmentation upon handover from LTE to NR, which would delay the handover process and could lead to handover failure. Therefore, we think the simplest solution would be to limit the number of segments that the UE sends when reporting segmented UE capabilities to LTE for NR SA. This solution should be sufficient for both the Rel-16 UL segmentation and the Rel-17 maximum number of segments allowed.

To clarify in 36.331: when reporting segmented UE capabilities to LTE for NR SA, the UE is only allowed to report up to 7 segments.

An example on how to implement this in 36.331 is provided below:

–                   ULDedicatedMessageSegment

The ULDedicatedMessageSegment message is used to transfer segments of the UECapabilityInformation message. In this version of the specification, the maximum size for the IE UE-NR-Capability defined in TS 38.331 [82] is 7 segments (if rat-Type set to nr is included in the corresponding UECapabilityEnquiry message and eutra-nr-only is absent in the same message).

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

ULDedicatedMessageSegment message

-- ASN1START

ULDedicatedMessageSegment-r16 ::=   SEQUENCE {

criticalExtensions                  CHOICE {

ulDedicatedMessageSegment-r16       ULDedicatedMessageSegment-r16-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

ULDedicatedMessageSegment-r16-IEs ::=   SEQUENCE {

segmentNumber-r16                       INTEGER (0..15),

rrc-MessageSegmentContainer-r16         OCTET STRING,

rrc-MessageSegmentType-r16              ENUMERATED {notLastSegment, lastSegment},

lateNonCriticalExtension                OCTET STRING                        OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                         OPTIONAL

}

-- ASN1STOP

ULDedicatedMessageSegment field descriptions

segmentNumber Identifies the sequence number of a segment within the encoded UL DCCH message.

rrc-MessageSegmentContainer Includes a segment of the encoded UL DCCH message. The size of the included segment in this container should be small enough that the resulting encoded RRC message PDU is less than or equal to the PDCP SDU size limit.

rrc-MessageSegmentType Indicates whether the included UL DCCH message segment is the last segment or not.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	N26 interface is limited to ~64 kB and will impact the overall size of the UE capability that can be transferred from LTE to NR upon handover.

Observation 2	Upon handover from NR to LTE, it is not mandatory to forward NR capabilities and thus the LTE capabilities should be able to fit in the N26 interface.

Based on the discussion in the previous sections we propose the following:

Proposal 1	To clarify in 36.331: when reporting segmented UE capabilities to LTE for NR SA, the UE is only allowed to report up to 7 segments.