3GPP TSG-RAN WG2 Meeting #129	R2-2500946

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.3.2.1

Source:	Nokia

Title:	Simulation results for RRM measurement predictions

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RRM measurement prediction has several sub-use-cases based on the objective and the input and outputs. In RAN#125bis the following cases were agreed:

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results

- Case 2: To directly predict cell level results based on cell level results.

- Case 3: To directly predict cell level results based on beam level results

The two agreed objectives are:

- Objective 1: Measurement reduction. Achieve the same mobility performance, but with reduced DL radio measurements that the UE needs to take and report, which improves UE energy efficiency and reduces the required signalling.

- Objective 2: Mobility optimization. Predict future UE DL radio measurements and use them to optimize mobility decisions.

Sub-use case prioritization (RAN#126):

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

This contribution builds upon our previous preliminary simulation results presented earlier in [1], which explored the use case of RRM measurement prediction. We now present the updated simulation results for temporal domain cases A and B.

# 2	Discussion

## 2.1	Temporal domain case A

In RRM temporal prediction case A, as illustrated in Figure 2.1-1, we assume no measurement reduction within the observation window. This distinguishes it from RRM temporal prediction case B [2]. To address this requirement, time series sequence-to-sequence prediction models are necessary. For evaluation purposes, we present results using various regression models and window sizes for RRM temporal prediction case A, categorized into the following sub-cases.

Figure 2.1-1: RRM temporal domain prediction – case A

Our simulations assumptions are highlighted in Table 2.1-1.

Table 2.1-1: Simulation parameters for the 7-site scenario based on TR38.843 (FR2)

| Reported simulation assumptions | UE trajectory option | Option 3 |
| --- | --- | --- |
|  | UE trajectory boundary processing option | Option 3 |
|  | UE speed | 30/60/120 km/h |
|  | SoftLOS | Yes |
|  | Spatial consistency option | Option B |
|  | Number of TX beams | 8 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate | N/A |
|  | Sub-use-case | Case 2 (cell to cell) |
|  | Observation window (OW) | [40, 80, 100, 200] ms |
|  | Prediction window (PW) | [40, 80, 100, 200] ms |
| Data Size (Number of Samples) | Training | 350MB |
|  | Testing | 150MB |
| AI/ML model input/output | Model input | L3 filtered serving cell RSRP measurements |
|  | Model output | L3 filtered serving cell RSRP measurements |
| AI/ML model description | Model type | Linear regression, Random forest, Gradient Boosting, |
|  | Model complexity in number of parameters | Linear regression, Random forest, Gradient Boosting: N/A |

We present the mean absolute error (MAE) evaluation results for sub-use case 2. This case applies cell-level measurements to directly predict cell-level measurements across different speed categories. We compare the performance of three models: linear regression, random forest, and gradient boosting.

Table 2.1-2: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different OW & PW, UE speed 30km/h, linear regression

| PW OW | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| 40ms | 0.6091 | 0.8940 | 1.0031 | 1.3631 |
| 80ms | 0.5996 | 0.8738 | 0.9807 | 1.3325 |
| 100ms | 0.5973 | 0.8708 | 0.9761 | 1.3289 |
| 200ms | 0.5967 | 0.8712 | 0.9766 | 1.3216 |

Table 2.1-3: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different OW & PW, UE speed 30km/h, random forest

| PW OW | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| 40ms | 0.6644 | 0.9366 | 1.0383 | 1.3670 |
| 80ms | 0.6414 | 0.8949 | 0.9867 | 1.2951 |
| 100ms | 0.6343 | 0.8760 | 0.9652 | 1.2635 |
| 200ms | 0.6158 | 0.8268 | 0.9025 | 1.1450 |

Table 2.1-4: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different OW & PW, UE speed 30km/h, gradient boosting

| PW OW | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| 40ms | 0.6468 | 0.8941 | 0.9888 | 1.2932 |
| 80ms | 0.6417 | 0.8860 | 0.9749 | 1.2746 |
| 100ms | 0.6402 | 0.8775 | 0.9654 | 1.2624 |
| 200ms | 0.6329 | 0.8567 | 0.9387 | 1.2076 |

For illustration purposes, we also summarize our findings in Figure 2.1-2.

Figure 2.1-2: Average MAE comparisons for different OW & PW and different models, 30km/h

Next, we present the results for UE speed 60km/h.

Table 2.1-5: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different OW & PW, UE speed 60km/h, linear regression

| PW OW | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| 40ms | 0.7617 | 1.1405 | 1.2988 | 1.8755 |
| 80ms | 0.7474 | 1.1226 | 1.2729 | 1.8402 |
| 100ms | 0.7526 | 1.1200 | 1.2714 | 1.8347 |
| 200ms | 0.7513 | 1.1252 | 1.2780 | 1.8404 |

Table 2.1-6: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different OW & PW, UE speed 60km/h, random forest

| PW OW | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| 40ms | 0.8318 | 1.1966 | 1.3465 | 1.8805 |
| 80ms | 0.7995 | 1.1394 | 1.2729 | 1.7645 |
| 100ms | 0.7933 | 1.1164 | 1.2455 | 1.7164 |
| 200ms | 0.7647 | 1.0427 | 1.1484 | 1.5305 |

Table 2.1-7: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different OW & PW, UE speed 60km/h, gradient boosting

| PW OW | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| 40ms | 0.8277 | 1.1634 | 1.3012 | 1.7950 |
| 80ms | 0.8081 | 1.1399 | 1.2685 | 1.7493 |
| 100ms | 0.8105 | 1.1309 | 1.2585 | 1.7296 |
| 200ms | 0.7971 | 1.0960 | 1.2145 | 1.6425 |

For illustration purposes, we also summarize our findings in Figure 2.1-3.

Figure 2.1-3: Average MAE comparisons for different OW & PW and different models, 60km/h

Next, we present the results for UE speed 120km/h.

Table 2.1-8: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different OW & PW, UE speed 120km/h, linear regression

| PW OW | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| 40ms | 0.9963 | 1.5125 | 1.7370 | 2.5733 |
| 80ms | 0.9998 | 1.5260 | 1.7485 | 2.5931 |
| 100ms | 1.0002 | 1.5285 | 1.7476 | 2.5968 |
| 200ms | 1.0275 | 1.5620 | 1.7930 | 2.6550 |

Table 2.1-9: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different OW & PW, UE speed 120km/h, random forest

| PW OW | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| 40ms | 1.0641 | 1.5646 | 1.7793 | 2.5689 |
| 80ms | 1.0419 | 1.5083 | 1.7048 | 2.4435 |
| 100ms | 1.0298 | 1.4828 | 1.6700 | 2.3920 |
| 200ms | 1.0143 | 1.4099 | 1.5727 | 2.1661 |

Table 2.1-10: Evaluation results for RRM temporal prediction Case A, case 2 – Mean absolute error (MAE) for different OW & PW, UE speed 120km/h, gradient boosting

| PW OW | 40ms | 80ms | 100ms | 200ms |
| --- | --- | --- | --- | --- |
| 40ms | 1.0736 | 1.5359 | 1.7325 | 2.4580 |
| 80ms | 1.0642 | 1.5191 | 1.7094 | 2.4293 |
| 100ms | 1.0609 | 1.5095 | 1.6942 | 2.4149 |
| 200ms | 1.0646 | 1.4873 | 1.6651 | 2.3237 |

For illustration purposes, we also summarize our findings in Figure 2.1-3.

Figure 2.1-4: Average MAE comparisons for different OW & PW and different models, 120km/h

The above simulation results analysis on RRM measurement prediction temporal domain case A led to the following observations and proposals.

Observation 1: For case A, in addition to observation window and prediction window, the impact of UE speed on MAE performance appears to be log-linearly proportional.

## 2.2	Temporal domain case B

The Oppo email discussion [POST127bis][022][AI mobility] outlines several L3 filtering options for RRM measurement prediction temporal domain case B:

Observation 1: For temporal domain prediction case B, the following filtering scheme can be considered: - Filtering option 1: L3 filtering is based on the L1 filtered result and the last L3 filtered cell result; - Filtering option 2: L3 filtering is based on the L1 filtered result if the last L3 filtered result is from prediction; - Filtering option 3: L3 filtering is based on the L1 filtered result and the L3 filtered result from last actual measurement. Take following as example: Figure 2.1.3-1: Temporal domain case B example 2 The 3 options also mean: - Filtering option 1: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T2; - Filtering option 2: L3 cell result at T3 is filtered based on the L1 filtered result at T3; in other words, not consider old L3 filtered result. - Filtering option 3: L3 cell result at T3 is filtered based on the L1 filtered result at T3 and L3 filtered cell result at T1 (i.e. the old L3 filtered cell result from last actual measurement);

While the email discussions have allowed companies to explore various filtering options for RRM measurement prediction in temporal case B, we think it is important to avoid broadening the scope of the SI too much. Thus, further refinement and down-selection of these options should be considered. Therefore, we do the following analysis by referring to Figure 2.1.3-1 of the email discussion.

Let L3T denote the L3-filtered cell result at time T, and L1T the L1-filtered measurement at time T. f∙ denotes the L3 filtering operation.

- Filtering option 1: L3T3=fL1T3,L3T2. The new L3 result at T3​ combines the current L1 measurement and the immediately previous L3 result at T2​ which may itself be from a prediction. Thus, if T2​​ was based on inaccurate predictions, that error is carried forward directly into T3​​.

- Filtering option 2: L3T3=fL1T3. The new L3 result at T3​ is only a function of the current L1 measurement, which prevents predicted error from being “inherited” from T2. However, it may cause bigger jumps at T3​ because the filter no longer has continuity with the previous L3 value.

- Filtering option 3: L3T3=fL1T3,L3T1. The new L3 result at T3​ combines the current L1 measurement and the last actual L3 result at T1​ (bypassing T2​ if T2​ is predicted). By always referencing the last actual L3 (at T1), the filter remains “anchored” to a real measurement. As soon as the system observes a new L1 measurement, it fuses it with a known-good reference, rather than a possibly stale or erroneous prediction.

Observation 2: Filtering option 3, i.e., obtaining new L3 result at T3​ by combining the current L1 measurement and the last actual L3 result at T1​, is closest to legacy implementation of L3 filter logic, which can also limit the accumulation of prediction errors. It also provides a balance between smoothness and correctness.

Proposal 1: RAN2 to prioritize Filtering option 3, i.e., obtaining new L3 result at some time T3​ by combining the current L1 measurement and the last actual L3 result at T1​, for RRM measurement prediction temporal case B.

In the following, we present the simulation results for temporal case B of RRM measurement prediction. As discussed above, we apply and prioritize Filtering option 3 to address missing values. The results will be analyzed across various speed categories and model types. We use the sample-and-hold method as a baseline for comparison against the performance of other AIML methods. The table of parameters are listed below.

Table 2.2-1: Simulation parameters for the 7-site scenario based on TR38.843 (FR1)

| Reported simulation assumptions | UE trajectory option | Option 3 |
| --- | --- | --- |
|  | UE trajectory boundary processing option | Option 3 |
|  | UE speed | 30/60/120 km/h |
|  | SoftLOS | Yes |
|  | Spatial consistency option | Option B |
|  | Number of TX beams | 4 |
|  | Number of RX beams | 1 |
|  | Measurement reduction rate | 50% |
|  | Sub-use-case | Case 2 (cell to cell) |
|  | Observation window (OW) | [160, 240, 320, 400, 800] ms |
|  | Prediction window (PW) | 40ms |
|  | L3 filtering option | Option 3 |
| Data Size (Number of Samples) | Training | 294MB |
|  | Testing | 126MB |
| AI/ML model input/output | Model input | Reduced serving cell RSRP measurements (following temporal case B example 2) |
|  | Model output | L3 filtered serving cell RSRP measurements |
| AI/ML model description | Model type | Linear regression, Random forest, Gradient Boosting |
|  | Model complexity in number of parameters | Linear regression, Random forest, Gradient Boosting: N/A |

The evaluation results are then presented in the sequel. We start with UE speed 30km/h.

Table 2.2-2: Evaluation results for RRM temporal prediction Case B, case 2 – Mean absolute error (MAE) for different OW, UE speed 30km/h

| OW Method | 160ms | 240ms | 320ms | 400ms | 800ms |
| --- | --- | --- | --- | --- | --- |
| Sample and hold | 0.6309 | 0.6221 | 0.6217 | 0.6236 | 0.6191 |
| Linear regression | 0.5774 | 0.5892 | 0.6001 | 0.6068 | 0.6390 |
| Random forest | 0.7154 | 0.7185 | 0.7247 | 0.7260 | 0.7426 |
| Gradient boosting | 0.7337 | 0.7353 | 0.7401 | 0.7442 | 0.7563 |

For illustration purposes, we also summarize our findings in Figure 2.2-1.

Figure 2.2-1: Average MAE comparisons for different OW and different models, 30km/h

Next, we present the results for UE speed 60km/h.

Table 2.2-3: Evaluation results for RRM temporal prediction Case B, case 2 – Mean absolute error (MAE) for different OW, UE speed 60km/h

| OW Method | 160ms | 240ms | 320ms | 400ms | 800ms |
| --- | --- | --- | --- | --- | --- |
| Sample and hold | 0.9941 | 0.9783 | 0.9747 | 0.9794 | 0.9909 |
| Linear regression | 0.9422 | 0.9527 | 0.9651 | 0.9825 | 1.0406 |
| Random forest | 1.1080 | 1.1116 | 1.1087 | 1.1150 | 1.1456 |
| Gradient boosting | 1.1531 | 1.1545 | 1.1513 | 1.1584 | 1.1799 |

For illustration purposes, we also summarize our findings in Figure 2.2-2.

Figure 2.2-2: Average MAE comparisons for different OW and different models, 60km/h

Last, we present the results for UE speed 120km/h.

Table 2.2-4: Evaluation results for RRM temporal prediction Case B, case 2 – Mean absolute error (MAE) for different OW, UE speed 120km/h

| OW Method | 160ms | 240ms | 320ms | 400ms | 800ms |
| --- | --- | --- | --- | --- | --- |
| Sample and hold | 1.4406 | 1.4222 | 1.4207 | 1.4116 | 1.4005 |
| Linear regression | 1.4542 | 1.4740 | 1.5112 | 1.5317 | 1.5990 |
| Random forest | 1.6213 | 1.6310 | 1.6510 | 1.6685 | 1.6577 |
| Gradient boosting | 1.6843 | 1.6953 | 1.7091 | 1.7318 | 1.7176 |

For illustration purposes, we also summarize our findings in Figure 2.2-3.

Figure 2.2-3: Average MAE comparisons for different OW and different models, 120km/h

Observation 3: For case B, the UE speed appears to be log-linearly proportional to the MAE performance.

Observation 4: For case B, AI/ML methods may have potential for performance improvement compared to the baseline method at lower speeds. However, as speed increases, AI/ML methods may not consistently outperform the baseline.

Proposal 2: Include the simulation results on RRM temporal case B in TR 38.744.

# 3	Conclusion

This document has made the following observations:

Observation 1: For case A, in addition to observation window and prediction window, the impact of UE speed on MAE performance appears to be log-linearly proportional.

Observation2: Filtering option 3, i.e., obtaining new L3 result at T3​ by combining the current L1 measurement and the last actual L3 result at T1​, is closest to legacy implementation of L3 filter logic, which can also limit the accumulation of prediction errors. It also provides a balance between smoothness and correctness.

Observation 3: For case B, the UE speed appears to be log-linearly proportional to the MAE performance.

Observation 4: For case B, AI/ML methods may have potential for performance improvement compared to the baseline method at lower speeds. However, as speed increases, AI/ML methods may not consistently outperform the baseline.

And proposed the following:

Proposal 1: RAN2 to prioritize Filtering option 3, i.e., obtaining new L3 result at some time T3​ by combining the current L1 measurement and the last actual L3 result at T1​, for RRM measurement prediction temporal case B.

Proposal 2: Include the simulation results on RRM temporal cases A and B in TR 38.744.

# 4	References

[1] R2-2408929, RRM Measurement Prediction Evaluation Results, Nokia, RAN2#127bis, Hefei, China, October 2024.

[2] TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR (Release 19)