3GPP TSG-RAN WG2 #129	R2-2500974

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.4

Source:	Ericsson

Title:	A-IoT Data Transmission and Other General Aspects

Document for:	Discussion/Decision

# 1	Introduction

In RAN#106, a new WI has been approved for Ambient IoT devices. The following RAN2 related objectives are capture in the WID:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

This paper discusses aspects of data transmission functionality and other remaining general aspects required for A-IoT D1T1 devices, such as MAC PDU format and control signaling, data (re)transmission for failure handling, segmentation/reassembly, AS ID handling and the use of IDs in general, as well as information related to message size of expected D2R transmission and command type information.

# 2	Discussion

## 2.1 	MAC PDU format and Control signaling

### 2.1.1	Octet vs. Bit aligned MAC PDU

In the previous meetings, some companies proposed the concept of not having octet aligned MAC PDU (or upper layer SDUs). It may be that the amount of control information to be sent is very low, just a few bits. In such case it might be beneficial to avoid octet alignment to avoid padding bits.

This discussion of course depends on how flexible and generic the TBS allocation is expected to be, also in future extensions.

Nevertheless, typically the length of a SDU is given in octet in NR. Avoiding octet-alignment means that now the length should be given in bits, which would increase the amount of signaling bits for indicating the TBS size. As a result, it is not obvious, without other assumptions, to determine if bit-alignment would lead to a more efficient MAC PDU encoding than octet-alignment.

Moreover, considering device limitations, memory works by addressing words of various length, typically a byte. Therefore, it might create excessive complexity on device side to have bit-aligned buffers and stored data. Also, current network design assumes octet alignment. Changing this may create excessive implementation effort on network side.

Bit-alignment may cause excessive complexity and implementation effort on both device and network size due to the normal functioning of memory and existing implementations.

Octet aligned MAC PDUs and Sup PDUs are used at AIoT MAC layer.

### 2.1.2	MAC PDU format

For MAC PDU design, it is worth listing possible A-IoT messages in both D2R and R2D transmission directions including field information and their use. Table 1 and table 2 below show possible messages to consider.

#### 2.1.2.1	D2R

Table 1: Possible D2R messages for MAC PDU design

In D2R direction, Msg1 contains RN16 and be distinguished from other D2R message; given that all other D2R messages are larger in size. It is therefore proposed that MAC PDU for Msg1 does not need any control information. If this construct is possible, the message size would be small. Then Msg3 and onwards contains upper layer data (NAS PDU) with possible variable size, and to that also UL control information such as a segmentation indication. In addition, Msg3 may need to carry security information in support of the challenge-response authentication mechanism if defined (as in SA3 TR). Figure 1 shows an example of MAC PDU content for Msg1 and other D2R messages, respectively. Note that this example does not comprehensively list options, e.g. for where a smaller optimized size could be made, due to e.g. a fixed size payload part or where a random ID is not needed.

Figure 1: Example of MAC PDU format for D2R messages.

MAC PDU for Msg1 contains only RN16 without control information.

For the D2R message from Msg3, given that we may have command responses, the reader can know from which device (or RN16 sender) the D2R message comes from, either by explicit addressing with a device ID (e.g., AS ID/RN16) inside D2R message itself, or by means of associating where the message is received with scheduling information. The latter would require less overhead given device ID is at least 16 bits. RAN2 should discuss and clarify that addressing information (AS ID) is not included in D2R message itself. A length field tis needed o indicate how many bytes the upper layer data part is unless a fixed size Device ID is used. In addition, as detailed in section 2.2, a 1-bit segmentation indication may be required to be included in the D2R message from Msg3, as detailed in Figure 1. This naturally depends on the TB sizes to assume additionally.

RAN2 to clarify the need of addressing information inside a D2R message from Msg3 and on.

MAC PDU for D2R message other than Msg1 includes:

- A length field indicating the length of the upper layer data part. Discuss the size of this field (e.g., 7 bits)

- A 1-bit segmentation indication

- Upper layer data (e.g., device ID, command reply, or other NAS data).

#### 2.1.2.2 	R2D

In the R2D transmission direction, there are many more messages over the air interface, as exemplified in Table 2. A R2D message type field is useful to allow device(s) to identify and parse the received message. With such a message type field, we support a leaner protocol structure if we have a low number of fixed formats, i.e., fewer flags and length fields resulting in lower header overhead as well as less processing at device side. Currently, there are approx. 7 types of R2D messages, and as a starting point 3 bits seems sufficient for a message type field.

A R2D message type field is used to identify R2D messages and their corresponding format.

It is still unclear what the content of the A-IoT paging message is, including if this is impacted from subsequent paging. For example, the configuration and scheduling information regarding access occasion, frequency offset (to support FDMA + TDMA in D2R transmission), may be either carried by MAC, by PHY control information or both. RAN2 should wait for RAN1 progress on this aspect. The same applies to the R2D message used to update/adjust configurations for all devices in an access/paging/inventory round. In addition, RAN2 can assume that that least the R2D message used for synchronization purposes (Qrep-like) is part of PHY layer control signaling.

Table 2: Possible R2D messages for MAC PDU design

For A-IoT paging of an access round, RAN2 wait for further input from other WGs (RAN1, SA3, SA2) regarding control information to be included in the MAC PDU.

RAN2 assume the R2D message for sync (Qrep-like) is L1 control signaling. Send an LS to RAN1 to inform this.

Figure 3: Example of MAC PDU for R2D messages.’

Msg2 used to echo the RN16 in response to Msg1 can have its own format. As Msg2 may contain more than one RN16 value, there needs a field to indicate number of RN16s. Size of this field can be discussed, e.g., 5 bits. In addition, RAN2 needs to wait for progress from RAN1 to see if any L2 control signaling is needed, e.g., to facilitate a device to process only some parts meant for it rather than processing the whole PDU. For example, if a relation to FDMA index to selected RN16 would be defined. That is, if valid RN16s are indicated in association with a FDMA index in the downlink (msg2) message, a device needs only to parse the RN16 related to the FDMA occasion where it had transmitted. It remains open if any or what information for scheduling Msg3 is needed in the Msg2 MAC PDU.

MAC PDU for Msg2 used to echo Msg1 includes:

- R2D message type

- A field indicating the number of RN16s. Discuss the size of this field.

- FDMA index. FFS if this information is available at L1 or MAC.

- One or multiple RN16s

- FFS if any L2 control information to facilitate processing of Msg2 at device and scheduling information for Msg3.

Before Msg3 is successfully delivered, there can be a need for some other R2D messages or indications to support a feedback mechanism, i.e., to indicate explicitly or implicitly to a device to perform retransmission of the previous D2R message, or to perform re-access, or to transmit the next D2R segment (as in Table 2). These cases can be combined in one MAC PDU format. In these cases there needs to be a device ID in such an R2D message indicating the targeted device. Given that the reader has not successfully received the complete upper layer device ID (in Msg3), this device ID can only be the RAN16/handle. RAN2 can discuss if 2 bits are sufficient for such an indication. One example of this R2D message is illustrated in Figure 2.

MAC PDU for R2D messages for feedback mechanism may include:

- R2D message type

- 16-bit ID for addressing information (e.g., AS ID/RN16)

- A field to indicate the usage, i.e., whether device needs to retransmission, re-access, or continue with next segment transmission.

MAC PDU for the DL message from Msg4 is to carry DL NAS data, i.e., command request received from CN and possibly other NAS information. The size of DL NAS data in this R2D message is likely variable. As a result, a length field is needed in the MAC sub-header to indicate the size of the MAC SDU. We assume 7 bits are sufficient for the length. For addressing purpose, a 16-bit AS ID/RN16 is also needed. Other potential DL control information can be decoded by using any of unused bits for future extension.

MAC PDU of R2D message from Msg4 includes:

- R2D message type

- 16-bit ID for addressing information (e.g., AS ID/RN16)

- A length field indicating the size of the upper layer data part. Discuss if 8 bits are sufficient.

Note that we assume that some of the formats e.g. subPDU, header fields for messages may in some cases be the same where the type field(s) may indicate the content, existence and use.

RAN2 waits for SA3 progress regarding if/what security information needs to be included in R2D and D2R message(s).

## 2.2	Segmentation and Data retransmission for Failure Handling

In here retransmission is to allow the A-IoT device to deliver the D2R message again in the same access occasion, possibly including a D2R segment without the need of performing the random access procedure again (re-access). To support this, one solution from the study is that the reader repeats the R2D transmission of Msg2 to specific devices whose D2R transmissions were not successfully received by the reader. This can apply also to the case of transmission pairs after Msg2 and Msg3, e.g., in case of a command procedure with multiple steps.

For the D2R segmentation mechanism, an indication is included to let the reader know if segmentation is used and if the PDU is the last segment. Details to the usage of the indication and its resulting size as well as how to realize this at MAC needs further discussion. For the segmentation, the current TR38.769 mentions as below:

-	The sequence number, the segment number and the number of segments are not supported;

-	An indication is used to indicate to the reader on whether the data is segmented and whether the MAC PDU is the last segment. It needs to be further discussed on the size of this indication (one or two bits) and the corresponding further details;

-	It is beneficial for the reader to be able to trigger a re-transmission of a segment;

-	It is assumed that the A-IoT device will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all segment(s) are stored in upper layer(s).

The overall mechanism is “Stop & Wait”, meaning that the transmitter waits for the receiver before continuing with a transmission of a next segment, or if possibly retransmit the (previous) segment.

For the indication, if the data is not segmented, this is equivalent to the first segment being also the last segment. On the other hand, if the device indicates that this is not the last segment, it is clear for the reader that this data is a segment. Therefore, only 1 bit is necessary to support segmentation, as also detailed in the Msg3 MAC PDU format in previous section.

The device sends a 1-bit indication indicating whether the current transmission is the last segment or not.

Failure handling

Figure 2 a) and b) show the normal functioning of the segmentation procedure under these assumptions, and how the device keeps track of the offset in the buffer (c.f. pointer or “ptr”). It is assumed that the trigger from the reader contains a New Data Indicator (NDI) that is set to 1 to request the next segment or 0 to request the retransmission of the previous one.

It is important to notice that if we assume that each segment is sent upon a trigger from the reader (e.g.: UL Grant + ACK/NACK) two failures can happen: either the device receives the UL Grant but the reader does not receive the segment, or the device does not receive the UL Grant. In both cases, the reader will not detect any transmission in the UL resources reserved for the segment transmission, and thus in both cases the reader should behave in the same manner, for instance by sending a NACK. The device updates the buffer pointer “ptr” when the previous transmission has been acknowledged (NDI=1)

The reader does not have a way to distinguish if a failure was caused by missing reception of the UL Grant or missing reception of the data segment.

The problem with this is represented in Figure 2 where in c) the device correctly receives the trigger message (and updates its buffer pointer) but the transmission of the segment fails. After NDI=0 has been sent the device just retransmits the previous segment and everything works as intended. On the other hand, if the trigger transmission fails (case d)) the device does not update the buffer pointer, so when the next NDI=0 is transmitted, the device would retransmit segment i, but the reader received segment i and is now expecting a retransmission of segment i+1. Therefore, in the reader buffer segment I will be appended twice.

| Device Pseudo-code: On Upper Layer Trigger received: - Build Reply - ptr := 0 TBS_prev := 0 - Process UL Grant (below) On UL Grant (TBS=x) received: - If NDI==1: - ptr := ptr + TBS_prev - if ptr == end_of_buffer: - Completed - else: - TBS_prev := x - Transmit bit from ptr to ptr + x-1. a) | b) |
| --- | --- |
| c) | d) |

Figure 2: Simple segmentation procedcure and error cases: a) pseudo code at device side, b) normal functioning, c) failed data transmission error case d) failed trigger transmission error case

Here we consider that the UU design is so that an UL Grant is sent with a robust MCS so that case d) is not actually possible, or at least unlikely as the case is otherwise that the SINR anyway is not sufficient to receive the UL Grant correctly, and the device is considered out of coverage.

Additionally, there may be a CRC assumed to be included in the overall transmission used as a mechanism to check the validity of the received data once the last segment has been received.

Given this, it can be considered a rare occurrence, and over the population of devices it would not be a significant issue.

It is also worth noticing that passive devices that will likely have enough energy for the whole duration of the session, will not have issues in losing the stored data due to lack of energy.

On the other hand, active devices may in fact lose energy during the transaction. In this case, assuming NVM is used for this feature, all information regarding the ongoing segmentation procedure will be lost. The reader will likely detect a failure and may decide to try again later. If the device is never able to transmit all the segments it may be considered simply out of coverage because unable to operate. Note that this will not be different from the case of a single segment (of the same size).

The above failures should therefore come with a simple out of coverage handling, as defining signaling and UE behavior for these cases would come with unnecessary signaling and complexity.

A device not able to complete the transmission of all the segments should be considered “out of coverage” or “unable to operate”.

For a failed data transmission, retransmission can be triggered with a repeated Msg2 transmission. This was agreed during SI phase and particularly discussed as a solution for non-segmented transmissions. However, the same solution can be applied for failed segment transmissions, i.e., reader indicates retransmission via Msg2 for failed segment transmission with new grant parameters.

If the segment is other than the last segment, we believe following behavior is reasonable.

- For transmission success, the reader does not need to indicate any explicit ACK signalling as long as there is message defined for granting resources for next segment.

- For indicating retransmission of a segment, the reader can send Msg2 again to trigger retransmission of the given segment with a simple NDI-type indication as in legacy.

- The NDI can be always present and would cater for both segmented and unsegmented cases.

- If a device performs re-access the transmission starts from a 1st segment if segmentation is perfomed.

For the case with segmented transmission, the device should transmit segments in order.

A bit in Msg2 response is introduced to indicate retransmission grant for previous segment or new transmission grant for next segment.

For the case with segmented transmission, if re-access is triggered, the device begins transmission from a 1st segment in a new occasion if contention-resolution is successful.

## 2.4	Device Addressing and AS identifier handling

Regarding AS ID, it is currently mentioned in TR38.769:

From higher layer perspective, it is assumed that "AS ID" (if defined according to the design in clause 6.1) is used at least for purpose of D2R scheduling and R2D reception. From higher layer perspective, it is assumed that this "AS ID" should be a short AS layer ID, rather than the full upper layer device ID. It needs to be further discussed if this "AS ID" can be based on partial upper layer device ID. It needs to be further discussed on the length of this "AS ID". From higher layer perspective, following options are possible for this "AS ID" (it is aimed to define one common design for all access procedures in clause 6.3.4, if technically possible):

-	Option 1: a random ID (if used in first D2R message) can be reused;

-	Option 2: the reader assigns this "AS ID" to the device. It needs to be further discussed via which R2D message.

-	Option 3:	It is up to the reader whether to reuse the random ID (if used in first D2R message) as the "AS ID" or to assign a new "AS ID". It needs to be further discussed via which R2D message.

NOTE 1:	The further down-selection for "AS ID" needs to consider the involved message and whether the reader needs to handle the collision.

The UE should be able to determine if a message received on the shared DL channel is directed to itself or not.

In this section, we call “address” the information needed to distinguish transmission from/to different devices from MAC perspective. (i.e.: it is out of scope higher-layer perspective such as NAS, Application, or inventory as a whole).

There are two main problems to solve regarding the device addressing:

- Which information is used as address (including considerations on scope of the address and collision probabilities)

- How the address is used

### 2.4.1	Which information is used as address

In general, the address should be a piece of information that is agreed between the reader and device (provisioning) and for which its validity is clear for both nodes. In the last meeting it was also agreed that this address should be “short” although it is still to be studied if it should be based on part of the device ID or not.

In assessing each possible option, we have to consider the following aspects:

- Security and Privacy: can the information used as address be shared with the involved nodes? Should it be used in its encrypted form or can be used in clear text?

- Uniqueness: is the address unique within a certain area or time scope? Could there be unacceptable collisions with other devices?

- Provisioning: how is the address provisioned (i.e.: agreed between device and network)

- Usage of memory: should the address be kept in volatile registers or written in NVM? How often? This affects energy consumption and complexity.

RAN2 to determine which information should be used as address based on security, privacy, uniqueness, provisioning, and usage of NVM.
FFS other aspects.

We can start by assessing the uniqueness of the address, meaning that within a certain scope in time and space there should not be excessive collisions between users.

The TR mentions 3 options on how the address may be generated:

- Option 1, i.e., randomly generated by the device: in this case there is a collision probability which depends on how many devices are sharing the same address scope and how many bits compose the address.

- Option 2, i.e., allocated by the network AS: in this case it is ensured that the address is unique, but it requires additional signalling overhead for the address provisioning. In this case the maximum number of devices that can share the address scope is exactly 2b.

- Option 3, i.e., reader decides if the random ID is reused or to assign a new AS ID. This is flexible but can also ensure the uniqueness given that reader knows if the random ID has been used by any other device in the system. Given that the Msg2 anyway needs to carry an RN16 to acknowledge the device sending corresponding Msg1, if the reader allocates a new AS ID, there needs another R2D message for such allocation.

It is worth noticing that if the address is randomly generated, there is a probability of collision. Regardless of how negligible it may be, the system should still be able to recover from this.

A classical example is the contention resolution ID in legacy. There is a non-zero probability that two devices select the same RACH resource and random ID in Msg3 and they would both win the contention without reader/network knowing this. This case is typically considered negligible, but if this happens the network would shortly after verify the 5G-TMSI sent in Msg3 and Msg5 understanding that something went wrong and terminating the procedure. A similar recovery should be considered for A-IoT.

A network-assigned address guarantees uniqueness but requires signalling overhead for its provisioning.

A randomly generated address, or address calculated as function of the Device ID or an already assigned CN ID, does not require provisioning overhead but has a collision probability. Regardless of how negligible this probability is, the system should be able to recover from this.

If within a certain scope (time or space) there are N devices with a randomly generated address composed by b bit, the probability that a collision between at least 2 devices happens can be estimated following the “birthday problem” approach: the probability of a collision is one minus the probability that in all possible pairs of devices they have a different address.

This probability is:

pc=1-1-12bNN-12

Figure 3: Collision probability for 16 bit randomly generated address as function of the number of competing users.

Considering that the AS ID, according to the discussion, will have a scope limited to the few devices that perform the random access concurrently, the number of competing devices will be relatively low: the number of frequency shifts for Msg1 and possibly a few TDM occasions. We can imagine this number will be around 16. This means that in each access occasion there is a probability not greater than 0.3% that at least two devices collide. In an inventory procedure for 10k devices there would be roughly 600 access occasions, so the probability that a collision happens at least in one access occasion is

pcIV=1-1-pcNround≃84%

In conclusion, despite the collision probability being relatively low, in a long inventory these chances accumulate so that it’s very likely that an address collision happens and should be handled.

Given the only two use cases in Rel-19 (inventory, and inventory + command), it is observed that even if collision happens, it may be possible to recover from upper layer processing thanks to security protection at A-IoT NAS. For example, if two devices happen to both win contention resolution with the same RN16 and are allocated as AS ID, the command request is intended for only one device, but is received by the two devices. However, only one of the devices succeeds in processing at A-IoT NAS layer. Thus, in worst case, there is a potential waste of energy for unintended devices. Therefore, it is proposed to adopt the simplest option, i.e., AS ID is promoted from the RN16 randomly generated by the device (option 1).

AS ID is promoted from the RN16 randomly generated by the device (option 1).

### 2.4.2	Ways to support addressing

Assuming that an address has been agreed in this section we discuss how it may be used to univocally determine which device is supposed to decode a certain R2D transmission or which device transmitted in a certain D2R resource.

There are two general ways. With explicit addressing a data transmission will contain the address as part of control information. The receiver of this message will decode the entire message, including the address control information. Otherwise with implicit addressing some separate signalling, containing the address, will indicate which device is supposed to receive/use some upcoming resources. In this case the data transmission itself will not contain the address.

Which option is more efficient depends on the expected message exchange. For instance, explicit addressing requires larger data transmission but no additional signalling, while the opposite is true for implicit addressing, therefore explicit addressing may be more suited for short sessions, while implicit for longer sessions.

Within the explicit addressing, the address may be included at different layers. For instance, if PHY layer will include a control part, this could contain the address. Otherwise, the address may be included as a MAC CE or part of the MAC subheader or even added at upper layers.

Within the implicit addressing, we distinguish one-shot and persistent implicit addressing.

One-shot implicit addressing consists in providing an UL/DL Grant containing the address and a description of a specific resource which will be used by the indicated device for a single transmission. This is analogous of legacy PDCCH followed by a PDSCH or PUSCH transmission.

Figure 5: Basic functioning of one-shot implicit addressing.

Persistent implicit addressing consists in a DL control signal blocking until further notice all the resources (or all resources in a specific frequency range in case of FDMA) for a specific device. In this case all data transmissions in this reserved band will belong to the same device. This option is more similar to how RFID works where once a slot begins with QueryRep all transmissions will belong to the same user until the following QueryRep starts a new slot.

Figure 6: Basic functioning of persistent implicit addressing.

Considering current progress on other agenda items, such as RACH, it is likely that either one-shot or persistent implicit addressing can fit nicely in the agreed mechanism. So, we propose to focus more on these.

Resources can either reserved via an UL/DL grant containing the device address, or by reserving indefinitely a frequency resource using an address until a further reservation is made.

## 2.5	Other MAC aspects

### 2.5.1	Expected D2R message size and command type information

In last RAN2 meeting, there was an agreement on visibility:

- The reader needs to know whether a response is expected and expected D2R message size.   FFS how the reader gets this information.  Wait for SA2 LS response on expected D2R message size to resolve the FFS.    FFS if command type is explicit, if it can be inferred from expected D2R message size if available.

SA2 has concluded that AIOTF may provide the approximate D2R message size based on AF request (as in TR23.700-13):

e.	The AIOTF may provide the following assistance information to AIoT RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

NOTE 6:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

NOTE 7:	Further assistance information can be added during the normative phase and in cooperation with other WGs if necessary.

In addition, SA2_166AH meeting also agreed that the inventory request is not an NAS message (S2-2501244) and command-only is not supported in Rel-19 (S2-2501249). Thus, to address the FFS on how the reader gets this information, we assume that assistant information is provided by AIOTF to RAN reader as NG-AP information rather than part of NAS message.

From SA2 progress, assistant information including approximate D2R message size is provided by AIOTF to RAN reader not as part of A-IoT NAS message.

Given that command-only is not supported in Rel-19 and only three command cases (read, write, and permanent disable) are considered, for inventory + command procedure, the expected D2R message size already tells the reader whether a command request is for read operation (with expected command reply on the data from device memory) or the reply is just an acknowledgment of a request for write operation or disable operation. In case of write and disable command, there is no need to distinguish the command type given that the D2R message in response to the command is in relatively small.

Command type can be inferred from expected D2R message size. No need of explicit assistant information regarding command type from AIOTF.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Bit-alignment may cause excessive complexity and implementation effort on both device and network size due to the normal functioning of memory and existing implementations.

Observation 2	The reader does not have a way to distinguish if a failure was caused by missing reception of the UL Grant or missing reception of the data segment.

Observation 3	A network-assigned address guarantees uniqueness but requires signalling overhead for its provisioning.

Observation 4	A randomly generated address, or address calculated as function of the Device ID or an already assigned CN ID, does not require provisioning overhead but has a collision probability. Regardless of how negligible this probability is, the system should be able to recover from this.

Observation 5	From SA2 progress, assistant information including approximate D2R message size is provided by AIOTF to RAN reader not as part of A-IoT NAS message.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Octet aligned MAC PDUs and Sup PDUs are used at AIoT MAC layer.

Proposal 2	MAC PDU for Msg1 contains only RN16 without control information.

Proposal 3	RAN2 to clarify the need of addressing information inside a D2R message from Msg3 and on.

Proposal 4	MAC PDU for D2R message other than Msg1 includes:

	A length field indicating the length of the upper layer data part. Discuss the size of this field (e.g., 7 bits)

	A 1-bit segmentation indication

	Upper layer data (e.g., device ID, command reply, or other NAS data).

Proposal 5	A R2D message type field is used to identify R2D messages and their corresponding format.

Proposal 6	For A-IoT paging of an access round, RAN2 wait for further input from other WGs (RAN1, SA3, SA2) regarding control information to be included in the MAC PDU.

Proposal 7	RAN2 assume the R2D message for sync (Qrep-like) is L1 control signaling. Send an LS to RAN1 to inform this.

Proposal 8	MAC PDU for Msg2 used to echo Msg1 includes:

	R2D message type

	A field indicating the number of RN16s. Discuss the size of this field.

	FDMA index. FFS if this information is available at L1 or MAC.

	One or multiple RN16s

	FFS if any L2 control information to facilitate processing of Msg2 at device and scheduling information for Msg3.

Proposal 9	MAC PDU for R2D messages for feedback mechanism may include:

	R2D message type

	16-bit ID for addressing information (e.g., AS ID/RN16)

	A field to indicate the usage, i.e., whether device needs to retransmission, re-access, or continue with next segment transmission.

Proposal 10	MAC PDU of R2D message from Msg4 includes:

	R2D message type

	16-bit ID for addressing information (e.g., AS ID/RN16)

	A length field indicating the size of the upper layer data part. Discuss if 8 bits are sufficient.

Proposal 11	RAN2 waits for SA3 progress regarding if/what security information needs to be included in R2D and D2R message(s).

Proposal 12	The device sends a 1-bit indication indicating whether the current transmission is the last segment or not.

Proposal 13	A device not able to complete the transmission of all the segments should be considered “out of coverage” or “unable to operate”.

Proposal 14	For the case with segmented transmission, the device should transmit segments in order.

Proposal 15	A bit in Msg2 response is introduced to indicate retransmission grant for previous segment or new transmission grant for next segment.

Proposal 16	For the case with segmented transmission, if re-access is triggered, the device begins transmission from a 1st segment in a new occasion if contention-resolution is successful.

Proposal 17	RAN2 to determine which information should be used as address based on security, privacy, uniqueness, provisioning, and usage of NVM. FFS other aspects.

Proposal 18	AS ID is promoted from the RN16 randomly generated by the device (option 1).

Proposal 19	Resources can either reserved via an UL/DL grant containing the device address, or by reserving indefinitely a frequency resource using an address until a further reservation is made.

Proposal 20	Command type can be inferred from expected D2R message size. No need of explicit assistant information regarding command type from AIOTF.