3GPP TSG-RAN WG2 Meeting #129			R2-2501015

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.4.2

Title:	Consideration on DSR enhancement for XR

Source:	CMCC

Document for:	Decision

Introduction

In the last meeting, RAN2 discusses some details on DSR, and has the following agreements [1].

Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

This contribution discusses some remaining details of DSR enhancement.

Discussion

  - Terminology

To establish conceptual clarity in Rel-19 standardization and explicitly distinguish from the single-threshold DSR mechanism defined in TS 38.321 Rel-18, we propose the term "Refined DSR" (analogous to Refined BSR) or extended DSR for the enhanced delay status reporting

Proposal 1: Introduce a terminology, e.g., refined DSR or extended DSR for Rel-19 DSR that supports multiple pairs of remaining time and buffer size.

  - Segmented/Truncated Refined DSR

With the introduction of multiple pairs of remaining time and buffer size, the length of the refined DSR will increase. Supposing that each LCG has 3 reporting thresholds and every portion has a non-zero buffer size, the size of Refined DSR MAC CE plus its subheader could reach up to 408 bits.

Due to the increased length of refined DSR, it might be even harder for gNB to allocation enough uplink radio resource. In legacy, if an SR is triggered by pending BSR, gNB, by implementation, will allocate resource enough for at least a BSR. The BSR can be shorter than DSR and there are truncated BSR MAC CE formats to ensure that at least part of BSR can be sent to gNB.

Similar, gNB should also allocate enough resource for refined DSR if UE reports an SR which is triggered by refined DSR. However, it might be impossible for gNB to know the size of refined DSR in advance as the length of it can be hard to estimate.  If the UL grant size is less than the size of refined DSR, UE may send padding BSR only and have to wait for next UL grant that can accommodate the refined DSR.

Consequently, if the refined DSR is delayed because the gNB fails to allocate adequate uplink resource, it could lead to the discarding of delay-critical data.

Observation 1a: The introduction of multiple reporting thresholds makes the length of refined DSR longer and unpredictable.

Observation 1b: Currently, if the allocated UL grant is insufficient to accommodate the intact refined DSR MAC CE, the UE must await subsequent UL grants, causing a delayed DSR.

Observation 1c: A delayed DSR may cause delay-critical data being discarded.

Considering that padding DSR was not introduced in Rel-18 and Truncated DSR was not defined, RAN2 may need to discuss the UE behaviour when the UL grant cannot accommodate a complete Refined DSR.

The simplest option is to temporarily withhold the sending of pending DSR and wait for gNB to allocate a UL grant that is large enough to accommodate the complete pending Refined DSR. However, this option carries some uncertainty, as it is possible that gNB may never allocate sufficient resources, preventing the UE from ever sending the DSR.

Another approach is to introduce truncated or segmented refined DSR. When the UL grant is insufficient to accommodate the intact refined DSR, the truncated or segmented refined DSR can be used to send an initial part of refined DSR MAC CE. Subsequently, UE may cancel the pending DSR, which is truncated DSR, or UE may keep the pending DSR and continue sending the remaining DSR, which is segmented DSR.

As can be inferred, the truncated DSR is simpler for UE implementation but the DSR is incomplete. While the segmented DSR may introduce segmentation indication but the DSR that gNB receives are intact. However, both truncated and segmented DSR will notify gNB that there is an DSR in the first uplink transmission after DSR triggers an SR.

Proposal 2a: RAN2 should specify UE behavior if UL grant cannot accommodate the intact refined DSR, FFS whether it can apply to legacy DSR.

Proposal 2b: If UL grant is not sufficient for the intact DSR, RAN2 may discuss the following options:

Opt.1: As legacy, UE keeps the pending DSR and waits for new UL grant.

Opt.2: UE sends Truncated/Segmented Refined DSR, FFS whether to send the remaining Refined DSR segmentations.

We slightly prefer Opt.2 since Opt.1 will also cause discarding of delay-critical data.

  - Refined DSR MAC CE format

Similar with legacy DSR and BSR, only one DSR should be included in a MAC PDU for efficiency considerations. It would avoid the overhead caused by multiple DSR MAC CEs. Additionally, multiple DSRs could potentially cause ambiguity in both UE and gNB MAC scheduler.

Proposal 3: As legacy, a MAC PDU contains at most one DSR MAC CE no matter it’s refined or not.

With at most one DSR in a MAC PDU, RAN2 should focus on a unified DSR MAC CE format that can support UE to report delay status for LCG consisting of multiple LCHs configured with both refined and legacy DSR, i.e., the refined DSR MAC CE format should also be able to interworking with legacy DSR.

Fortunately, the Rel-18 legacy DSR MAC CE format can be compatible with both refined DSR and legacy DSR with some minor modifications.

Proposal 4a: To avoid multiple DSR MAC CEs in one MAC PDU, the refined DSR MAC CE format should be compatible with legacy DSR MAC CE, i.e. the refined DSR MAC CE format can support DSR configured with single or multiple reporting threshold(s).

For that, an indication is necessary to indicate whether the pair of remaining time and buffer size belongs to the same LCH or not in the refined DSR MAC CE format. And before each pair of remaining time and buffer size field, the legacy DSR MAC CE has an R field as the reserved bit. This R field can be reused to indicate whether the consecutively distributed pairs of remaining time and buffer size belongs to the same LCG, i.e., whether the corresponding LCG is configured with multiple reporting threshold and the LCG has multiple pairs of nonzero value to report in this DSR.

Proposal 4b: Reuse the R field in legacy DSR MAC CE to indicate whether there are multiple pairs of remaining time and buffer size for the corresponding LCG.

The following is a proposed refined DSR MAC CE format where the R field is replaced by the M field which stands for Multiple reporting thresholds. The final terminology can be further discussed.

The proposed refined DSR MAC CE can format support both LCGs configured with Refined DSR and LCGs configured with Legacy DSR. For example, M=0 stands for that the corresponding pair of reporting values is a new LCG, M=1 stands for otherwise.

Figure 1: Proposed DSR MAC CE

The following is an example of description for the introduced M field:

- M: This field is present only if the corresponding LCG is configured with multipleReportingThresholds and the buffer size indicated by the corresponding reporting threshold of the corresponding LCG is not zero; otherwise, this field is reserved and set to 0. If present, the T field set to 1 indicates that the pair of remaining time and buffer size is the first or the only one of the corresponding LCG, while the BT field set to 0 indicates otherwise;

Another threshold related issue is the maximum number of reporting thresholds that can be configured for an LCG. Since the portion with a buffer size of zero no longer needs to be reported in the DSR, the DSR length can be effectively shortened. As a result, the limitation on the maximum number of reporting thresholds can be appropriately relaxed.

Based on the contribution of RAN2-127bis, there are proposals supporting 2, 3 and 4 reporting thresholds per LCG. As kind of a majority view, it is proposed that there will be at most 5 reporting thresholds for each LCG. Although the proposed DSR MAC CE can support a lot of reporting thresholds per LCG, configuring too many thresholds would be inappropriate due to the overhead of the DSR itself. RAN2 can further discuss to reduce the maximum number of reporting threshold per LCG.

Proposal 5: As a start, RAN2 agrees that there should be no more than 5 reporting thresholds per LCG. RAN2 can further discuss whether to reduce the maximum number of reporting thresholds.

From our perspective, 3 reporting threshold is a good compromise.

  - UE capability

Considering that refined DSR is actually an enhancement of legacy DSR, and it will be also reported in UE capability. And some time gNB may just need very basic information reported in legacy DSR, it may only allow UE to report legacy DSR. Hence, UE should not only support refined DSR.

Proposal 6: If UE supports Refined DSR, it should also support Rel-18 legacy DSR by mandatory.

Conclusion

This contribution discusses some remaining issues on DSR enhancement, and propose the following,

Terminology:

Proposal 1: Introduce a terminology, e.g., refined DSR or extended DSR for Rel-19 DSR that supports multiple pairs of remaining time and buffer size.

Truncated or segmented refined DSR:

Observation 1a: The introduction of multiple reporting thresholds makes the length of refined DSR longer and unpredictable.

Observation 1b: Currently, if the allocated UL grant is insufficient to accommodate the intact refined DSR MAC CE, the UE must await subsequent UL grants, causing a delayed DSR.

Observation 1c: A delayed DSR may cause delay-critical data being discarded.

Proposal 2a: RAN2 should specify UE behavior if UL grant cannot accommodate the intact refined DSR, FFS whether it can apply to legacy DSR.

Proposal 2b: If UL grant is not sufficient for the intact DSR, RAN2 may discuss the following options:

Opt.1: As legacy, UE keeps the pending DSR and waits for new UL grant.

Opt.2: UE sends Truncated/Segmented Refined DSR, FFS whether to send the remaining Refined DSR segmentations.

Refined DSR details:

Proposal 3: As legacy, a MAC PDU contains at most one DSR MAC CE no matter it’s refined or not.

Proposal 4a: To avoid multiple DSR MAC CEs in one MAC PDU, the refined DSR MAC CE format should be compatible with legacy DSR MAC CE, i.e. the refined DSR MAC CE format can support DSR configured with single or multiple reporting threshold(s).

Proposal 4b: Reuse the R field in legacy DSR MAC CE to indicate whether there are multiple pairs of remaining time and buffer size for the corresponding LCG.

Proposal 5: As a start, RAN2 agrees that there should be no more than 5 reporting thresholds per LCG. RAN2 can further discuss whether to reduce the maximum number of reporting thresholds.

UE capability:

Proposal 6: If UE supports Refined DSR, it should also support Rel-18 legacy DSR by mandatory.

References

[1]	R2-2410915, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei)