3GPP TSG RAN WG2 Meeting#129	   							                                       R2-2501064

Athens, Greece, 17th - 21st February 2025

Agenda item:	8.2.2

Source:	ETRI

Title:	Discussion on Ambient IoT Paging

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, we reached agreements on the A-IoT paging as follows[1].

Agreements on paging

- An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.

- Capture the scenario that different readers may page the same device for same service request.  We will discuss this in WI phase (if in scope of WI) and consider progress from all the WGs

- Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

This contribution discusses the A-IoT paging, with a particular focus on content of paging and structure of paging message.

Discussions

Content of A-IoT Paging

Device ID

The paging message may include the following device IDs[2].

- The A-IoT paging message containing an identifier of a single A-IoT device

- The A-IoT paging message containing a group ID that maps to multiple A-IoT devices

- The A-IoT paging message containing multiple identifiers of A-IoT devices

These IDs are delivered to the reader by a Core Network (CN) functional entity upon an A-IoT service request. The device ID is then included in the paging message and transmitted to the device over the A-IoT radio interface. A candidate A-IoT device ID as presented in TR 23.700[3] consists of a Home Network Identifier, an Owner Identifier, and an Instance Identifier. Additionally, it may optionally include a 3rd Party Defined Identifier as shown in Figure 1.

- Home Network Identifier: an identifier used to identify the home MNO;

- Owner Identifier: an identifier used to identify a 3rd party who sends service requests to trigger 5GC to perform Ambient IoT service operation;

- Instance Identifier: an identifier used to identify a specific Ambient IoT device owned by the 3rd party.

Figure 1. Structure of Ambient IoT Device ID and optional 3rd Party-defined Identifier [3]

Such a device ID may be similar to the 5G-SUPI (Subscription Permanent Identifier) and its maximum length would be 64 bytes if NSI-based SUPI is considered. One key consideration when including this type of device ID in a paging message is its length. Therefore, it is necessary to discuss whether the device ID received from the CN should be used as-is in the paging message or if an alternative ID format should be used.

Proposal 1. RAN2 is kindly asked to discuss whether the device ID received from the CN should be used as-is in the paging message or if an alternative ID format should be used.

D2R Resource configurations

A device may perform either Random Access (RA) or Contention-Free Access (CFA) in response to paging. To support this, the paging message should include the resource configuration for D2R transmission. If a fixed-size D2R message is considered, a fixed-size resource allocation can be applied. However, when variable-sized upper-layer data is considered, the resource allocation should also be variable. Additionally, the resource configuration may account for both cases where the resources are allocated sequentially and where they are distributed across different time-domain resources. Therefore, the D2R resource configuration should include the index of the starting resource and the number of allocated resources.

In A-IoT D2R transmission, resources are structured as Access Occasions (AOs). Consequently, the paging message should include the AO(s) information assigned to each device ID. This AO(s) information may consist of the starting AO and the number of allocated AOs.

A-IoT access can be categorized into Contention-Free Access (CFA) and Contention-Based Random Access (CBRA).

For CFA, the paging message should provide dedicated resource information, such as the access occasion (AO) required for the device’s D2R transmission. Accordingly, the D2R resource configuration for CFA includes the starting AO and the number of allocated AOs.

For CBRA, since slotted ALOHA-based random access is considered, the paging message should provide the necessary information for the device to randomly select an access occasion (AO). Thus, the D2R resource configuration for CBRA includes the starting AO, the number of allocated AOs, and additional information for random AO selection.

The Figure 2 illustrates an example of D2R resource allocation.

Figure 2. Example of D2R resource allocation

Proposal 2. As D2R resource configuration, following information is required in paging message.

- For CFA: information indicating the starting of access occasion and the number of access occasion(s).

- For CBRA: information indicating the starting of access occasion, the number of access occasion(s), and information for random AO selection.

Paging ID

The CN can transmit the same A-IoT service request to a device through multiple readers. In this case, the device may receive multiple paging messages from different readers for the same service and repeatedly perform the corresponding access procedures, resulting in increased battery consumption and unnecessary radio resource usage.

To address this issue, the paging message can include a paging ID, which may consist of a combination of the service identifier provided by the CN and the reader identifier. Upon receiving a paging message, the device can determine whether the request is for the same service by checking the service identifier and reader identifier. If the service ID is the same but the reader ID is different, the device can recognize it as a duplicate paging and avoid redundant access procedures. If the device receives a paging message with the same paging ID as one previously received, it can infer that the previous access was unsuccessful and perform a re-access procedure.

However, implementing this process requires the device to store previous states, increasing device complexity. For battery-limited devices, making this a mandatory feature may not be practical. Therefore, even if a paging ID is introduced, it would be preferable to use it as an optional feature.

Proposal 3. The paging ID, consisting of a service ID and a reader ID, can be used as an optional feature for multiple or subsequent paging by the same or different readers.

RA type

The device can implicitly determine whether the access is Contention-Based Random Access (CBRA) or Contention-Free Access (CFA) through inference. This implicit determination is based on the mapping between the device ID and the allocated D2R resource configuration in the paging message.

For example, If the D2R radio resource is configured with an identifier referring to a single device and an access occasion(s) assigned to it, the device can infer that the access is CFA-based. Conversely, if the D2R radio resource is configured with a group device ID referring to multiple devices and one or more access occasions, the device can infer that the access is CBRA-based.

Therefore, since the access type between CFA and CBRA can be inferred from the mapping of the device ID and allocated D2R resources, it does not need to be explicitly included in the paging message.

Proposal 4. Since the access type between CFA and CBRA can be inferred from the mapping of the device ID and allocated D2R resources, it does not need to be explicitly included in the paging message.

In contention-based random access, the determination of a 2-step or 3-step CBRA may be based on the allocated resource size. For 3-step CBRA, since only a 16-bit random number is transmitted, a predefined fixed-size resource allocation can be considered. However, because resource allocation may vary depending on physical layer transmission parameters, defining a fixed size is challenging. Therefore, for CBRA, the paging message can include RA type information to explicitly indicate whether the procedure follows a 2-step or 3-step approach.

Proposal 5. For CBRA, the paging message can include RA type information to explicitly indicate whether the procedure follows a 2-step or 3-step approach.

Structure of Paging message

Since A-IoT paging can be received by all devices within the signal coverage area, a single A-IoT frame may need to support paging for multiple devices. Therefore, the paging message structure should be carefully designed to meet this requirement. . The design approach depends on the number of paging entries included in a single paging message.

If a paging message contains only one paging entry, devices should receive multiple paging messages and check each one to determine whether it is intended for them. In contrast, if a paging message includes multiple paging entries, each device can extract its corresponding paging entry from a single message and proceed with the paging procedure accordingly. Therefore, a device’s behavior in receiving and decoding paging messages may vary depending on the structure of the paging message.

Figure 3 illustrates example paging message structures based on these approaches, where the paging message follows either a single-entry or multiple-entry structure.

Proposal 6. RAN2 is kindly asked to discuss the structure of the paging message based on the number of paging entries included.

Figure 3. Candidate structures of paging message

Conclusions

Based on the discussion in the contribution, we have the following proposals:

Proposal 1. RAN2 is kindly asked to discuss whether the device ID received from the CN should be used as-is in the paging message or if an alternative ID format should be used.

Proposal 2. As D2R resource configuration, following information is required in paging message.

- For CFA: information indicating the starting of access occasion and the number of access occasion(s).

- For CBRA: information indicating the starting of access occasion, the number of access occasion(s), and information for random AO selection.

Proposal 3. The paging ID, consisting of a service ID and a reader ID, can be used as an optional feature for multiple or subsequent paging by the same or different readers.

Proposal 4. Since the access type between CFA and CBRA can be inferred from the mapping of the device ID and allocated D2R resources, it does not need to be explicitly included in the paging message.

Proposal 5. For CBRA, the paging message can include RA type information to explicitly indicate whether the procedure follows a 2-step or 3-step approach.

Proposal 6. RAN2 is kindly asked to discuss the structure of the paging message based on the number of paging entries included.

References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #128”, Nov. 2024

- 3GPP TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, Sep., 2024.

- 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things, Sep., 2024.