3GPP TSG-RAN WG2 #129	R2-2501065

Athens, Greece, Feb 17th– 21st, 2025

Agenda Item:	8.5.4

Source:	Ericsson

Title:	Adaptation of common signal/channel transmissions for NES

Document for:	Discussion

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised in [4]. In this paper, we address the third objective given in the revised WID as:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

For reference regarding the progress of the discussion in RAN1 on the third objective, please see the list of RAN1 agreements in the Appendix.

# 2	Discussion

In this section, we consider the following aspects:

- adaptation of SSB signals,

- adaptation of PRACH occasions and related signaling, and

- adaptation of paging.

### 2.1 Adaptation of SSB signals

2.1.1 Adaptation of SSB signals for UEs in RRC connected mode

Concerning the UEs in RRC connected mode, we observe that the UEs supporting the Rel-19 SSB adaptation feature specified based on Objective 3, can be informed dynamically about the change in the periodicity of SSB transmissions. Always-on SSBs can, for example, be provided sparsely as a baseline, and upon need, such as prior to/during handover, positioning services (E-CID) or alike, on-demand SSBs can be provided additionally.

Support on-demand SSB transmissions that can be configured dynamically when needed, e.g., prior to/during handover, positioning services etc., on top of always-on SSB transmissions in RRC connected mode.

Dynamic SSB adaptations on PCell for UEs in RRC Connected mode are beneficial in multiple scenarios such as the case of fast moving UEs, during handover, positioning, etc. Similar to what was agreed for Objective 1 for provisioning of OD-SSB in SCells via MAC-CE signalling, MAC-CE signalling can be used for PCell to adapt the SSB burst periodicity for UEs in RRC connected mode. The existing value range (i.e., between 5ms and 160 ms) is sufficient for such adaptation. The 160ms periodicity ensures that a UE can operate in the network and gives the network the opportunity to enter deep sleep state in between SSB transmissions. SSBs with 160ms could then be the baseline configuration. When needed, on-demand SSBs can be provided on top of always-on SSBs, which may be broadcasted as often as every 5ms.

For connected mode, support dynamic adaptation of SSB transmission in time domain via MAC CE, i.e., similar signalling to OD-SSB in SCells.

Furthermore, it shall be noted that one important scenario relevant for SSB-adaptations on PCell is for mobility, where on-demand SSBs from neighbour cells are transmitted when the UE approaches the cell border, or positioning, where on-demand SSBs from neighbour cells improve the measurement accuracy and reduce the measurement delay. When it comes to neighbour cells, a gNB can configure the neighbouring active (present) SSBs via ssb-ToMeasure and the associated rate/periodicity via the SSB Measurement Timing Configuration (SMTC), which defines the time window during which the UE can measure SSBs that belong to these neighbouring cells. SMTC of neighbour cells needs to cover the on-demand SSBs for a UE to benefit from SSB adaptation in neighbour cells. That is, in addition to the two SSB configurations and the suggested signalling, SMTC configuration for a UE needs to be dynamically adapted accordingly.

Dynamic adaptation of SSB transmissions in neighbour cells needs to be accompanied by corresponding dynamic adaptation of SMTC.

Introduce mechanism for dynamic SMTC adaptation.

### 2.1.2 Adaptation of SSB signals for UEs in RRC idle/inactive mode

Concerning the UEs in RRC idle/inactive mode, we observe that having a longer SSB periodicity e.g., > 20ms as a baseline, and activating on-demand SSBs upon need can also be considered. Note that for UEs in RRC idle/inactive mode, it is specified that a non-initial-access cell may have SSB periodicity of 160ms:

SSB-MTC2-LP-r16 ::= SEQUENCE {

pci-List SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId OPTIONAL, -- Need R

periodicity ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

For example, as illustrated in Figure 2, if a capacity cell is overlapped by a coverage cell, the SSB transmission rate of the capacity cell may be every 160ms while the UE will be able to find the overlapping coverage cell for initial access as it is providing SSBs every 20ms. Then through neighbor cell information (SSB-MTC2-LP) provided by the coverage cell the UE may reselect to the capacity cell.

Figure 2: Deployment according to legacy specifications in which a coverage cell transmits SSBs every 20ms, while the overlapped capacity cell (non-initial access) transmits SSBs every 160ms.

The periodicity of SSB can be 160ms for non-initial access cells.

However, in practice such capacity cells with relatively less frequent SSB transmissions are not deployed. The main reason is that there may be UEs, for example UEs in poor coverage, in need of multiple SSBs (up to 3 SSBs) before such UEs can decode the paging message. Similarly, there may be a need for multiple SSB transmissions prior to random-access occasions.

The example that illustrates the problem of a UE that is in poor coverage and in need of multiple SSBs prior to POs is illustrated in Figure 3, In this example, the UE needs to wake up more than 320ms ahead of the PO for processing the required number of SSBs. Note that PEI would not have helped in this scenario as the PEI occasion would still be 3 SSBs ahead and upon paging the UE would not be able to go back to deep sleep until the PO.

Figure 3: Timeline of RRC Idle/Inactive UE in need of multiple SSBs before the PO.

Capacity cells do not typically provide SSB transmissions with long periodicity, e.g., 160ms, since some UEs may need multiple SSB transmissions prior to PO/PEI decoding and/or random-access occasions.

Instead, as illustrated in Figure 4, on-demand SSBs (in addition to always-on SSBs) can be transmitted prior to the POs for Rel-19 UEs in RRC Idle/Inactive. Considering that this would be backwards compatible as legacy UEs can rely on the always-on SSB transmissions, it would become practical to deploy such cells that are configured with less frequent SSB transmissions. Hence, we think that adaptations to SSBs in capacity cells should be supported for UEs in RRC Idle/inactive modes.

Figure 4: Timeline of RRC Idle/Inactive UE in need of multiple SSBs before the PO if SSB adaptations are adapted. The UE may choose to wake up closer to PO when on-demand SSBs are provided.

In RRC idle/inactive mode, transmitting on-demand SSBs prior to POs and random-access occasions makes it possible to deploy capacity cells with low periodicity, e.g., with respect to 5ms SSB transmissions.

Transmitting on-demand SSBs prior to POs and random-access occasions can be beneficial for UEs since there would not be a need to wake up as far in advance before random-access and paging occasions, i.e., the UE can sleep longer.

For idle/inactive mode, SSBs can be transmitted on-demand (dynamically or semi-statistically), e.g., on-demand SSBs are transmitted prior to POs and random-access occasions, in addition to always-on SSB transmissions.

### 2.2 Adaptation of PRACH occasions and related signaling

In Rel-19, a UE in RRC connected mode can be informed dynamically about the change in the periodicity of PRACH occasions. It is also possible for a UE in RRC idle/inactive mode. Additional signalling can be introduced to configure PRACH occasions dynamically using, for example, DCI to activate those PRACH occasions per need basis. This would be backwards compatible as legacy UEs would continue using the legacy PRACH configuration provided via system information broadcast.

Support activation of additional PRACH occasions dynamically for UEs in connected and idle/inactive modes.

Dynamic activation of additional PRACH resources can be achieved without requiring UEs to wake-up. For idle/inactive UEs, activation can be indicated during paging occasions (POs), e.g., in paging DCI, and/or in SIB1 DCI. Providing such indication at POs and/or in the message that schedules SIB1 would be beneficial for a UE assuming it is likely a UE triggers network access after SIB1 acquisition or receiving a paging message.

For UEs in idle/inactive mode additional PRACH occasions are activated at paging and/or SIB1 transmission occasions.

UEs in connected mode monitor a paging occasion at least once per modification period, see from 38.331, section 5.2.2.2.2:

“UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.”

Concerning the DCI-based adaptation of additional PRACH resources, RAN1 adopted the following working assumption during RAN1#118bis meeting:

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

The motivation behind the above working assumption adopted by RAN1 is that if paging DCI supports an activation mechanism for additional PRACH resources for both idle/inactive and connected UEs, this simplifies dynamic control of random-access resources and improves efficiency by using the same signaling for both scenarios. Furthermore, it does not require additional wake-up for UEs as they anyway receive this DCI.

A unified signalling method for activating additional PRACH resources for both idle/inactive and connected UEs simplifies the dynamic control and improves efficiency by utilizing same signalling for both cases.

However, it may be beneficial to separate the indication for the activation of additional PRACH resources for UEs in idle/inactive mode and UEs in connected mode. For UEs in connected mode another random-access event that needs to be considered is the one triggered by the PDCCH order. The network may send the PDCCH order to a UE in connected mode when there is pending data in the DL but the UE is out-of-sync. It may be beneficial if the network activates additional random-access occasions so that the UE is in sync again quickly. In that case it would be better if additional random-access resources are activated for that particular UE with a dedicated indication. This may be achieved with an indication introduced in the dedicated PDCCH order signaling for that UE, e.g., the message scrambled with C-RNTI.

For UEs in connected mode, additional PRACH occasions are activated at SIB1 transmission occasions and/or when PDCCH order is transmitted.

Regarding the two types of random-access procedures that can be configured, i.e., CBRA and CFRA. While the former can be configured in all RRC states, i.e., in RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states, the latter can only be configured for UEs in RRC_CONNECTED state.

PRACH resource adaptation is supported for both CBRA and CFRA.

Another aspect that needs to be addressed is whether adaptation of PRACH resources is supported for 2-step and 4-step RA.

PRACH resource adaptation is supported for both 4-step and 2-step RA.

### 2.3 Adaptation of paging

The adaptation of paging was discussed during RAN2#125bis, RAN2#127, RAN2#127bis and RAN2#128, and the following agreements were made:

Agreements on paging adaptation (RAN2#125bis):

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

Agreements on paging adaptation (RAN2#127):

- Option-a) is about the bundling of PF for R19 NES UEs.

- R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.

Agreements on paging adaptation (RAN2#127bis):

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values).

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

Agreements on paging adaptation (RAN2#128):

- Introduce value for Ns=8. FFS on 16.

- Introduce value for N= T/32. FFS on T/64, T/128, T/256.

- RAN2 will revisit PEI configuration and PEI transmission aspects with the consideration of offline discussion in the next meeting.

The motivation for the existing framework of paging messages is to distribute the paging frames/occasions (PFs/POs) evenly across DRX cycles in idle/inactive modes. This is to balance the load not only from gNB perspective in a cell, but also from UE perspective to receive the paging message within a period intended based on the configured DRX cycle. From network energy saving perspective, such framework does not provide many opportunities for a gNB to sleep. It may become even more challenging to achieve energy savings when paging messages are broadcasted in all cells in a tracking area.

One of the aspects that needs to be addressed is how dynamic the paging adaptations should be. In our opinion, there is no need to introduce a dynamic adaptation mechanism due to its complexity. We think that semi-static configurations and updates via system information update notification, as in legacy, are good enough.

Paging adaptations are configured semi-statically and updated via system information update notification.

We discuss the remaining aspects concerning the signalling overhead associated with extended values of N and Ns parameters, and the aspects concerning the co-existence of PEI and Rel-19 paging adaptations.

### 2.3.1 Signalling overhead and extended values of N and Ns parameters

As highlighted in the list of agreements from RAN2#127bis meeting, option-b) is selected as baseline solution for supporting paging adaptations. According to the agreement from RAN2#125bis meeting, option-b) is about extending the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. We think that the values of N smaller than T/32 should not be supported for the following reasons.

First, we observe that decreasing the number of PFs by extending the set of possible values for N would not be significantly beneficial in terms of NW energy savings since it was agreed in RAN#127bis meeting that a legacy UE is not barred in cells where paging adaptations are configured. Therefore, a cell with such configuration would still need to wake up for the sake of legacy PFs/POs that are configured according to the legacy value range of parameter N.

Second, we observe that supporting values of N that are smaller than T/32 could lead to N < 1. This is because the final DRX cycle length value T for a UE in idle is determined as the minimum of the UE specific DRX value, if allocated by core network, and a default DRX value broadcasted in system information (c.f., TS 36.304).Therefore, since the minimum possible value for both UE specific and default DRX cycle length is 32, N can be smaller than 1 for N = T/X, if the value of denominator X is greater than 32. For example, if the core network configures the UE specific DRX cycle with T = 32 and the default DRX cycle is configured with T > 32 (e.g., 64), then N < 1 if the values smaller than T/32 are supported for N.

Introducing values smaller than T/32 for N does not significantly benefit network energy savings since the network would anyway need to wake up for legacy UEs based on the legacy configuration for N.

The DRX cycle length for a UE in idle is determined as the minimum of UE specific DRX value, if allocated by core network, and the default DRX value broadcasted in system information.

Introducing values smaller than T/32 for N could lead to N < 1 in cases where core network configures the UE specific DRX cycle with a T value smaller than the T value of the default DRX cycle (e.g., the UE specific DRX cycle is configured with T = 32 and the default DRX cycle is configured with T = 64).

Based on the observations above:

For N, values smaller than T/32 are not supported.

One of the questions raised during RAN2#127 and discussed further in the subsequent email discussion and RAN2#127bis concerns the RRC signalling overhead in the case of option-b). Concerning this issue, RAN2 agreed in RAN2#127bis that “R2 should aim at signalling overhead minimization” and there is an FFS on supporting Ns values larger than 8. Next, we discuss the means of minimizing the signalling overhead for option-b).

Regarding the maximum possible value for Ns for Rel-19 UEs, one needs to consider the paging capacity for different values of parameter Ns. To do so, let us recall that paging adaptations can provide network energy savings in scenarios where legacy PFs are sparsely configured. Here’s an assessment of the paging capacity for a typical scenario suitable for paging adaptations in which legacy PFs are configured as sparse as possible (i.e., N = T/16 according to the current specifications), SCS is set to 15 kHz, and Ns for legacy UEs is set to the highest possible value (i.e., Ns = 4) whereas Ns for Rel-19 UEs is set to 8, and the number of UEs that can be paged simultaneously is set to 32 (i.e., maxNrofPageRec INTEGER ::= 32 -- Maximum number of page records).

Then, we have the following for the number of UEs that can be paged per second:

- It is possible to page 6,25*(4 + 8) *32 UEs = 2400 UEs per second, where

  - 6,25 is the number of PFs per second (follows from the fact that for SCS = 15 kHz and N = T/16 there is a PF every 160ms).

Now, let us consider a legacy scenario with Ns set 4, SCS set 15 kHz, and considering that the number of UEs that can be paged simultaneously is set to 32. In this case, we have the following for the number of UEs that can be paged per second:

- For N = T/4, it is possible to page 25*4*32 UEs = 3200 UEs per second, where

  - 25 is the number of PFs per second (follows from the fact that for SCS = 15 kHz and N = T/4 there is a PF every 40ms).

- For N = T/8, it is possible to page 12,5*4*32 UEs = 1600 UEs per second, where

  - 12,5 is the number of PFs per second (follows from the fact that for SCS = 15 kHz and N = T/8 there is a PF every 80ms).

Based on the above calculations, we observe that the paging capacity that can be provided by setting the Ns for Rel-19 UEs to 8 is sufficient and comparable to the paging capacity that can be provided with legacy configurations. Therefore, we conclude that supporting higher values for parameter Ns for Rel-19 UEs would lead to unnecessary signalling overhead.

It is sufficient to extend the value range of Ns for Rel-19 UEs with 8 considering the paging capacity that can be achieved with respect to legacy configurations.

Furthermore, as we discuss in Section 2.3.2, we observe that increasing Ns values may negatively affect PEI due to the current maximum size of the PEI DCI_2_7, with the impact becoming more significant for higher values of Ns.

Increasing Ns values may negatively affect PEI due to the maximum size of PEI DCI_2_7, with the impact becoming more significant for higher values of Ns (e.g., values higher than 8).

Based on the above observations, we propose to limit the maximum possible value of Ns to 8.

The maximum possible value for Ns is 8.

Furthermore, to allow for configuration flexibility, we propose to also support 1, 2 and 4, similar to legacy, as possible values for parameter Ns for Rel-19 UEs.

The codepoints for Ns are 1, 2, 4 or 8.

Below, we evaluate the message size when Ns for Rel-19 UEs is set to 8 and compare the outcome with the message size of legacy configuration as baseline. In the following example, for the sake of evaluation, we assume a SIB1 configuration with hardly anything but the paging configuration in all scenarios.

- Legacy baseline scenario: T = 128, N = T/16, Ns = 4 (i.e., every 16th RF is a PF, and each PF is associated with 4 POs), and four instances of firstPDCCH-MonitoringOccasionOfPO parameter is configured for SCS = 15 kHz (i.e., corresponds to the case sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO). This scenario is illustrated in Figure 6.

Figure 6: Legacy baseline scenario (T = 128, N = T/16, Ns = 4, and SCS =15 kHz).

- Option b) scenario: T = 128, N = T/16, for legacy UEs Ns = 4 and for Rel-19 UEs Ns = 8 (i.e., every 16th RF is a PF, and each PF is associated with 4 POs for legacy UEs and 8 POs for Rel-19 UEs), SCS = 15 kHz (i.e., corresponds to the case sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO). This scenario is illustrated in Figure 7.

Figure 7:  Option b) scenario (T = 128, N = T/16, Ns for legacy = 4, Ns for Rel-19 UEs = 8 and SCS =15 kHz).

In Table 1, we provide the message sizes for the scenarios above. We assumed that the firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs are defined in the same way as in legacy (i.e., the range of firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs is given in symbols).

Table 1: Message sizes for different paging configurations

|  | Message size [octets] |
| --- | --- |
| Legacy (baseline) | 28 |
| Option b) with SCS = 15 kHz | 41 |
| Option b) with SCS = 480 kHz | 50 |

According to the results above in Table 1, option b) with SCS = 15 kHz and option b) with SCS = 480 kHz have 13 and 22 octets larger message size than the legacy (baseline) configuration, respectively. This accounts for approximately 46% and 79% increase in message size. One can observe that option b) introduces significant signalling overhead for the case where Ns for Rel-19 UEs is set to 8.

Option b) introduces significant signalling overhead (~46% larger message size than the respective legacy configuration with T = 128, N = T/16, Ns for legacy = 4, SCS = 15 kHz) even for the case where Ns for Rel-19 UEs is set to 8.

Based on the results in Table 1 above, we think that it would be beneficial to reduce the impact of option b) on the message size. This can be simply achieved by defining the range of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level instead of symbol level as currently defined for legacy UEs. Note that we are not proposing to change the meaning of firstPDCCH-MonitoringOccasionOfPO, but its resolution only (i.e., the unit of firstPDCCH-MonitoringOccasionOfPO parameter would still be monitoring occasions).  This would allow us to define the range of the firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs to be the same as of the firstPDCCH-MonitoringOccasionOfPO parameters for legacy UEs but with fewer bits. In the Appendix, we present the details of how this modification could be captured in the context of the ASN.1 signalling.

In Table 2, we summarize the evaluation for message sizes for two scenarios with option b) in which firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs are defined at the slot level.

Table 2: Message size for different paging configuration scenarios (slot level configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs).

|  | Message size [octets] |
| --- | --- |
| Option b) with SCS = 15 kHz (slot level configuration) | 37 |
| Option b) with SCS = 480 kHz (slot level configuration) | 46 |

Based on the results in Table 2 above, the configuration of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level results in 4 octets smaller message size than the configuration of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at symbol level, which corresponds to approximately 31% reduction of message size.

The configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level results in 4 octets (~31%) smaller message size than the configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at symbol level.

Define values of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level.

### 2.3.2 Paging adaptations and PEI

Given that the PEI is an important Rel-17 feature that enables the UE energy savings in idle and inactive modes, we believe that supporting the co-existence of PEI and paging adaptations would be beneficial.

Support co-existence of PEI and paging adaptations.

In order to understand the limitations associated with supporting the co-existence of PEI and paging adaptations, let us recall the following from the TS 38.331:

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...

}

PEI-Config field descriptions

payloadSizeDCI-2-7 Payload size of PEI DCI, i.e., DCI format 2_7. The size is no larger than the payload size of paging DCI which has maximum of 41 bits and 43 bits for licensed and unlicensed spectrums, respectively.

pei-FrameOffset Offset, in number of frames from the start of a reference frame for PEI-O to the start of a first paging frame of the paging frames associated with the PEI-O, see TS 38.213 [13], clause 10.4A.

po-NumPerPEI The number of PO(s) associated with one PEI monitoring occasion. It is a factor of the total PO number in a paging cycle, i.e N x Ns, as specified in TS 38.304 [20]. The maximum number of PF associated with one PEI monitoring occasion is 2. The number of PO mapping to one PEI should be multiple of Ns when po-NumPerPEI is larger than Ns.

maxPEI-perPF-r17                        INTEGER ::= 4       -- Maximum number of PEI occasion per paging frame

Furthermore, let us recall the following from Section 7.21 in the TS 38.304:

“If one PEI-O is associated with POs of two PFs, the two PFs are consecutive PFs calculated by the parameters PF_offset, T, Ns, and N.”

According to the current specifications, maximum number of paging occasions per PF is four, the maximum number of PFs associated with one PEI monitoring occasions is two, and if one PEI occasion is associated with POs of two PFs, the two PFs are consecutive PFs.

Current specifications need to be updated in order to support co-existence of PEI and paging adaptations, i.e. Ns = 8.

A straightforward solution for supporting co-existence of PEI and paging adaptations would be to increase the maximum number of paging occasions per PF to 8 (or 16, depending on the outcome of the related discussion in RAN2). However, this potential solution has an impact on RAN1 due to the current maximum size of the PEI DCI_2_7, with the impact that becomes significant due to higher values of Ns.

Supporting co-existence of PEI and paging adaptations depends on the constraints associated with the current maximum PEI DCI_2_7 size, which falls into RAN1 domain.

Wait for RAN1 input to conclude on how to support co-existence of PEI and paging adaptations.

# 3	Conclusion

In this contribution we discuss the objective on the adaptation of common signal/channel transmissions. Based on the discussion in the previous sections we made the following observations:

Observation 1	Dynamic adaptation of SSB transmissions in neighbour cells needs to be accompanied by corresponding dynamic adaptation of SMTC.

Observation 2	The periodicity of SSB can be 160ms for non-initial access cells.

Observation 3	Capacity cells do not typically provide SSB transmissions with long periodicity, e.g., 160ms, since some UEs may need multiple SSB transmissions prior to PO/PEI decoding and/or random-access occasions.

Observation 4	In RRC idle/inactive mode, transmitting on-demand SSBs prior to POs and random-access occasions makes it possible to deploy capacity cells with low periodicity, e.g., with respect to 5ms SSB transmissions.

Observation 5	Transmitting on-demand SSBs prior to POs and random-access occasions can be beneficial for UEs since there would not be a need to wake up as far in advance before random-access and paging occasions, i.e., the UE can sleep longer.

Observation 6	A unified signalling method for activating additional PRACH resources for both idle/inactive and connected UEs simplifies the dynamic control and improves efficiency by utilizing same signalling for both cases.

Observation 7	Introducing values smaller than T/32 for N does not significantly benefit network energy savings since the network would anyway need to wake up for legacy UEs based on the legacy configuration for N.

Observation 8	The DRX cycle length for a UE in idle is determined as the minimum of UE specific DRX value, if allocated by core network, and the default DRX value broadcasted in system information.

Observation 9	Introducing values smaller than T/32 for N could lead to N < 1 in cases where core network configures the UE specific DRX cycle with a T value smaller than the T value of the default DRX cycle (e.g., the UE specific DRX cycle is configured with T = 32 and the default DRX cycle is configured with T = 64).

Observation 10	It is sufficient to extend the value range of Ns for Rel-19 UEs with 8 considering the paging capacity that can be achieved with respect to legacy configurations.

Observation 11	Increasing Ns values may negatively affect PEI due to the maximum size of PEI DCI_2_7, with the impact becoming more significant for higher values of Ns (e.g., values higher than 8).

Observation 12	Option b) introduces significant signalling overhead (~46% larger message size than the respective legacy configuration with T = 128, N = T/16, Ns for legacy = 4, SCS = 15 kHz) even for the case where Ns for Rel-19 UEs is set to 8.

Observation 13	The configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level results in 4 octets (~31%) smaller message size than the configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at symbol level.

Observation 14	According to the current specifications, maximum number of paging occasions per PF is four, the maximum number of PFs associated with one PEI monitoring occasions is two, and if one PEI occasion is associated with POs of two PFs, the two PFs are consecutive PFs.

Observation 15	Current specifications need to be updated in order to support co-existence of PEI and paging adaptations, i.e. Ns = 8.

Observation 16	Supporting co-existence of PEI and paging adaptations depends on the constraints associated with the current maximum PEI DCI_2_7 size, which falls into RAN1 domain.

Based on the discussion in the previous section, we propose the following:

Proposal 1	Support on-demand SSB transmissions that can be configured dynamically when needed, e.g., prior to/during handover, positioning services etc., on top of always-on SSB transmissions in RRC connected mode.

Proposal 2	For connected mode, support dynamic adaptation of SSB transmission in time domain via MAC CE, i.e., similar signalling to OD-SSB in SCells.

Proposal 3	Introduce mechanism for dynamic SMTC adaptation.

Proposal 4	For idle/inactive mode, SSBs can be transmitted on-demand (dynamically or semi-statistically), e.g., on-demand SSBs are transmitted prior to POs and random-access occasions, in addition to always-on SSB transmissions.

Proposal 5	Support activation of additional PRACH occasions dynamically for UEs in connected and idle/inactive modes.

Proposal 6	For UEs in idle/inactive mode additional PRACH occasions are activated at paging and/or SIB1 transmission occasions.

Proposal 7	For UEs in connected mode, additional PRACH occasions are activated at SIB1 transmission occasions and/or when PDCCH order is transmitted.

Proposal 8	PRACH resource adaptation is supported for both CBRA and CFRA.

Proposal 9	PRACH resource adaptation is supported for both 4-step and 2-step RA.

Proposal 10	Paging adaptations are configured semi-statically and updated via system information update notification.

Proposal 11	For N, values smaller than T/32 are not supported.

Proposal 12	The maximum possible value for Ns is 8.

Proposal 13	The codepoints for Ns are 1, 2, 4 or 8.

Proposal 14	Define values of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level.

Proposal 15	Support co-existence of PEI and paging adaptations.

Proposal 16	Wait for RAN1 input to conclude on how to support co-existence of PEI and paging adaptations.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

3GPP TS 38.304 V18.1.0 User Equipment (UE) procedures in Idle mode and in RRC Inactive state

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN#105, October 2024.

# Appendix

### Examples of Rel-19 firstPDCCH-MonitoringOccasionOfPO configuration

Example 1 Symbol level configuration of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs.

Example 2 Slot level configuration of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs.