3GPP TSG-RAN WG2 Meeting #129	R2-2501072

Athens, Greece, Feb 17th – Feb 21st, 2025

Agenda item:	8.3.3

Source:	Qualcomm Incorporated

Title:	Simulation Results for Measurement Event Prediction

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 meeting (RAN2 #128 [1]), the simulation framework, assumptions, and parameters for Measurement Event Prediction for handover events were discussed. Among the agreements captured in the Chair notes [1], definitions were agreed for 4 indirect measurement event prediction cases:

- Indirect measurement event prediction for temporal domain case A: Future measurement result(s) is predicted by an RRM measurement prediction model at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at a future time occurs, without further involvement of an AI/ML model

- Indirect measurement event prediction for temporal domain case B: Future measurement result(s) is predicted by an RRM measurement prediction model at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at a future time occurs, without further involvement of an AI/ML model

- Indirect measurement event prediction for frequency domain: Measurement result(s) is predicted by an RRM measurement prediction model at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at a time instance occurs, without further involvement of an AI/ML model​

- Indirect measurement event prediction for spatial domain: Measurement result(s) is predicted by an RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model

In this contribution, we provide simulation evaluations and discussion for indirect measurement event prediction for temporal domain Case A, using an AI/ML RRM predictor to indirectly predict handover events for mobile UEs in 21-cell UMi simulations.

# Simulation assumptions

In this section, we discuss assumptions applicable to measurement event prediction data generation simulations, detailing the methodology for UE trajectory generation, soft line-of-sight (LoS) updates, and shadow fading updates.

## UE trajectory generation formulation

This section details the chosen model for generating random trajectories for mobile UEs in data generation simulations. The chosen model is based on Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 in [2] and a minor modification for extended mobility.

A UE has initial position (x0,y0, z0), constant forward velocity s, and initial azimuthal velocity angle ϕv,0. Temporal granularity tgran is defined as the smallest unit of time considered in random trajectory generation and mean temporal step size tstep is defined as the mean travel time for walk steps along the random trajectory. tstep is chosen such that it will be a whole-number multiple of the temporal granularity:

tstep=Nsubstepstgran, where

- Nsubsteps∈Z+ is the mean number of substeps per walk step.

- Maximum walk time tmax≫tstep is likewise chose to be a whole-number multiple of tgran.

- Distance granularity dgran is computed as dgran=stgran.

For the purposes of random trajectory generation for urban macro and urban micro simulations, a geometric sector SNb is defined as a geometric region consisting of the intersection of two regions: the first region consists of one third of the hexagonal cell surrounding a gNB and symmetric with respect to the gNB’s azimuthal orientation ϕNb, and the second region consists of the area outside a circle with radius dNb centered on the gNB. Figure 1 provides an example of a geometric sector.

Figure 1 Geometric sector defined for UE mobility constraints

The overall valid UE trajectory region Svalid is the geometric region in which a UE trajectory may be generated and consists of the union of all geometric sectors corresponding to each gNB in the simulation:

Svalid=i=1NNbSi

Where NNb is the number of gNBs in the simulation. Figure 2 visualizes Svalid for a 2-tier UMa simulation with ISD 200.

Figure 2 Geometric sectors for every gNB in a 2-tier Urban Macro simulation. Each geometric sector is labelled with the MAC ID of its corresponding gNB. The valid UE trajectory region Svalid consists of the union of all sectors.

Given the above setup, the following algorithm is used to generate a random trajectory for a particular UE:

- Initialize total sub-step counter m=0.

- For walk step k=1,2,…:

  - Draw number of sub-steps for the kth walk step Nsubsteps,k from a geometric distribution with probability of success 1Nsubsteps.

  - Draw azimuthal velocity angle delta for the kth walk step Δϕv,k from a uniform distribution in the range [-45°, 45°].

  - Compute sub-step velocity angle delta: Δϕv,k,substep=Δϕv,kNsubsteps,k

  - For each sub-step n=1, …,Nsubsteps,k:

    - Increment total sub-step counter m=m+1. If the UE’s total travel time thus far mtgran is greater than tmax, terminate the walk. Otherwise, continue.

    - Update the UE’s sub-step velocity angle: ϕv,m=ϕv,m-1+Δϕv,k,substep.

    - Compute potential updated UE position: (xm,ym,zm)=xm-1+dgrancosϕv,m,ym-1+dgransinϕv,m,z0.

If xm,ym,zm  is outside the valid UE trajectory region Svalid, terminate the walk. Otherwise, update the UE’s position.

Figure 3 shows an example set of UE trajectories generating using the above algorithm.

Figure 3 Example random trajectories for 20 UEs within a 2-Tier UMa model

## Channel modelling: soft LoS

This section details the approach used to update line-of-sight (LoS) states for mobile UEs in data generation simulations. Discussion of the motivation for adopting this soft LoS model has been provided in [3].

In the soft LoS model provided in TR 38.901 [4], Section 7.6.3.3, each gNB-UE link maintains a soft LoS state LOSsoft∈[0, 1], calculated as

LOSsoftG,d=12+1πarctan20λG+Fd, where

- G is a spatially consistent Gaussian R.V. with correlation distance dcor according to 38.901 Table 7.6.3.1-2,

- λ is the wavelength of the center frequency fc (in meters), and

- Fd=2erf-12PrLOSd-1.

To apply this soft LoS state, for each link, both LoS and NLoS channel coefficients HLOS and HNLOS are generated per the 38.901 Fast Fading Model (Section 7.5, Steps 2-12), and the overall channel is calculated per 38.901 Equation 7.6-18 as

HLOSsoft=HLOSLOSsoft+HNLOS1-LOSsoft2.

Although TR 38.901 provides correlation distances for LoS random variables, a specific correlation function is not provided. To correlate the Gaussian random variables G used in soft LoS state generation in Option B, we use exponential correlation:

RLOSΔd=exp-Δddcor, where

- Δd is the distance between any two points in the xy-plane, and

- dcor is the correlation distance provided in Table 7.6.3.1-2.

As discussed in [3], this exponential correlation function is chosen to be consistent with the exponential autocorrelation function provided for shadow fading in 38.901, Section 7.4.4, as well as with the spatial correlation procedure provided in WINNER II [5], Section 3.3.1. We also note that the distribution statistics of the Gaussian random variables G are not specified in TR 38.901; for simplicity, we choose G to be standard normal Gaussians.

We assume that exponential correlation is applied on a per-gNB basis; that is, the Gaussians used to generate the soft LoS states for all UE-gNB links associated with a particular gNB are correlated per RLOSΔd, and there is no inter-gNB LoS correlation. We also assume that the generated LoS states for given xy-locations are fixed in time, meaning UEs will only experience LoS state transitions as a result of mobility. With these assumptions, the following procedure is used to calculate soft LoS states for mobile UEs:

- For each gNB in the simulation n=1,2,…,NNB:

  - Establish a grid of location points Dn that covers the area in which mobile UE trajectories will be constrained.

  - Over Dn generate a grid Gn of standard normal Gaussian realizations at each grid point correlated in distance by RLOSΔd=exp-Δddcor.

  - Using the soft LoS state procedure outlined in TR 38.901, Section 7.6.3.3, generate a grid of soft LoS realizations Ln= LOSsoft(Gn, Dn) from Gn and PrLOSd for the given scenario.

- During the channel update for each mobile UE i=1,2,…,NUE:

  - For each gNB in the simulation n=1,2,…,NNB, interpolate the soft LoS state for the UE-gNB link from Ln given the mobile UE’s xy-location.

Figure 4 shows an example soft LoS grid for an FR2 UMi simulation with the gNB located at (0, 0).

Figure 4 Example Soft LoS grid (UMi, fc = 30 GHz)

To demonstrate the impact of soft LoS on the generated channels for mobile UEs, Figure 5 provides a comparison between simulation results for a mobile UE with and without soft LoS updates enabled. In the simulation with LoS updates disabled (orange line), the mobile UE has a constant LoS = 1 state with its serving gNB, while in the simulation with LoS updates enabled (blue line), the LoS state is updated as a function of the UE’s position. As the mobile UE enters NLoS regions, the maximum RSRP across all beam pair links in the simulation with LoS updates tends to drop significantly compared to the maximum RSRP for the simulation where the LoS state remains 1.

Figure 5 Simulation comparison for a mobile UE with and without soft LoS updates enabled. Top: maximum RSRP for mobile UE with and without soft LoS updates. Bottom: LoS state for mobile UE with and without LoS updates. A value of 0 corresponds to a completely NLoS link, while a value of 1 corresponds to a completely LoS link.

## Channel modelling: shadow fading

This section describes the model used for updating shadow fading values for mobile UEs in data generation simulations. Similarly to soft LoS state updates, we use a grid-based approach to update shadow fading values in a spatially consistent manner. Discussion of the motivation for adopting this approach has been provided in [3].

As with soft LoS values, we assume that shadow fading values are fixed in time for given xy-locations and independently calculated on a per-gNB basis. We also assume that a gNB’s shadow fading values are autocorrelated exponentially as per TR 38.901 [4], Section 7.4.4, with autocorrelation function

RΔx=exp-Δxdcor, where

- Δx is the 2D-distance between any two locations in the simulation, and

- dcor is the environment-specific correlation distance provided in TR 38.901, Table 7.5-6.

With these assumptions, the following procedure is used to update shadow fading values for mobile UEs:

- For each gNB in the simulation n=1,2,…,NNB:

  - Establish a grid of location points Dn that covers the area in which mobile UE trajectories will be constrained.

  - On top of Dn generate a grid of correlated shadow fading values SFn~N0, σsf, where σsf is the scenario/LoS-state-specific shadow fading standard deviation provided in TR 38.901, Table 7.5-6, and the R.V.s at each grid point are correlated in distance by RLOSΔd=exp-Δddcor .

- During the channel update for each mobile UE i=1,2,…,NUE:

  - For each gNB in the simulation n=1,2,…,NNB, interpolate the LoS and NLoS shadow fading values for the UE-gNB link from SFn given the mobile UE’s xy-location and UE-gNB link LoS.

Figure 6 shows an example pair of LoS and NLoS shadow fading grids for a UMi simulation.

Figure 6 Example LoS (left) and NLoS (right) shadow fading grids, UMi

## Channel modelling: disabling wraparound

To facilitate modelling of inter-site UE mobility, we disable wraparound for UMa and UMi models. Figure 7 demonstrates the impact of disabling wraparound on the channel modelling parameters on the arrival and departure LoS cluster angles between UEs in the bottom sector and gNBs in the upper-left sector of a 2-tier model. Disabling wraparound ensures that the channels between mobile UEs and distant gNBs do not experience instantaneous changes as the UEs draw nearer.

Figure 7 Effect of disabling wraparound on LoS cluster angles between UEs and gNBs in separate sectors. Left: when wraparound is enabled, the UE LoS arrival angles (blue arrows) point toward a “wrapped” version of gNB 40 (not shown). Right: when wraparound is disabled, the UE LoS arrival angles point toward the non-wrapped version of gNB 40.

## Derivation of cell-level metrics

For a given UE-gNB link, given measured RSRPs across all Tx-Rx beam pairs and polarizations, the cell-level metric for RRM prediction is derived from Tx-Rx beam pair RSRPs via the following procedure:

- RSRPs are summed across Rx polarizations, and the maximum RSRP is taken across Tx polarizations.

- For k gNBs, the cell level metric Mn is calculated as

Mn=m1,n,m2,n,…,mk,n, where

- mi,n=1Nbj=1Nbri,j,n,

- ri,j,n is the RSRP of the jth top SSB for the ith sector at measurement instance n.

By taking the maximum RSRP across Rx beams, we assume a genie Rx beam selection algorithm that provides an upper-bound approximation for a realizable Rx beam selection algorithm.

The number of top beams averaged for this metric Nb is chosen to be Nb=3 for FR2 simulations and Nb=1 for FR1 simulations.

The resulting metrics are filtered in time via the Layer 3 filtering procedure outlined in 3GPP TS-38.331 [6], Section 5.5.3.2:

F0=M0

Fn=1-aFn-1+aMn, where

- Fn is the filtered cell metric result at measurement instance n,

- a=12k4, where k is the filter coefficient for the corresponding measurement quantity.

For the simulations discussed in this document, a filtering coefficient of k=4 is used to filter cell-level metrics. Figure 8 shows the effect of L3 filtering on calculated cell-level metrics for a given UE trajectory and 3 co-located gNBs.

Figure 8 Cell-level metrics with and without L3 filtering, k=4

# Simulation evaluation for measurement event prediction for temporal domain Case A

In this section, we discuss the details of the simulation assumptions, parameters, and methodology for FR2 intra-frequency, temporal domain Case A model generalization, and discuss the simulation results.

## Trajectory generation

For FR2 intra-frequency, temporal domain Case A model generalization dataset generation, we use the trajectory generation formulation outlined in Section 2.1 of this document. Table 1 lists the parameters used in this case.

Table 1: UE trajectory generation parameters for FR2 measurement event prediction

| Parameter | Value |
| --- | --- |
| tstep | 1 s |
| tgran | 0.01 s |
| tmax | 160 s |
| s | 60 km/h |
| dNb | 10 m |
| ISD | 200 m |

For this dataset, we consider inter-cell mobility, meaning UE trajectories span all 21 geometric sectors corresponding to each gNB in the 2-tier model. Figure 9 visualizes several example random trajectories for this inter-site mobility case.

Figure 9 Example random trajectories, FR2 measurement event prediction

## Soft LoS and shadow fading grids

When modeling inter-site mobility, for each set of 3 co-located gNBs a unique soft LoS grid is used to model line-of-sight states for mobile UEs, totaling 7 grids for the entire dataset. Similarly, 7 pairs of LoS/NLoS shadow fading grids are used to model the shadow fading states between the mobile UEs and each set of co-located gNBs. Figure 10 shows the soft LoS grids used for this dataset corresponding to the sets of co-located gNBs at (0, 0) and (0, 200), and Figure 11 shows the corresponding shadow fading grids.

Figure 10 soft LoS grids for FR2 measurement event prediction. Left: grid for gNBs located at (0, 0) m. Right: grid for gNBs located at (0, 200) m.

Figure 11 Shadow fading grids for FR2 measurement event prediction studies. Top: LoS (left) and NLoS (right) shadow fading grids for gNBs located at (0, 0) m. Bottom: LoS (left) and NLoS (right) shadow fading grids for gNBs located at (0, 200) m.

## Summary of simulation assumptions

Table 2 summarizes FR2 measurement event prediction simulation assumptions and parameters.

Table 2: Simulation assumptions for FR2 measurement event prediction

| Parameter | Value |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 200m ISD, 7 sites, 3 sectors/cells per site |
| Channel model | UMi with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Fast fading is considered for all the RRM sub-cases. Do not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8), and blockage (7.6.4), for channel modelling (38.901). Soft LoS and Shadow Fading aspects of Channel modelling are discussed in Sections 2.1.2 and 2.1.3 in this document. |
| System BW | 80MHz |
| UE Speed | 60 km/h |
| UE distribution | 100% outdoor. UEs are randomly dropped within a cell with uniform distribution. |
| BS Antenna Configuration | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ. Number of BS beams: 8 SSB beams per sector Beam pattern: 8 azi. beams with 22.5-deg. separation |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6. |
| UE Antenna Configuration | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Number of UE beams: 4 Rx per UE panel Beam pattern: 4 beams with 45-deg. separation |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8. |
| BS Tx Power | 40dBm |
| Maximum UE Tx Power | 23dBm |
| BS receiver Noise Figure | 7dB |
| UE receiver Noise Figure | 10dB |
| Inter site distance | 200m |
| BS Antenna height | 10m |
| UE Antenna height | 1.5m |
| Spatial consistency | Procedure A in TR38.901 |
| UE trajectory model | Based on Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility. Refer to section 2.1.1 for a detailed discussion of the trajectory model. |
| L1 sampling period | 20ms |
| Prediction window | 400 ms |

## Simulation dataset evaluation

Table 3 provides statistics for the trajectories within the FR2 measurement event prediction dataset; Figure 12 shows a CDF of the trajectory lengths.

Table 3: FR2 measurement event prediction dataset statistics

| Parameter | Dataset value |
| --- | --- |
| # of trajectories (total) | 4051 |
| Total data time (hr.) | 17.41 |
| Training/test split | 3240/811 |
| Mean trajectory length (s) | 15.47 |
| Median trajectory length (s) | 11.72 |
| Min. trajectory length (s) | 0.02 |
| Max. trajectory length (s) | 113.5 |

Figure 12 Empirical CDFs of FR2 measurement event prediction dataset trajectory lengths

To evaluate the generated dataset for FR2 measurement event prediction, in Figure 13 we plot the cell level metrics for each sector as a function of UE position across the dataset for two example gNBs (22 and 25). These plots demonstrate the impacts of UE-gNB distance, gNB panel azimuthal pointing angle, and soft LoS state on the cell-level metric values.

Figure 13 Cell-level metrics for each cell as functions of UE position for FR2 measurement event prediction dataset. Left: cell metric data for gNB 22. Right: cell metric data for gNB 25.

## AI/ML model and methodology

In the indirect measurement event prediction problem for temporal domain case A, an RRM measurement prediction model is used to produce predictions for future measurement results across the cells for which measurement events are considered. Given a set of past RRM measurements collected across all cells within a sliding observation window (OW), the measurement predictor produces predicted RRM values across all cells at a set of future measurement instances within a sliding prediction window (PW). Figure 14 illustrates these sliding observation/prediction windows. The predicted measurement results are then used to predict whether a measurement event occurs at a future time.

Figure 14 sliding observation/prediction windows in temporal domain case A prediction

We consider prediction of handover events in which the handover procedure outlined in [7] is used to initiate handover based on measured RRM metrics. This procedure depends on two parameters, a3-offset (specified in dB) and time-to-trigger (TTT) (specified in ms). A handover event occurs if the measured RRM metric of the serving cell falls below the measured RRM metric of a different cell by at least the a3-offset value for a duration of TTT ms.

For the purposes of describing handover event prediction, we establish the following nomenclature:

- Potential handover event: any event in which the measured RRM metric of the serving cell falls below the metric of some other cell by at least the a3-offset value

- Handover event: any potential handover event in which the RRM metric of serving cell remains below the metric of another serving cell by at least the a3-offset value for a duration of TTT ms (handover event occurs)

- “Null” handover event: any potential handover event in which the RRM metric of a serving cell does not remain below the metric of another serving cell by at least the a3-offset value for a duration of TTT ms (no handover event occurs)

Figure 15 illustrates a potential handover event between two cells and its possible outcomes of either a handover event or a null handover event.

Figure 15 Illustration of a potential handover event (top) and its possible outcomes (bottom): handover event (bottom left), and “null” handover event (bottom right)

We now outline a procedure for the use of AI/ML prediction at potential handover event occurrences within a temporal prediction Case A framework. Consider a scenario with Ncell cells. Let Δa3,ΔTTT be the specified a3-offset (in dB) and time-to-trigger (in ms), respectively. For a given mobile UE, assume RRM measurements are taken at a periodicity of Δmeas ms across all cells. Furthermore, assume we have an RRM predictor which, given measured RRM over the last Nmeas instances, produces predictions for the next Npred measurement instances, where Npred is chosen s.t. NpredΔmeas>ΔTTT, so that the predictor preduces predictions into the future for at least the duration of TTT. Finally, we specify maximum time delta ΔETD∈0, ΔTTT used in evaluating predicted handover events. Given the above setup, we specify the following procedure for handover prediction:

- For cell metric measurement instances k=0, 1, …:

  - Let sk∈1, Ncells denote the current serving cell of the mobile UE at time tk, and let ri,k1≤i≤Ncells denote the set of RRM measurements across all cells at measurement instance k.

  - If ∃i≠sk s.t. ri,k>rs,k+Δa3, a potential handover event has occurred. If so:

    - Using the RRM predictor, produce a set of predictions ri,j1≤i≤Ncells,j≤k+1≤k+Npred+1 across all cells for the next Npred measurement instances into the future.

    - If ∀j s.t. tj∈tk+1, tk+ΔTTT-ΔETD, ∃i≠sk s.t. ri,j>rs,k+Δa3, we consider that a handover has been predicted. Otherwise, we consider that a null handover event has been predicted.

Figure 16 shows examples of this procedure for a predicted handover event and a predicted null handover event.

Figure 16 handover event prediction examples. Left: predicted handover event. Right: predicted null handover event.

To analyse the accuracy of this method, at each potential handover event in our test dataset we compare the outcome of the handover prediction procedure to the ground-truth handover event outcome. Note that in computing these outcomes we do not use the outcome of the predicted handover event to determine the new serving cell; the legacy handover procedure outcome is always used. As per the agreements in [1], we define the following prediction accuracy outcomes:

- True event detection: a handover event occurs, and the outcome of the prediction procedure is a predicted handover event

- False event detection: a null handover event occurs, but the outcome of the prediction procedure is a predicted handover event

- Missed event detection: a handover event occurs, but the outcome of the prediction procedure is a predicted null handover event

Then, letting n1 be the number of false event detections, n2 be the number of missed event detections, and n3 be the number of true event detections, we analyse the accuracy of the prediction method via the following metrics:

- Precision: P=n3n1+n3 if n1+n3>0 else 1

- Recall: R=n3n2+n3 if n2+n3>0 else 1

- F1 score: F1=2PRP+R

Our AI/ML RRM predictors are encoder-only transformer models; their performance is compared to baseline sample-and-hold predictors, which use the most recent measured RRM metric values as predictions for all future time instances within the PW. (We note that in this prediction setting, sample-and-hold predictors will always predict a handover event.) We compare for a3-offset values of 2 dB and 3 dB, and for TTT values of 160 ms and 320 ms.

## Simulation Results

The following table summarizes simulation results for FR2 inter-cell, intra-frequency, temporal Case A indirect measurement event prediction.

Table 4: Simulation results for FR2 measurement event prediction

| Report parameters |  |  |  |  |  | Qualcomm |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option |  |  |  |  | Option 2 in TR 38.343 [2], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility |  |  |  |
|  | UE trajectory boundary processing option |  |  |  |  | Trajectory termination |  |  |  |
|  | UE speed (30,60,90,120 Km/h) |  |  |  |  | 60 km/h |  |  |  |
|  | Inter-frequency correlation assumption in general (yes or no) |  |  |  |  | N/A |  |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no) |  |  |  |  | N/A |  |  |  |
|  | Whether soft LoS is modeled or not |  |  |  |  | Yes |  |  |  |
|  | Spatial consistency option (A or B) |  |  |  |  | A |  |  |  |
|  | Number of TX beams |  |  |  |  | 8 SSB beams |  |  |  |
|  | Number of RX beams |  |  |  |  | 4 beams × 2 panels = 8 UE beams |  |  |  |
|  | Measurement reduction rate |  |  |  |  | N/A |  |  |  |
|  | Observation window |  |  |  |  | <=1280 ms |  |  |  |
|  | Prediction window (Note3) |  |  |  |  | 400 ms |  |  |  |
|  | Any other parameters |  |  |  |  | N/A |  |  |  |
| Data Size (Number of Samples) | Training/validity |  |  |  |  | 2592/648 |  |  |  |
|  | Testing |  |  |  |  | 811 |  |  |  |
| AI/ML model input/output | Model input |  |  |  |  | Up to 32 sets of past measured L3-filtered cell-level metric values |  |  |  |
|  | Model output |  |  |  |  | Prediction for future L3-filtered cell-level metric values |  |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) |  |  |  |  | Transformer |  |  |  |
|  | Model complexity in a number of parameters (K) |  |  |  |  | 1280K |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) |  |  |  |  | 10.24 Mb |  |  |  |
| Results | Handover parameters |  | Handover statistics |  |  | Predictor | Metric |  |  |
|  | A3-offset (dB) | TTT (ms) | # potential handover events | # true handover events | # null handover events |  | Precision (%) | Recall (%) | F1-score |
|  | 2 | 160 | 17107 | 7514 | 9593 | AI/ML | 46.64 | 89.22 | 0.6126 |
|  |  |  |  |  |  | Sample-and-hold | 43.92 | 100.00 | 0.6104 |
|  |  | 320 | 15799 | 5133 | 10666 | AI/ML | 34.95 | 89.67 | 0.5030 |
|  |  |  |  |  |  | Sample-and-hold | 32.49 | 100.00 | 0.4904 |
|  | 3 | 160 | 15240 | 6825 | 8595 | AI/ML | 44.99 | 97.60 | 0.6159 |
|  |  |  |  |  |  | Sample-and-hold | 44.26 | 100.00 | 0.6136 |
|  |  | 320 | 14559 | 4726 | 9833 | AI/ML | 33.23 | 98.20 | 0.4965 |
|  |  |  |  |  |  | Sample-and-hold | 32.46 | 100.00 | 0.4901 |

We note that both although AI/ML and sample-and-hold baseline predictors demonstrate poor precision, the precision of AI/ML slightly outperforms sample-and-hold, more strongly in the case when a3-offset = 2 dB. As sample-and-hold always predicts a handover event to occur, it will produce false positives for all null handover events, which for our dataset constitute the majority of potential handover events. The slight improvement in AI/ML precision demonstrates that the although the AI/ML approach still produces many false positive predictions, it will occasionally correctly predict null handover events. However, the increase in AI/ML precision results comes at the cost of a decrease in recall, demonstrating that the AI/ML approach will also occasionally incorrectly predict a null handover event when a true handover occurs. These precision and recall trends have a counteracting influence on the F1 scores for the AI/ML predictors, resulting in AI/ML predictors having similar F1 scores to the corresponding baseline predictors. We also observe that AI/ML predictors have better recall when a3-offset = 3 dB, demonstrating that for larger a3-offset values, the indirect AI/ML approach is less likely to predict a false negative.

Observation 1: For FR2 Case A AI/ML indirect measurement event prediction, in comparison to the sample-and-hold baseline, AI/ML predictors demonstrate a slight improvement in precision and a reduction in recall, resulting in similar F1 scores for the AI/ML approach and the baseline.

Observation 2:  For FR2 Case A AI/ML indirect measurement event prediction, AI/ML predictors have a higher recall rate for a 3-dB a3-offset compared to a 2-dB a3-offset, demonstrating that the probability of an AI/ML predictor is inversely proportional to the a3-offset value.

# Conclusions

Observation 1: For FR2 Case A AI/ML indirect measurement event prediction, in comparison to the sample-and-hold baseline, AI/ML predictors demonstrate a slight improvement in precision and a reduction in recall, resulting in similar F1 scores for the AI/ML approach and the baseline.

Observation 2:  For FR2 Case A AI/ML indirect measurement event prediction, AI/ML predictors have a higher recall rate for a 3-dB a3-offset compared to a 2-dB a3-offset, demonstrating that the probability of an AI/ML predictor is inversely proportional to the a3-offset value.

# References

[1] Chair notes, RAN2 #128 meeting.

[2] “3GPP TR 38.843 V18.0.0: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, 3GPP, January 2024, https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3983 (accessed May 7, 2024).

[3] “3GPP RAN2 2405205: Considerations for mobile UE trajectory generation and channel modelling for simulation evaluation”, 3GPP, May 2024, 3gpp.org.

[4] “3GPP TR 38.901 V17.0.0: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz (Release 17).”  3GPP RAN Plenary, January 2024, 3gpp.org.

[5] IST-WINNER II Deliverable 1.1.2 v.1.2, "WINNER II Channel Models", IST-WINNER2, Tech. Rep., 2007

[6] “3GPP TR 38.331 V18.2.0: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification (Release 18).” 3GPP RAN Plenary, June 2024, 3gpp.org.

[7] “3GPP TR 36.839 V11.1.0: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks (Release 11).” 3GPP RAN Plenary, January 2013, 3gpp.org