3GPP TSG-RAN WG2 Meeting #129	R2-2501104

Athens, Greece, Feb. 17th – 21st, 2025

Title: 	Discussion on MRO for LTM

Source: 	Huawei, HiSilicon

Agenda item:	8.10.2.1

Document for:	Discussion/Decision

- Introduction

In this paper, we mainly analyze possible RAN2 impacts for LTM.

- Discussion

RAN2#128 made the following agreements:

Agreements - No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues. - Reuse the existing ra-InformationCommon for the RA-based LTM failure. - If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF.

To facilitate the further discussion, we illustrate the steps during the LTM procedure to have a retro on the design of Rel-18 LTM feature, compared to those during legacy L3 handover.

Figure 1: Handover procedures for Legacy L3 HO and LTM

The division of failure cases for LTM is similar as those for handover, since the LTM is built and augmented on top of L3 handover. For handovers, the SON features including the radio link failure (RLF) report and successful handover report (SHR) provides the UE feedback information. The RLF and SHR could be augmented to also provide useful feedback information to address the LTM mobility issues. We will discuss the possible MRO enhancement for those SON reports.

Discussion on RLF report for LTM

The UE logs and reports whether and how the UE got the TA value

- The last RAN2 meeting had the conclusion that the network could know the access type implicitly. The implicit way is the presence of RA information. Per current specification, the UE could perform the UE-based TA measurement and perform PDCCH ordered TA acquisition simultaneously. How the UE chooses the access type upon reception of cell switch command is divided into the following cases:

- Case1: UE receives the TA value from cell switch command (CSC), regardless whether UE has valid measured TA or not.

-> RA-less access based on TA value from cell switch command

- Case2: UE does not receive any TA in CSC, and only got the UE measured TA

-> RA-less access based on UE measured TA value

- Case3: UE does not have neither the TA value from CSC, nor UE measured TA value

-> RA-based

Some companies thought that everything the network instructs the UE to perform could be stored in the UE context, e.g., whether the UE has been enabled with UE-based TA measurement and whether the UE has received the TA from CSC. However, the instructions could be made by DU. The UE context stored at the CU may or may not record the instructions made by the DU. What is actually included in UE context is highly dependent on the network implementation. At least the most essential information could be reported by the UE to make this functionality really work. Therefore, we propose that the UE logs whether the UE receives the TA from CSC.

Furthermore, with this indication the network could distinguish between the TA from the network and the TA from UE-based measurement to make proper adjustment of the LTM related parameters, e.g., whether to enable the UE-based TA measurement.

Proposal 1: It is proposed RAN2 to include in the RLF report whether the UE receives the TA from the cell switch command.

Early sync info in RLF report

Early sync is the key to achieving the goal of reducing the interruption time. If it is RACH-less, RAN2 discuss whether to include early sync info to existing RACH information to identify the issues of network-based TA procedure, e.g., how frequent and which is the proper triggering timing the source DU should trigger the PDCCH order to maintain the TA validity, and which of candidate cells should be involved to acquire the TA so as to have a target cell with a valid TA. The detailed early sync info is FFS.

Proposal 2: It is proposed RAN2 to discuss whether to include early sync info to existing RACH information to optimize the frequency, the timing and involved candidate cells in the early sync process. The detailed early sync info is FFS.

Beam Info

We notice that only the existence of L1 measurement is not sufficient. L1 measurement is the result of the UE movement at the time of failure occurrence.  The beam indicated in the CSC determines whether the RA-less access, i.e., the UL transmission, could succeed before T304 expires. It is suggested that UE logs the beam info in the cell switch command, for the network to know how the UE movement affects the beam selection of the target cell. Combining this beam info with the L1 measurement result, and the time gap since the CSC till the failure, the network could perform analysis to optimize the choice of target beam.

Proposal 3: It is proposed RAN2 to agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

Discussion on SHR report for LTM

stage-3 implementation

Some companies thought the RA information included in SHR report could be used as an implicit indication of RACH-less or RACH-based LTM switch. Different with RA information in RLF report in case of HOF, the RA information is not always included in the SHR report. According to TS 38.331, only when T304 trigger is fulfilled, the UE logs the RA information. Otherwise, the RA information is absent.

If we just use the implicit indication, it is only associated with the T304 trigger. In other words, for other triggers, the implicit indication does not work, and that is why we thinks an explicit indicator of RACH-less or RACH based LTM switch is needed to SHR report.

Proposal 4: It is proposed RAN2 to introduce a flag of whether it is RACH-less for SHR, since RA information is only logged due to T304 trigger.

- Conclusion

Based on the discussions above, we have the following observations and proposals:

Proposal 1: It is proposed RAN2 to include in the RLF report whether the UE receives the TA from the cell switch command.

Proposal 2: It is proposed RAN2 to discuss whether to include early sync info to existing RACH information to optimize the frequency, the timing and involved candidate cells in the early sync process. The detailed early sync info is FFS.

Proposal 3: It is proposed RAN2 to agree to enhance the RLF report with the beam information contained in the LTM cell switch command.

Proposal 4: It is proposed RAN2 to introduce a flag of whether it is RACH-less for SHR, since RA information is only logged due to T304 trigger.