3GPP TSG-RAN RAN2 #129					             R2-2501106

Athens, Greece, Feb. 17th - 21st, 2025

Title: 	Discussion on SON/MDT for NTN

Source: 	Huawei, HiSilicon

Agenda item:	8.10.4

Document for:	Discussion/Decision

1   Introduction

According to the RAN3 #126 meeting, the LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT [1] was sent to RAN2, among which the SON/MDT-for-NTN related issues are as follows:

- Regarding SON/MDT for NTN: On MRO for NTN, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs in case of “time and measurement based trigger condition” or “location and measurement based trigger condition”, it is up to RAN2 to decide to reuse the existing IE or not. On MDT for NTN, RAN3 assumes that the geographical area and/or the mapped cell ID can be used as the Area Scope for the logged MDT for NTN over NGAP. Further, RAN3 would like to confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for Area Scope of logged MDT over Uu for NTN, to help RAN3 decide whether to specify geographical area and/or mapped cell ID over NGAP.

In this paper, we aim to start the discussion on above two issues of SON/MDT for NTN based on RAN3’s LS.

2   Discussion

2.1 Discussion on MRO for NTN

Considering the joint condition cases of CHO in NTN mobility, i.e., the time-measurement and location-measurement based trigger condition, RAN3 agreed that UE reports the fulfilled CHO trigger conditions before RLF occurs, which benefits the optimization of the condition triggers. According to §5.3.10.5 of TS 38.331 [2], the current RLF report only supports the fulfilled execution condition at the moment of handover failure, or radio link failure. The contents related to joint condition record are defined as follow,

5.3.10.5	RLF report content determination

<<<<<skip the text>>>>

2>	for each neighbour cell, if any, included in measResultListNR in measResultNeighCells:

3>	if the UE supports RLF-Report for conditional handover and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure:

4>	set choConfig in MeasResult2NR to the execution condition for each measId within condTriggerConfig associated to the neighbour cell within the MCG VarConditionalReconfig;

4>	if the first entry of choConfig corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure; or

4>	if the second entry of choConfig, if available, corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure:

5>	set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of choConfig or to the execution condition condSecondEvent corresponding to the second entry of choConfig, whichever execution condition was fulfilled first in time;

5>	set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in choConfig that was fulfilled first in time, and the point in time of fulfilling the condition in choConfig that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the choConfig were fulfilled;

Therefore, we think the current fulfilled condition reporting can not support the logging of historical fulfilled condition  for MRO for NTN, so we suggest to let UE record the historical fulfilled CHO trigger conditions before RLF occurs.

Similar to the current contents of joint trigger condition, the historical fulfilled trigger condition can be recorded in format of a list (e.g., with the maximum length of 8), where each element refers to each neighbour cell. For each neighbour cell, the time interval (to RLF occurrence) and the indication of the fulfilled trigger condition (first-fulfilled-event or second-fulfilled-event) can be provided, in order to optimize the trigger conditions. In summary, there are three logging types:

- The first trigger event: For each neighbour cell, UE only records the first trigger event of each condition (time/location and measurement), respectively. This implies the very beginning of the CHO failure, and may help the NW to analyse whether the trigger conditions are set properly.

- The latest trigger event: For each neighbour cell, UE only records the latest trigger event of each condition (time/location and measurement) before RLF occurs, respectively. This refers to the last chance before the failure occurs and may also benefit the optimization of the trigger conditions.

- All trigger events: For each neighbour cell, UE records all the fulfilled events of each condition (time/location and measurement) before RLF occurs, respectively. This provides the full information before the failure occurs, with which the NW can make thorough analysis of the trigger condition configures. However, this method increases the data volume to be recorded and sent. Thus, the further mechanisms such as setting the maximum record length, and the updating rules of the record buffer can be further discussed.

Based on the discussion above, we propose some details of the new IE definition:

Observation 1: The current IE related to joint condition CHO in RLF report only supports to report the fulfilled execution condition at the moment of handover failure, or radio link failure.

Proposal 1: It is proposed RAN2 to agree on introducing the historical fulfilled CHO trigger conditions in RLF report, and it can be a list (e.g., with maximum length of 8), where each element refers to the records of each neighbour cell.

Proposal 2: It is proposed RAN2 to discuss the logging content of each historical fulfilled CHO trigger condition i.e., the first trigger event, the latest trigger event, and all trigger events.

2.2 Discussion on MDT for NTN

As for the Area Scope for the logged MDT for NTN, considering that 1) the legacy area scope is defined by cell ID, and 2) the area scope in NTN is dynamic and large, the fine-grained area scope definition is needed. Additionally, for UEs in idle/inactive state, the NW cannot get the accurate location or speed of UEs. Hence, merely reusing the current definition with some NW-side enhancement may be insufficient, and the geographical area defined for MBS NTN may be considered to meet such requirements. Thus, to support the logged MDT for NTN according to RAN3’s LS, we think the geographical area (coverageAreaInfoList-r18 defined in SIB 25) is needed.

–	SIB25

SIB25 contains TN coverage information to assist neighbour cell measurements for the UEs in an NTN cell.

SIB25 information element

-- ASN1START

-- TAG-SIB25-START

SIB25-r18 ::=                  SEQUENCE {

coverageAreaInfoList-r18       CoverageAreaInfoList-r18         OPTIONAL,  -- Need R

lateNonCriticalExtension       OCTET STRING                     OPTIONAL,

...

}

CoverageAreaInfoList-r18 ::=   SEQUENCE (SIZE (1..maxTN-AreaInfo-r18)) OF CoverageAreaInfo-r18

CoverageAreaInfo-r18 ::=       SEQUENCE {

tn-AreaId-r18                  TN-AreaId-r18,

tn-ReferenceLocation-r18       ReferenceLocation-r17,

tn-DistanceRadius-r18          INTEGER(0..65535)

}

-- TAG-SIB25-STOP

-- ASN1STOP

SIB25 field descriptions

coverageAreaInfoList Contains a list of TN coverage area's information to assist skipping TN measurements for NTN UEs in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20].

tn-DistanceRadius Distance from the TN coverage area reference location. It is used for skipping TN measurements in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 1m.

Proposal 3: It is proposed RAN2 to agree on reusing the geographical area defined for MBS NTN for Area Scope of logged MDT over Uu for NTN.

3   Conclusion

In this paper, we provide the following observations and proposals:

MRO for NTN:

Observation 1: The current IE related to joint condition CHO in RLF report only supports to report the fulfilled execution condition at the moment of handover failure, or radio link failure.

Proposal 1: It is proposed RAN2 to agree on introducing the historical fulfilled CHO trigger conditions in RLF report, and it can be a list (e.g., with maximum length of 8), where each element refers to the records of each neighbour cell.

Proposal 2: It is proposed RAN2 to discuss the logging content of each historical fulfilled CHO trigger condition i.e., the first trigger event, the latest trigger event, and all trigger events.

MDT for NTN:

Proposal 3: It is proposed RAN2 to agree on reusing the geographical area defined for MBS NTN for Area Scope of logged MDT over Uu for NTN.

4   Reference

[1] R3-247908, LS on RAN3 agreements with impact on UE related to the Rel-19 SON/MDT.

[2] TS 38.331, Radio Resource Control (RRC) protocol specification (Release 18).