3GPP TSG-RAN WG2 Meeting #129                                                                 R2-2501175

Athens, GR, 17th – 21st Feb, 2025

Agenda item:	6.1.2

Source: 	Huawei, HiSilicon

Title: 	On association between RLC entities and PDCP entity

Document for:	Discussion and Decision

1. Introduction

In the current PDCP specification, a DRB is not allowed to be associated with more than one AM RLC entity or more than two UM RLC entity (one for each direction) unless it is configured as a split/SDAP bearer or configured with PDCP duplication. However, it is ambiguous whether DL PDCP duplication is applicable to the above limitation since there is no explicit definition or description for DL PDCP duplication in the specifications.

In this contribution, we will analyse the impacts of the above issue and propose to clarify for the limitation to take DL PDCP duplication into account.

2. Discussion

There are the following limitations on the association between RLC entities and PDCP entity in the current PDCP specification in TS 38.323 [1].

Each RB (except for SRB0 for Uu interface) is associated with one PDCP entity. Each PDCP entity is associated with one, two, three, four, six, or eight RLC entities depending on the RB characteristic (e.g. uni-directional/bi-directional or split/non-split) or RLC mode: - For split bearers, each PDCP entity is associated with two UM RLC entities (for same direction), four UM RLC entities (two for each direction), or two AM RLC entities; - For RBs configured with PDCP duplication, each PDCP entity is associated with N UM RLC entities (for same direction), 2 × N UM RLC entities (N for each direction), or N AM RLC entities, where 2 <= N <= 4; - For DAPS bearers, each PDCP entity is associated with two UM RLC entities (for same direction, one for source and one for target cell), four UM RLC entities (two for each direction on source cell and target cell), or two AM RLC entities (one for source cell and one for target cell); - For UM MRBs, each PDCP entity is associated with one UM RLC entity (for MTCH or for downlink DTCH), two UM RLC entities (one for MTCH and one for downlink DTCH, or one for downlink DTCH and one for uplink DTCH), or three UM RLC entities (one for MTCH, one for downlink DTCH, and one for uplink DTCH); - For AM MRBs, each PDCP entity is associated with one AM RLC entity (for downlink DTCH and uplink DTCH), or one UM RLC entity (for MTCH) and one AM RLC entity (for downlink DTCH and uplink DTCH); - For MP split bearers with SL indirect path, each PDCP entity is associated with one or more Uu RLC entities and one SRAP entity; - For MP split bearers with N3C indirect path, each PDCP entity is associated with one or more Uu RLC entities and the N3C; - Otherwise, each PDCP entity is associated with one UM RLC entity, two UM RLC entities (one for each direction), or one AM RLC entity.

According to the limitations, it is seen that a DRB is not allowed to be associated with more than one AM RLC entity or more than two UM RLC entities (one for each direction) unless it is a split bearer or a SDAP bearer or it is configured with PDCP duplication. It is ambiguous whether the PDCP duplication mentioned in the limitations contains DL PDCP duplication or not. Actually, DL PDCP duplication is not standardized and there is no corresponding configuration or description in the specifications. In this way, people may only consider UL duplication for the limitation, which means DRBs cannot be enabled with DL PDCP duplication without configuring UL PDCP duplication in CA case.

Observation 1: It is ambiguous whether DRB associated with multiple RLC entities is applicable to DL PDCP duplication in the PDCP specification.

However, DL CA PDCP duplication is important since DL traffic is more essential and DL CA is widely used in practical networks. Usually, there are more DL traffics with high reliability requirement. To improve the service quality and the user experience, DL CA duplication is a useful method. In this case, multiple RLC entities are expected to be associated to the same DRB so that DL duplication can be performed by utilizing multiple carriers. Meanwhile, UL PDCP duplication may not be needed.

With the above consideration, DRBs should be allowed to be associated with multiple RLC entities when DL PDCP duplication is used. However, the current PDCP specification is unclear on whether such configuration is allowed or not due to the ambiguity. To avoid different understandings in the practice, RAN2 should give some further clarification. The simplest way is to clarify that both UL and DL PDCP duplication are applicable to the limitation on association between RLC entities and PDCP entity. There is no need to change the specification with the above clarification.

Proposal 1: RAN2 to clarify that DRB associated with multiple RLC entities is applicable to both UL and DL PDCP duplication.

3. Conclusion

In this contribution, we had some discussions on the association between RLC entities and PDCP entity. The following observation and proposal were made:

Observation 1: It is ambiguous whether DRB associated with multiple RLC entities is applicable to DL PDCP duplication in the PDCP specification.

Proposal 1: RAN2 to clarify that DRB associated with multiple RLC entities is applicable to both UL and DL PDCP duplication.

4. Reference

[1]. 3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) protocol specification".