3GPP RAN WG2 Meeting #129	R2-2501180

Athens, Greece, February 17th – 21st 2025

Agenda Item:	8.8.4

Source:	InterDigital

Title:	Support for broadcast service in non-terrestrial networks

Document for:	Discussion, Decision

# Introduction

An objective for Rel-19 NR non-terrestrial networks (NTN) is to provide enhanced signaling support for multicast/broadcast services (MBS). MBS provides an important add-value for NR NTN system, leveraging the large coverage of the NTN compared to TN. Terrestrial MBS features are equally available for NR NTN in the 5G specifications, but for some cases the intended service area is expected to be smaller than the coverage of a Uu cell, requiring enhancements to notify the service area of a Broadcast service.

Detailed objectives from the WID [1] are as follows:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

This contribution discusses necessary enhancements to support broadcast service in non-terrestrial networks, specifically the impact of NTN characteristics on broadcast services such as cell size and cell movement.

# Discussion

## Broadcast service area description

In RAN2#126 [3] it was agreed that both (quasi)earth fixed and earth moving cells are supported for MBS broadcast service. Supporting NGSO satellites requires consideration that the cells move continuously relative to earth. For example, a cell within the boundaries of a country at time T1 may cross into a region where the MBS session is not applicable at time T2. This means that the cells which make up an MBS session area will change over time due to satellite movement, requiring methods to dynamically update the MBS session area.

Observation 1:	Satellite movement in NSGO deployments could cause the cell(s) serving a service area to constantly change.

Figure 2: Due to satellite movement, the cells within a session area will change over time.

To avoid possible service continuity issues, the UE should be aware of how long each cell is serving the intended service area. Whether the existing mechanisms (e.g., t-Service) are sufficient can be further studied, as well as where this information is provided (e.g., within the MBS cell ID list or separately via SIB19).

Proposal 1:	In NGSO deployments, the duration a cell serves a geographic service area is indicated to the UE. FFS whether t-Service is reused, or new signalling is introduced.

The Rel-17/18, UE can trigger various actions (e.g., measurements) prior to cell coverage expiry to help the UE find a suitable cell before coverage disappears. RAN2 may also study whether similar enhancements are needed in NGSO deployments to ensure service continuity during a broadcast service. It is noted that the service continuity discussion may also require coordination with RAN3 (e.g., for network signaling aspects).

Proposal 2:	Study whether existing procedures are sufficient to ensure broadcast service continuity in NGSO deployments.

## Service area information provisioning

In RAN2#127bis, the method of intended service area information provisioning was discussed and RAN2 made the following agreement [4].

Agreements:

- For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

The remaining open issue is how to support skipping MCCH re-acquisition when UE is not located within intended service area of any interested broadcast.

According to the agreement, the association between the MBS session and the intended service area(s) will be provided by an MCCH message. It means that UE knows the association between the broadcast service and the intended service areas only when the UE receives MCCH messages including the interested broadcast session information.

There are 2 main cases for the MCCH modification scenario.

- MCCH modification due to new broadcast session start event.

- MCCH modification due to intended service area change for already started broadcast session(s).

For case 1, all MBS capable UEs would need to acquire the MCCH message and then the UEs check whether their interested services starts or not and so no UE expects skipping MCCH acquisition. If a UE’s interested broadcast starts, then UE would keep re-acquiring the MCCH while the UE is within the intended service areas of the interested broadcast while the interested service is being provided and the UE can stop re-acquiring the MCCH when UE moves out of the intended service areas.

Observation 2: All UEs need to acquire MCCH message for the case of new session start event.

For case 2, the network may add a new intended service area for the already started broadcast. If there is a UE, which is interested in the broadcast service and is located out of the previously configured intended service area of the broadcast service, then the UE needs to acquire the MCCH message so that the UE obtains the newly configured intended service area(s) and the UE starts getting the interested broadcast service when the UE is located in the intended service areas of the interested broadcast service.

Observation 3: Even when UE is out of the intended service area, the UE needs to acquire the updated MCCH message because the new MCCH message may include new intended service areas of the interested service.

By seeing the above observations, all UEs need to acquire the updated MCCH messages regardless of UE location and so from RAN2 point of view, there is nothing to specify. However any smart UE implementation for skipping MCCH re-acquisition should be allowed. Thus, we propose:

Proposal 3:	it’s up to UE implementation to avoid acquiring MCCH when the UE is outside the intended area of the MBS broadcast service.

# Conclusion

In this contribution the following observations and proposals are made regarding support for MBS in NTN

Observation 1:	Satellite movement in NSGO deployments could cause the cell(s) serving a service area to constantly change.

Observation 2:  All UEs need to acquire MCCH message for the case of new session start event.

Observation 3:  Even when UE is out of the intended service area, the UE needs to acquire the updated MCCH message because the new MCCH message may include new intended service areas of the interested service.

Proposal 1:	In NGSO deployments, the duration a cell serves a geographic service area is indicated to the UE. FFS whether t-Service is reused, or new signalling is introduced.

Proposal 2:	Study whether existing procedures are sufficient to ensure broadcast service continuity in NGSO deployments.

Proposal 3:	it’s up to UE implementation to avoid acquiring MCCH when the UE is outside the intended area of the MBS broadcast service.

# References

RP-241789 "Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3", Thales, CATT

R2-2404102 RAN2#125bis chair notes

R2-2406202 RAN2#126 chair notes

RAN2#127bis chair notes