3GPP TSG-RAN-WG2 Meeting #129	R2- 2501182

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item	:		8.8.2

Source	: 	Sharp

Title	: 	Discussion on DL coverage enhancement

Document for	:	Discussion and Decision

Introduction

In the RAN2 #128 meeting, RAN2 made some agreements as follows [1].

Agreements:

- If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

Agreements:

1.	As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

2.	RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

Agreements:

1.	For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states.

In this paper, we further discuss the impacts for larger SSB periodicity and DTX configuration.

Discussion

[Access Control]

In the last meeting, RAN2 agreed to introduce new barring in SIB1 to be able to selectively bar UEs supporting DL CE if it is needed to bar UEs not supporting DL-CE via legacy mechanism. In this case, to avoid the useless cell reselection of legacy UE, NW should also broadcast barring information for neighbour cells. In the current spec, SIB3/SIB4 includes the excluded cell list for intra-frequency/inter-frequency neighbouring cells. To prevent re-selection of legacy UE, it is natural to include cells operating with DL CE into intraFreqExcludedCellList (in SIB3) and interFreqExcludedCellList (in SIB4). In addition, new information to indicate whether listed cell is barred or not for UEs supporting DL CE should be introduced.

Observation 1: To avoid the useless cell reselection of legacy UE, NW should also broadcast barring information for neighbour cells.

Proposal.1: If it is needed to bar UEs not supporting DL-CE (via legacy mechanism), cells operating with DL CE are included into intraFreqExcludedCellList (in SIB3) and interFreqExcludedCellList (in SIB4). In addition, new information to indicate whether listed cell is barred or not for UEs supporting DL CE is introduced.

[SMTC]

It was agreed that RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity. SMTC is a parameter used to neighbour cell measurements and can specify the window of the SSB to be measured per frequency/cell by using the parameters of periodicity, offset, and duration.

The maximum measurement window for one SSB burst is currently 5ms, and SSB periodicity and offset can be supported up to 160ms in the current specification. For DL CE, there is no requirements to extend the number of SSB index (i.e. extension of duration), and current assumed extended SSB periodicity is 160ms. So, SMTC parameters (periodicity, offset, and duration) are not needed to extend.

On the other hand, to cover various beam hopping patterns, it was proposed to increase the total number of SMTC configuration. In the current spec, 4 SMTCs (smtc1 + 3 x smtc4) can be configured per frequency for NR NTN. Though this is originally introduced to cover differential propagation delay between satellites, if the hopping pattern is as shown in Figure.1, 4 SMTC configuration should be enough even if UE is located among multiple satellites.

Figure 1

Observation 2: Though 4 SMTCs (smtc1 + 3 x smtc4) was introduced to cover differential propagation delay between satellites, the number of current configurations is enough to support DL CE cells.

Proposal.2: Additional SMTC is not needed.

[Cell DTX / Beam DTX]

Using NES cell DTX/DRX mechanism is considered for NR NTN power sharing purpose. Current Cell DTX parameters are configured per serving cell. So, if cell DTX mechanism is reused for DL CE and each beam has different DTX pattern, NW needs to reconfigure cell DTX pattern to align with the pattern of concerned beam each time UE moves to a different beam area, and this signalling overhead becomes large.

So, we think it is needed to consider how to adopt the cell DTX/DRX mechanism with reducing the signalling overhead.

There are at least two types of options to adopt NES DTX for NTN beam power sharing.

Option 1: one NTN payload serves small cells formed by one or a few beams and each cell is associated to a DTX configuration like R18 NES.

Option 2: one NTN payload serves some cells formed by one or multiple beams and each beam is associated to a DTX configuration.

If option 1 is used, there are few changes from R18 NES specification for RRC Connected mode and there are less works in RAN2. However, if option 1 is used, handover should be occurred frequently compared with option 2. So, we think option 2 should be considered to reduce the signalling overhead and latency.

Proposal 3: Beam level DTX configuration should be considered for DL coverage enhancement to reduce the frequent handover.

Conclusion

In this paper, we made following observations and proposals;

[Access Control]

Observation 1: To avoid the useless cell reselection of legacy UE, NW should also broadcast barring information for neighbour cells.

Proposal.1: If it is needed to bar UEs not supporting DL-CE (via legacy mechanism), cells operating with DL CE are included into intraFreqExcludedCellList (in SIB3) and interFreqExcludedCellList (in SIB4). In addition, new information to indicate whether listed cell is barred or not for UEs supporting DL CE is introduced.

[SMTC]

Observation 2: Though 4 SMTCs (smtc1 + 3 x smtc4) was introduced to cover differential propagation delay between satellites, the number of current configurations is enough to support DL CE cells.

Proposal.2: Additional SMTC is not needed.

[Cell DTX / Beam DTX]

Proposal 3: Beam level DTX configuration should be considered for DL coverage enhancement to reduce the frequent handover.

Reference

- Draft Report of 3GPP TSG RAN WG2 meeting #128