3GPP TSG-RAN2 #129	R2-2501186

Athens, Greece, Feb.  17th – 21st, 2025

Agenda item:	8.6.3 (NR_Mob_Ph4-Core)

Source:	LG Electronics Inc.

Title: 	Event triggered LTM report

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss issues related to event triggered LTM measurement report.

- Agreements of RAN2#128 - TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. - TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam. - TTT is applied to the leaving condition. - Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation. - Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1. - SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE. - For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE. - For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE. - A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals. - Revisit it once we can have better understanding on MR MAC CE format and how much bits actually can be saved from differential RSRP values. - Support the truncated measurement report MAC CE.

2. Discussion

Restriction of successive triggering of LTM MR MAC CEs

At RAN2#128, TTT per beam was agreed. When TTT per beam is applied, it may happen that two or more beams satisfy the entering condition of the same event successively. In this case, each TTT corresponding to different beam expires at slightly different times, and consequently multiple LTM MR MAC CEs are triggered successively, as shown in Figure 1.

Figure 1. Successive LTM MRs triggered by the same report config

Note that such successive reporting can already occur in event triggered L3 MR. If two different cells of the same measurement object satisfy the entry condition of the same event at different times (say, cell A first and then cell B) and they keep satisfying the entry condition for TTT respectively, the cell A triggers an initial measurement reporting upon expiry of TTT for cell A and the cell B shortly triggers a subsequent measurement reporting upon expiry of TTT for cell B. If the second measurement reporting is triggered, the numberOfReportsSent is reset but the existing cellsTriggeredList is not reset but updated by the cell B (i.e., cellB is included into the cellsTriggeredList).

If triggering of MR MAC CE follows event triggered L3 MR approach, the reporting may be frequent. For example, if the acquisition times of beam measurement results for different beams are slightly different, or two different beams satisfy the entering condition at slightly different times, each beam will trigger reporting successively. Higher dynamics of beam quality also contributes to a frequent subsequent LTM reporting, compared to L3 cell quality, provided that L2 filtering of L1 measurement results is not introduced.

To reduce successive triggering of LTM MR MAC CEs within a short period of time, it is beneficial to introduce some restrictions in triggering LTM MR MAC CEs. For example, when the LTM MR MAC CE is triggered at t1 by the beam1 for an LTM event, if there is other beam (beam2 and beam3) for which TTT is running for the same event, the LTM MR MAC CE at t1 includes the beam1 as triggering beam, and also includes beam2 and beam3. Since the LTM MR MAC CE includes beam1, beam2, and beam3 with beam1 as triggering beam, the necessity of the triggering subsequent LTM MR MAC CE by beam2 and beam3 at t2 and t3 becomes weakened. Then, UE can refrain from triggering the subsequent LTM MR MAC CEs, regardless of the TTT status for these beams, as shown in Figure2. The details of how to refrain from triggering the subsequent LTM MR MAC CEs can be further discussed at the next meeting once RAN2 agrees to introduce some restrictions in triggering LTM MR MAC CEs.

Proposal 1: When LTM MR MAC CE is triggered by the beam (i.e., triggering beam) for a LTM report config ID, the LTM MR MAC CE includes the triggering beam and other beam(s) for which TTT is running for the same report config ID.

Proposal 2: To introduce some restrictions to reduce or prevent successive LTM reporting triggered by different beams of the same candidate cell for the same report config ID. Details of how to restrict is FFS.

Triggering of LTM MR MAC CE by J good/better beams

For selecting a cell switch candidate, network may be interested in a candidate cell that can provide at least J good beams. For example, network may want to trigger cell switch to a cell only if the cell can offer at least two good beams. In this case, if UE sends a first Event LTM report triggered by a single good beam, network has to wait until subsequent report including two good triggering beams are received. In this case, the first Event LTM report is considered redundant from network point of view, because the subsequent report including the sufficient good beams is only necessary for triggering a cell switch to the cell.

To avoid such redundant/unnecessary reporting, network can configure a restriction parameter J for configured Event LTMs, so that the event is considered to be met if at least J triggering beams satisfy the entry condition of the Event LTM. For example, with J=2, in case of Event LTM3, UE triggers reporting only if at least 2 beams of a candidate cell are offset better than the serving cell beam.

Proposal 3: Network can configure J for Event LTMs. If J is configured, the event is considered to be met if at least J beams of the candidate cell satisfy the entry condition of the event for TTT.

Activation/deactivation of Event LTM report conditions

When a candidate cell becomes a new serving cell, UE applies the candidate cell’s configuration including Event LTM report configuration, as serving cell configuration. If the Event LTM report configuration includes several (or large number of) LTM events/conditions, satisfaction of entry/leaving conditions of those events would happen frequently, and this will result in large LTM report signaling overhead. In contrast, if the Event LTM report configuration includes minimal LTM events/conditions, immediate LTM report signaling overhead would be small, but the new serving cell may need to configure UE with additional LTM events/conditions to trigger proper LTM reports via reconfiguration, which also increases signaling overhead. Furthermore, for LTM cell switching, network may need to update Event LTM report conditions more frequently to adapt to changes in beam conditions, and if we consider subsequent cell switching, the resulting update of Event LTM report conditions could be more frequent. Considering the potential UL signaling overhead in transmitting LTM MR MAC CEs and DL signaling overhead in transmitting LTM report condition reconfiguration, we think dynamic activation or deactivation of LTM Events via L1/L2 signaling would be beneficial. Network can configure several LTM report conditions and activate some of them as per their demand dynamically.

Proposal 4: Network can activate or deactivate some of the configured Event LTM report conditions via MAC CE.

Construction of N beams in LTM MR MAC CE

RAN2#127bis made the following agreements.

Agreements on L1 event triggered MR 13. Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW. 14. Additional information included in MR MAC CE: - The information and quantity of current beam, based on NW configuration.

Given that the MR MAC CE can include up to N beams, we should decide which N beams should be included in the report. In our view, it is beneficial for MR MAC CE to include the following information.

- Triggered beam information as essential information

  - The triggering beam is the beam that has fulfilled the event (entry condition is met for TTT duration)

- Non-triggered beam information as additional information

  - For Event LTM 3/4/5, best beam(s) other than the triggering beam of the same candidate cell can be included. This information is useful for network to figure out the overall quality of beams of the concerned candidate cell.

  - For Event LTM2, best beam(s) across all candidate cells can be included as assistance information. This information is useful for network to roughly figure out quality of configured candidate cells.

- Serving cell’s activated beam information as additional information

  - Event LTM2/3/4/5, best activated beam(s) of the serving cell can be included. This information is useful for network to determine whether cell switch is better or beam switching without cell switch is better

Since the maximum number of beams is N, UE should first fill up the all of k triggered beams, and then if the number of triggered beams is less than N, UE can further fill up the non-triggered best beams up to min(L, N-k) beams, and then if there is remaining room, UE can further fill the best activated beams.

Proposal 5: As additional information, activated beams of the special cell can be included in LTM MR MAC CE, based on network configuration.

Proposal 6: For Event LTM 3/4/5, the N beams in LTM MR MAC CE comprise

- a) All of k triggered beam(s) up to N beams, if current beam is not included, or up to N-1 beams, if current beam is included (those fulfilling the event),

- b) Current beam information, if configured to be included, after filling up triggered beam(s)

- c) Non-triggering best beams of the candidate cell, after filling up the triggering beams, up to min(L, remaining room) beams, if L is configured.

- d) Activated beams of the special cell, after filling up the triggering beams and non-triggering beams, if remaining room is available

Proposal 7: For Event LTM 2, the N beams in LTM MR MAC CE comprise

- a) One triggered beam (the one fulfilling the event),

- b) Activated beams of the special cell up to min (L, remaining room) beams, if L is configured

- c) Best beams across all candidate cells, after filling up the activated beams, if remaining room is available

L3 measurement results in LTM MR MAC CE

L3 cell quality of the serving/candidate cell is a good side information for the decision making at network side on whether to trigger cell switch.

Regarding the L3 measurement result of the special cell, since L3 measurement result of the special cell is always available, UE can simply include L3 RSRP of the special cell in the LTM MR MAC CE. It is desirable for network to be able to control UE via RRC whether is required to include L3 RSRP if the special cell in the report.

Regarding the L3 measurement result of the candidate cell, if the candidate cell beam satisfies Event LTM, it is quite likely that L3 measurement result of the candidate cell is available. Therefore, UE can also include L3 RSRP of the candidate cell to assist network cell switch decision. It is desirable for network to be able to control UE via RRC whether UE is required to include the L3 RSRP of the candidate cell in the report.

Proposal 8: L3 measurement result of the concerned special cell is included in the LTM MR MAC CE, if configured to be included via include/notInclude flag in RRC report configuration of event triggered LTM MR.

Proposal 9: L3 measurement result of the concerned candidate cell is included in the LTM MR MAC CE, if configured to be included via include/notInclude flag in RRC report configuration of event triggered LTM MR.

Truncated MR MAC CE

RAN2 agreed in RAN2#128 to introduce truncated LTM MR MAC CE. RAN2 need to discuss the details related to Truncated MR MAC CE

Proposal 10: The truncated LTM MR MAC CE includes triggered beam ID (SSBRI or CRI) and the report config ID (event config ID), without L1 RSRP/SINR

For truncated LTM MR MAC CE, it is straightforward to use a separate LCID from non-truncated LTM MR MAC CE.

Proposal 11: For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

Handling of multiple LTM MRs triggered

RAN2 agreed in RAN2#127bis that legacy SR procedure is used for transmission of LTM MR MAC CE, and dedicated SR for LTM MR MAC CE is introduced.

Single MAC CE vs multiple MAC CEs

When UE receives UL grant, UE may have more than one LTM MRs to be transmitted. For the case the UL grant is insufficient to transmit multiple LTM MRs, RAN2 has to decide whether UE shall generate a single LTM MR MAC CE containing multiple LTM MRs or multiple LTM MR MAC CEs.

In our view, a single LTM MR MAC CE containing multiple LTM MRs is simpler because it incurs less signaling overhead, given that inclusion of single LCID suffices. With this approach, LTM MR MAC CE should have a field indicating how many LTM MRs are included in the LTM MR MAC CE. In addition, length-field for each included LTM MR is needed for the receiving side to know the boundary between LTM MRs.

Proposal 12: If multiple LTM MRs are included in a single MAC PDU, a single LTM MR MAC CE containing multiple LTM MRs is generated.

Proposal 13: LTM MR MAC CE has a field indicating how many LTM MRs are included in the LTM MR MAC CE, and a length-field for each LTM MR.

Non-truncated format MAC CE vs truncated format MAC CE,

When UE receives UL grant, UE may have more than one LTM MRs to be transmitted. If the UL grant is insufficient to transmit all the LTM MRs, UE should decide which LTM MR(s) to send first, and/or which MAC CE format to use between non-truncated format and truncated format to be tailored for the UL grant.

Proposal 14: For UE having more than one LTM MRs to be transmitted, when UE receives UL grant, if the UL grant is sufficient to transmit multiple LTM MRs in truncated MAC CE format but insufficient to transmit multiple LTM MRs non-truncated MAC CE format, it is up to UE implementation which MAC CE format to use.

Prioritization of LTM MR among multiple LTM MRs

For UE having more than one LTM MRs triggered by the same event LTM config to be transmitted, when UE receives UL grant insufficient to transmit all the LTM MRs, UE should decide which LTM MR(s) to transmit first. Among LTM MRs associated with the same event LTM config, UE may prioritize the latest one, since the latest one is the most relevant information with respect to cell switch decision making at network side, but such prioritization adds UE complexity, Among LTM MRs associated with the different event LTM configs, the order of the event fulfillment is more relevant, but as long as those LTM MRs are not lost, the transmission order is not so important. Considering these, we believe which LTM MR to transmit first can be left to UE implementation.

Proposal 15: For UE having more than one LTM MRs triggered by the same event LTM config to be transmitted, when UE receives UL grant insufficient to transmit all the LTM MRs, it is up to UE implementation which LTM MR(s) to transmit first in the LTM MR MAC CE.

Transmission of remaining LTM MRs

When LTM MR MAC CE triggered by a beam of a candidate cell fulfilling a concerned LTM event is transmitted, if there is a pending SR already triggered for LTM MR, one may consider the SR to be cancelled. Regarding the SR cancellation, we need to carefully consider in which case the SR should be cancelled or should not be cancelled. For example, if the LTM event associated with the transmitted LTM MR MAC CE (LTM MR MAC CE#1 for LTM MR#1) is different from the LTM event associated with the pending SR triggered by another LTM MR (say LTM MR2), cancellation of the pending SR results in delayed transmission of contents related to LTM MR2.

One approach to avoid delayed transmission of remaining LTM MR(s) could be that pending SR triggered for LTM MR triggered by a certain LTM event config is cancelled if transmitted MAC PDU contains LTM MR triggered by the same LTM event config, but not cancelled if transmitted MAC PDU contains LTM MR triggered by different LTM event config.

Another approach to avoid delayed transmission of remaining LTM MR(s) could be to introduce a flag, in LTM MR MAC CE, indicating whether there is another LTM MR(s) pending to transmit. This should be applied to both non-truncated LTM MAC CE and truncated LTM MAC CE. In this approach, pending SR previously triggered for any LTM MR MACE can be cancelled. The indication may further indicate the number of remaining LTM MRs. The indication may further indicate information size related to remaining LTM MRs

In our view, an explicit indication approach is simpler and thus preferred.

Proposal 16: LTM MR MAC CE includes a flag indicating whether there is another LTM MR pending to transmit. This is applied to both non-truncated LTM MAC CE and truncated LTM MAC CE.

3. Conclusion

Restriction of successive triggering of LTM MR MAC CEs

Proposal 1: When LTM MR MAC CE is triggered by the beam (i.e., triggering beam) for a LTM report config ID, the LTM MR MAC CE includes the triggering beam and other beam(s) for which TTT is running for the same report config ID.

Proposal 2: To introduce some restrictions to reduce or prevent successive LTM reporting triggered by different beams of the same candidate cell for the same report config ID. Details of how to restrict is FFS.

Triggering of LTM MR MAC CE by J good/better beams

Proposal 3: Network can configure J for Event LTMs. If J is configured, the event is considered to be met if at least J beams of the candidate cell satisfy the entry condition of the event for TTT.

Activation/deactivation of Event LTM report conditions

Proposal 4: Network can activate or deactivate some of the configured Event LTM report conditions via MAC CE.

Construction of N beams in LTM MR MAC CE

Proposal 5: As additional information, activated beams of the special cell can be included in LTM MR MAC CE, based on network configuration.

Proposal 6: For Event LTM 3/4/5, the N beams in LTM MR MAC CE comprise

- a) All of k triggered beam(s) up to N beams, if current beam is not included, or up to N-1 beams, if current beam is included (those fulfilling the event),

- b) Current beam information, if configured to be included, after filling up triggered beam(s)

- c) Non-triggering best beams of the candidate cell, after filling up the triggering beams, up to min(L, remaining room) beams, if L is configured.

- d) Activated beams of the special cell, after filling up the triggering beams and non-triggering beams, if remaining room is available

Proposal 7: For Event LTM 2, the N beams in LTM MR MAC CE comprise

- a) One triggered beam (the one fulfilling the event),

- b) Activated beams of the special cell up to min (L, remaining room) beams, if L is configured

- c) Best beams across all candidate cells, after filling up the activated beams, if remaining room is available

L3 measurement results in LTM MR MAC CE

Proposal 8: L3 measurement result of the concerned special cell is included in the LTM MR MAC CE, if configured to be included via include/notInclude flag in RRC report configuration of event triggered LTM MR.

Proposal 9: L3 measurement result of the concerned candidate cell is included in the LTM MR MAC CE, if configured to be included via include/notInclude flag in RRC report configuration of event triggered LTM MR.

Truncated MR MAC CE

Proposal 10: The truncated LTM MR MAC CE includes triggered beam ID (SSBRI or CRI) and the report config ID (event config ID), without L1 RSRP/SINR

Proposal 11: For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

Handling of multiple LTM MRs triggered>>Single MAC CE vs multiple MAC CEs

Proposal 12: If multiple LTM MRs are included in a single MAC PDU, a single LTM MR MAC CE containing multiple LTM MRs is generated.

Proposal 13: LTM MR MAC CE has a field indicating how many LTM MRs are included in the LTM MR MAC CE, and a length-field for each LTM MR.

Handling of multiple LTM MRs triggered>>Non-truncated format MAC CE vs truncated format MAC CE,

Proposal 14: For UE having more than one LTM MRs to be transmitted, when UE receives UL grant, if the UL grant is sufficient to transmit multiple LTM MRs in truncated MAC CE format but insufficient to transmit multiple LTM MRs non-truncated MAC CE format, it is up to UE implementation which MAC CE format to use.

Handling of multiple LTM MRs triggered>>Prioritization of LTM MR among multiple LTM MRs

Proposal 15: For UE having more than one LTM MRs triggered by the same event LTM config to be transmitted, when UE receives UL grant insufficient to transmit all the LTM MRs, it is up to UE implementation which LTM MR(s) to transmit first in the LTM MR MAC CE.

Handling of multiple LTM MRs triggered>>Transmission of remaining LTM MRs

Proposal 16: LTM MR MAC CE includes a flag indicating whether there is another LTM MR pending to transmit. This is applied to both non-truncated LTM MAC CE and truncated LTM MAC CE.