3GPP TSG RAN WG2 Meeting #129                                                          R2- 2501216

# Athens, Greece, February 17-21, 2025

Agenda item:		8.5.3

Source:			III

Title:		On-demand SIB1 for Idle/Inactive mode UEs

Document for:	 	Discussion

- Introduction

In RAN#105, the issue regarding on-demand SIB1 for idle/inactive mode UEs was approved to enter normative phase from study phase with focus on case 2 only, whose up-to-date objectives [1] are listed as follows.

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In RAN2#128, some relevant agreements [2] were made, including:

Agreements on OD-SIB1: 1. NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy. 2. New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency. 3. Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs. 4. Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold). 5. The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission. 6. The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request. 7. If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact. 8. A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell. 9. UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level. 10. Working assumption: UL-WUS configuration in RRC release is not supported.

This contribution provides some input on WUS configuration validity and SIB1 checking before on-demand requests for further discussions, which is related to some of the agreements [3] in RAN2#127-bis.

- Discussion

WUS configuration validity

RAN2#127-bis agreement [3]:

- A NES cell can include neighboring NES cell’s WUS configuration

In [1], it is agreed that the WUS configuration of any NES cells is normally retrieved from a Cell A. However, if an NES cell is to provide the WUS configuration of its neighboring NES cells, this will seemingly make no difference between a Cell A and an NES cell, which goes against the purpose of energy saving on NES cells.

Meanwhile, it is also agreed that for a Cell A which is periodically transmitting its own SIB1, the WUS configuration of any NES cells is regularly provided via a new SIB in Cell A. However, for an NES cell which may transmit SIB1s in response to UL WUS from a UE, it can only do so during the period of its on-demand SIB1 transmission. Beyond that, the WUS configuration of its neighboring NES cells may not be possibly retrieved.

Moreover, if an NES cell happens to be in the coverage of more than one Cell A, the provision of neighboring NES cell’s WUS configuration will become more complicated, because extra filtering on the NES cell will be needed to determine which neighboring NES cells are located in the coverage of the same Cell A.

Proposal 1. RAN2 focuses on the provision of NES cell’s WUS configuration by Cell A, and deprioritizes that by neighboring NES cells.

SIB1 checking before on-demand requests

RAN2#127-bis agreement [3]:

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Checking if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell does have the benefit of reducing redundant on-demand SIB1 requests from other NES UEs. However, such checking is strongly related to the issues below:

In RAN1 #118, the following is agreed for time domain on-demand SIB1 transmission behavior: Agreement At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration Agreement At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, on reference time point to determine the window starting time, RAN1 to down select from the following two options: - Option 1: The reference time point is defined based on the RAR reception time of the UL-WUS transmission - FFS: Definition of RAR reception time - Option 2: The reference time point is defined based on the UL-WUS transmission time - Option 3: The reference time point is defined based on the RAR window of the UL-WUS transmission Agreement For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options: - Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell - Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration - Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS. Combination of multiple options is not precluded.

As these issues are still being discussed in RAN1 [4, 5], and therefore RAN2 may start discussions on the SIB1 checking after some progresses have been made in RAN1.

Proposal 2. Considering the strong relationship with issues being discussed in RAN1, RAN2 may start discussions on the SIB1 checking after some progresses have been made in RAN1.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1. RAN2 focuses on the provision of NES cell’s WUS configuration by Cell A, and deprioritizes that by its neighboring NES cells.

Proposal 2. Considering the strong relationship with issues being discussed in RAN1, RAN2 may start discussions on the SIB1 checking after some progresses have been made in RAN1.

References

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR.

[2] Draft RAN2 128 Meeting Report v1.

[3] Draft RAN2 127b Meeting Report v3.

[4] R1-2409016 FL summary 5 for on-demand SIB1.

[5] R1-2410681 FL summary 5 for on-demand SIB1.