3GPP TSG-RAN WG2 Meeting #129	R2-2501220

Athens, Greece, Feb. 17th – 21st, 2025

Source:        ZTE Corporation

Title:           Clarification on the less than 5M Configuration

Agenda item:   7.8.1

Document for:  Discussion and Decision

- Introduction

In the RAN2#127bis meeting, a LS [1] was sent to RAN4. In the last RAN4 meeting, a Reply LS[2] was sent by RAN4 regarding how to indicate 12PRB/20PRB in the SIB1.

RAN4 would like to thank RAN2 for their LS R2-2409409. RAN4 confirms that it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling. In addition, RAN4 would like to provide the background that 12 PRB and 20 PRB transmission bandwidth configuration are two special cases only for band n100, and only can be configured with SS Block frequency positions in additional GSCN parameters defined in TS 38.101-1 Table 5.4.3.1-3. Therefore, in these two special cases UE can determine 12 or 20 PRB transmission bandwidth configuration should be used instead of 15 or 25 PRB transmission bandwidth configuration, respectively, even though carrierBandwidth is configured to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

In this paper, we’d like to share our views on this LS.

- Discussion

In this chapter, we will first give a short background description, and then share our understanding of this LS.

For the following 2 GSCNs, only the 12PRB/20 PRB transmission bandwidth can be configured, however the defined minimum UE supported channel bandwidth is 15PRB/20PRB respectively.

Table 1: GSCN for 12PRB/20 PRB Transmission Bandwidth

| SSREF(MHz) | GSCN | Note |
| --- | --- | --- |
| 920.73 | 41637 | Only applicable for 12 PRB transmission bandwidth configuration within 3 MHz channel with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |
| 921.45 | 41638 | Only applicable for 20 PRB transmission bandwidth configuration within 5 MHz channel with unpunctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |

During the offline discussion of previous meetings, companies pointed out the issue on the cell barring check upon receiving the SIB1.Thus, most companies suggested to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1.

2> if the UE supports a downlink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]) which - is smaller than or equal to the carrierBandwidth (indicated in downlinkConfigCommon for the SCS of the initial downlink BWP ….. 2> else: 3> consider the cell as barred in accordance with TS 38.304 [20]; and

On this suggestion, some companies also point out that the carrierBandwidth configured in the SIB1 was also used in the RAN1 spec, e.g.

TS 38.211 - 4.4.2 Resource grid < -- Omitted -- > For uplink and downlink, the carrier bandwidth Ngridsize,μ for subcarrier spacing configuration μ is given by the higher-layer parameter carrierBandwidth in the SCS-SpecificCarrier IE. The starting position Ngridstart,μ for subcarrier spacing configuration μ is given by the higher-layer parameter offsetToCarrier in the SCS-SpecificCarrier IE. The frequency location of a subcarrier refers to the center frequency of that subcarrier. < -- Omitted -- > - 5.3.1 OFDM baseband signal generation for all channels except PRACH and RIM-RS The time-continuous signal on antenna port p and subcarrier spacing configuration μ for OFDM symbol in a subframe for any physical channel or signal except PRACH is defined by sl(p,μ)t=sl(p,μ)ttstart,lμ≤t<tstart,lμ+Tsymb,lμ0otherwise sl(p,μ)t=k=0Ngrid,xsize,μNscRB-1ak,l(p,μ)ej2πk+k0μ-Ngrid,xsize,μNscRB2Δft-NCP,lμTc-tstart,lμ k0μ=Ngrid,xstart,μ+Ngrid,xsize,μ2NscRB-Ngrid,xstart,μ0+Ngrid,xsize,μ02NscRB2μ0-μ Tsymb,lμ=Nuμ+NCP,lμTc

Thus an LS was sent to RAN4 and copied to RAN1 for the confirmation.

Now, according to the RAN4 reply LS, two main points are included:

- In terms of the network setting, it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

- In terms of the UE’s behaviour, the UE can determine that 12PRB or 20 PRB transmission bandwidth configuration should be used respectively, even though carrierBandwidth is configured to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

The bullet a) can solve the cell barring check issue upon receiving the SIB1, while the bullet b) can be used to avoid any impact on the RAN1 spec. Thus both of these points should be specified. Furthermore, though the issue is caused by the barring check upon SIB1 receiving, based on the previous agreements:

- RAN2#115 - Fields that are present in ServingCellConfigCommon delivered by dedicated signalling shall have the same value as the corresponding field in SIB1. - Confirm that dedicatedSIB1-Delivery shall have the same fields and values as the broadcasted SIB1.

The same clarification should also be added to the dedicated RRC signaling. Since the SIB and the dedicated signaling share the same ASN.1 structure for the “SCS-SpecificCarrier-> carrierBandwidth”, the above two bullets can be added to the field description of the carrierBandwidth immediately.

Proposal 1: Add the following two clarifications to the field description of the carrierBandwidth for the case where only 12/20 PRB transmission bandwidth can be used:

- The network shall configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

- The UE shall use 12/20 PRB as the transmission bandwidth, even though carrierBandwidth is configured to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

Based on proposal 1, we can further discuss the issue on the “Cond LessThan5MHz” in the [4].

InterFreqCarrierFreqInfo-v1800 ::= SEQUENCE { dl-CarrierFreq-r18 ARFCN-ValueNR OPTIONAL, -- Cond LessThan5MHz frequencyBandList-r18 MultiFrequencyBandListNR-SIB OPTIONAL, -- Cond LessThan5MHz }

| Conditional Presence | Explanation |
| --- | --- |
| LessThan5MHz | The field is mandatory present if the carrierBandwidth in SIB1 indicates UL or DL transmission bandwidth other than 15 PRB, and the corresponding neighbour cell(s) support(s) 12 PRB, 15 PRB or 20 PRB transmission bandwidth configuration as defined in TS 38.101-1 [15], TS 38.211 [16] and TS 38.213 [13]. Otherwise, the field is optional, Need S. |

The intention of this condition is that if a cell doesn’t support less than 5M transmission bandwidth configuration but the neighbor cell(s) support(s) less than 5M transmission bandwidth, the cell shall mandatorily indicate dl-CarrierFreq-r18/ frequencyBandList-r18. However, even if the 12PRB/20 PRB were added as below, this intention can’t be described exactly for the asymmetric case.

| LessThan5MHz | The field is mandatory present if the carrierBandwidth in SIB1 indicates UL or DL transmission bandwidth other than 12PRB,15 PRB or 20PRB, and the corresponding neighbour cell(s) support(s) 12 PRB, 15 PRB or 20 PRB transmission bandwidth configuration as defined in TS 38.101-1 [15], TS 38.211 [16] and TS 38.213 [13]. Otherwise, the field is optional, Need S. |
| --- | --- |

For the asymmetric case (e.g. band 28 with UL 3M and DL 25M), the cell can configure UL with 3M (15 PRB) and DL with 5M (25 PRB). In this case it’s not mandatory for this cell to indicate dl-CarrierFreq-r18/ frequencyBandList-r18. But according to the current description, the condition “DL transmission bandwidth other than 12PRB,15 PRB or 20PRB” can be satisfied and thus this cell has to indicate dl-CarrierFreq-r18/ frequencyBandList-r18. To avoid such issue, the following modification is suggested, which is also similar to the wording used in the MeasIdleConfig field descriptions.

| LessThan5MHz | The field is mandatory present if the carrierBandwidth in SIB1 indicates UL or DL transmission bandwidth other than 15 PRB a cell doesn’t support 12 PRB, 15 PRB or 20 PRB transmission bandwidth, and but the corresponding neighbour cell(s) support(s) 12 PRB, 15 PRB or 20 PRB transmission bandwidth configuration as defined in TS 38.101-1 [15], TS 38.211 [16] and TS 38.213 [13]. Otherwise, the field is optional, Need S. |
| --- | --- |

Proposal 2: Change the “Cond LessThan5MHz” description as below:

| LessThan5MHz | The field is mandatory present if the carrierBandwidth in SIB1 indicates UL or DL transmission bandwidth other than 15 PRB a cell doesn’t support 12 PRB, 15 PRB or 20 PRB transmission bandwidth, and but the corresponding neighbour cell(s) support(s) 12 PRB, 15 PRB or 20 PRB transmission bandwidth configuration as defined in TS 38.101-1 [15], TS 38.211 [16] and TS 38.213 [13]. Otherwise, the field is optional, Need S. |
| --- | --- |

The corresponding CR based on the above proposals was also provided as in [5].

Proposal 3: Agree the related CR in [5].

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: Add the following two clarifications to the field description of the carrierBandwidth for the case where only 12/20 PRB transmission bandwidth can be used:

- The network shall configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

- The UE shall use 12/20 PRB as the transmission bandwidth, even though carrierBandwidth is configured to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively in SIB1 and in RRC dedicated signaling.

Proposal 2: Change the “Cond LessThan5MHz” description as below:

| LessThan5MHz | The field is mandatory present if the carrierBandwidth in SIB1 indicates UL or DL transmission bandwidth other than 15 PRB a cell doesn’t support 12 PRB, 15 PRB or 20 PRB transmission bandwidth, and but the corresponding neighbour cell(s) support(s) 12 PRB, 15 PRB or 20 PRB transmission bandwidth configuration as defined in TS 38.101-1 [15], TS 38.211 [16] and TS 38.213 [13]. Otherwise, the field is optional, Need S. |
| --- | --- |

Proposal 3: Agree the related CR in [5].

- Reference

- R2-2409409  LS on carrierBandwidth configuration for less-than-5MHz carriers          RAN2

- R4-2420382  Reply LS on carrierBandwidth configuration for less-than-5MHz carriers     RAN4

- R2-2109301  Report of 3GPP TSG RAN WG2 meeting #115-e, Online                 RAN2

- R2-2409055  Correction on carrier bandwidth configuration for less than 5MHz	 Huawei, HiSilicon

- R2-2501221	  Correction on the Less than 5M Configuration	38.331  18.4.0  ZTE Corporation