3GPP TSG-RAN WG2 Meeting #129	R2-2501222

Athens, Greece, Feb. 17th – 21st, 2025

Source:        ZTE Corporation

Title:           Consideration on the Asymmetric DL sTRP/UL mTRP

Agenda item:   8.12.2

Document for:  Discussion and Decision

- Introduction

In the previous RAN2 meetings, the PL offset MAC CE was discussed and some agreements have been achieved as below:

=>New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID. =>Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB. =>In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states.

Meanwhile, RAN1 also send a LS [1] on the 2TA topic, in this contribution, we’d like to share our understanding on these topics.

- Discussion

In this chapter, we first discuss more detail issues on the PL offset MAC CE, then focus the 2TA impact on RAN2 spec.

2.1 PL Offset MAC CE

In this chapter, we’d like to discuss some details issues for the PL offset update MAC CE.

Issue 1: Relationship between the RRC parameter and the MAC CE

In the last meeting, the relationship between the RRC parameter and the MAC CE was also discussed in many companies’ contributions [2][3][4][5]. For this topic, as analyzed in the [2], 2 scenarios need to be considered:

- Scenario 1: The UE was configured with PL offset value 1 by the RRC configuration message first and then receive a PL offset MAC CE with PL offset value 2, after that, the UE receives a second RRC configuration message with PL offset value 1 implicitly (e.g. absent with need M structure). For this scenario which PL offset value should be used.

- Scenario 2: On BWP 1, the UE was configured with PL offset value 1 by the RRC configuration message first and then receive a PL offset MAC CE with PL offset value 2, after that the UE switch to the BWP2 and then switch back to the BWP1. For this scenario, which PL offset value should be used.

For the scenario 1, if the network intends to use the PL offset value 2, the network can include PL offset value 2 in the reconfiguration message, which means the network can always indicate the latest value when reconfigure the UE.

For the scenario 2, the key issue is that when the BWP is deactivated, whether the PL offset value configured by the MAC CE is still valid. For the BWP switch, there is also another case that before switching the UE from the BWP 2 back to the BWP 1, the network may send a new MAC CE to the BWP1 to update the PL offset value. Thus, it should be allowed for the UE to update the PL offset value for a deactivated BWP. In other words, the PL offset value configured by the MAC CE shall be still valid even the BWP is deactivated. Thus for this case, the UE still need to take latest PL offset value no matter it is configured by RRCreconfiguration or MAC CE update.

Proposal 1: The UE should take the latest offset value no matter it is configured by RRCreconfiguration or MAC CE update.

Issue 2:  How many joint/UL TCI states are mapped to a PL offset

On this issue, there are two options:

a.	Option 1: one TCI state (either joint or UL) is indicated per PL offset. 
    b.	Option 2: multiple TCI states (either joint or UL) are indicated per PL offset, considering multiple TCI states can have the same updated PL offset value.
On these 2 options, the option 2 can be seen as the optimization of the option 1. A similar optimization was also discussed in RAN1 for the RRC configurations, e.g. whether to define a list of the PL offset, then an index can be used for each TCI state to indicate which PL offset is used. However, this mechanism was not agreed in RAN1, thus there is also no need to support optimization in the option 2. Though there are some differences between the MAC CE and RRC mechanism, the key issue is the same (i.e. whether it’s necessary to introduce any optimization for the case that multiple TCI states have the same updated PL offset value).

Proposal 2: The mechanism “multiple TCI states (either joint or UL) are indicated per PL offset” is not supported.

Issue 3: BWP ID indication.

According to many legacy MAC CE definition, if unifiedTCI-StateType is configured with joint for the serving cell, DL BWP ID and joint TCI states are included in the MAC CE; if unifiedTCI-StateType is configured with separate for the serving cell, UL BWP ID and UL TCI states are included in the MAC CE. For the PL offset MAC CE, this rule can also be taken for the joint/separate case.

Proposal 3: If unifiedTCI-StateType is configured with joint for the serving cell, DL BWP ID and joint TCI states are included in the MAC CE; if unifiedTCI-StateType is configured with separate for the serving cell, UL BWP ID and UL TCI states are included in the MAC CE.

Issue 4: Applied Channel types

This issue is about the applied channel type, besides the PUCCH/PUSCH/SRS, it has also been agreed in RAN1 that the PL offset would be used for the PDCCH-order PRACH. Thus it’s easy to get the proposal 4.

R1#119 Agreement Support to apply PL offset on PDCCH-order PRACH in FR2 - The design for applying PL offset on PDCCH-order PRACH in FR1 is fully reused here. Note: there is no extra enhancement for Tx beam determination for PDCCH-order PRACH in FR2 in Rel-19

Proposal 4: The TCI states in the MAC CE are applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

Issue 5: The number of updated PL offsets:

About the number of the updated PL offsets, consider that for the separate TCI state configuration case, it should be allowed for the network to configure 16 updated PL offsets in one MAC CE.

Proposal 5: Up to 16 joint/UL TCI states can be indicated in the PL offset MAC CE.

Issue 6: Whether the PL offset update MAC CE is applied to a single serving cell or multiple serving cells

In the R17/18, the serving cell set mechanism has been supported with simultaneousSpatial-UpdatedList1 or simultaneousSpatial-UpdatedList2. Thus this mechanism can also be kept for the PL offset update MAC CE. However if it requires to introduce new simultaneousSpatial-UpdatedList, it would cause some specification complexity. Furthermore, this issue can be taken as lower priority, after we have clear picture on the MAC CE design for the single CC case.

Proposal 6: On “Whether the PL offset update MAC CE is applied to a single serving cell or multiple serving cells”, the single serving cell case can be taken as baseline, the multiple serving cells case can be further discussed with lower priority.

2.2 About 2 TA

On 2 TA, as described in the RAN1 LS [1], many agreements have been achieved.

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped. “RAN2 could check whether this RAN1 agreement would cause any MAC impact”

In RAN2 spec, the restriction on coresetPoolIndex for the 2TA feature is mainly included in the RRC spec,e.g.

tag2 This field to indicate the second TAG information for the serving cell, it is optionally configured in a serving cell if and only if the serving cell is configured with more than one value for the coresetPoolIndex.

Considering that it would not cause too much specification work and this topic is quite separate from the other topics, this can be done in the RAN2 running CR review stage.

Proposal 7: On 2TA with single DCI case, RAN2 would remove the “the restriction that coresetPoolIndex needs to be configured” in the RRC running CR review stage.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: The UE should take the latest offset value no matter it is configured by RRCreconfiguration or MAC CE update.

Proposal 2: The mechanism “multiple TCI states (either joint or UL) are indicated per PL offset” is not supported.

Proposal 3: If unifiedTCI-StateType is configured with joint for the serving cell, DL BWP ID and joint TCI states are included in the MAC CE; if unifiedTCI-StateType is configured with separate for the serving cell, UL BWP ID and UL TCI states are included in the MAC CE.

Proposal 4: The TCI states in the MAC CE are applicable to PUCCH, PUSCH, SRS, and PDCCH-order PRACH.

Proposal 5: Up to 16 joint/UL TCI states can be indicated in the PL offset MAC CE.

Proposal 6: On “Whether the PL offset update MAC CE is applied to a single serving cell or multiple serving cells”, the single serving cell case can be taken as baseline, the multiple serving cells case can be further discussed with lower priority.

Proposal 7: On 2TA with single DCI case, RAN2 would remove the “the restriction that coresetPoolIndex needs to be configured” in the RRC running CR review stage.

- Reference

- R1-2410758  LS to RAN2 on RRC and MAC impacts for Rel-19 NR MIMO Ph5 RAN1

- R2-2410520	Discussion on Asymmetric DL sTRP/UL mTRP	Huawei, HiSilicon

- R2-2409954	RAN2 Impacts for Rel-19 NR MIMO	       Apple

- R2-2410326	Discussion on asymmetric DL sTRP and UL mTRP	CMCC

- R2-2410523	Discussion on Asymmetric DL sTRP/UL mTRP	Samsung