3GPP TSG-RAN WG2 Meeting #129	R2-2501241

Athens, Greece, 17 – 21 February 2025

Agenda Item:		8.3.3

Source:				ETRI

Title:					Discussion on system level simulation

Document for:		Discussion and Decision

1. Introduction

In RAN2#128 meeting, some agreements on the measurement event predictions and for system level simulation were achieved [1]. In this contribution, we discuss some issues on these agreements on measurement event prediction and for system level simulation.

2. Discussion

2.1 Handover model for system level simulation

In RAN2#128 meeting, the following agreements for system level simulation were achieved [1]:

For System level simulation 1 For measurement event prediction for temporal domain case B: UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation. Remove T0 from the picture 2 For measurement event prediction for temporal domain case A, company focus on option 2 or option 3. Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1)) Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT. 3 Handover Preparation time is 40ms and handover execution time: 40ms 4 RAN2 focus on indirect prediction methodology.

After further consideration, we observed some problems with Option 3.

First, it may cause confusion with the following agreements on the definitions of indirect measurement event prediction:

Agreements on event prediction 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. [skipped]

When PW length is shorter than TTT length or TTT is already running based on actual historical measurement results, the operation of the current HO model of Option 3 may be ambiguous as below:

Figure 1. The current HO model of Option 3 (left), when PW length is shorter than TTT length or TTT is already running (right)

Observation 1: When PW length is shorter than TTT length or TTT is already running based on actual historical measurement results, the operation of the current HO model of Option 3 may be ambiguous.

The agreements were made from the rapporteur’s recommendations based on the following question and the company’s responses [2]:

Question 8: Which option of handover model for temporal domain case A do you prefer?

| Company | Option 1/2/3 | Comments |
| --- | --- | --- |
| Ericsson | Out of these options, option 1 | Regarding option 3, the formula should not contain TTT, since it is already taken into account when the event prediction is triggered (at t1). The event prediction is triggered when the cell quality is predicted to be higher than a certain threshold for the TTT time. So, the predicted event is triggered (at t1) TTT ms in advance. |
| Huawei | Option 3 | It seems there may be different interpretations of whether TTT is considered or not. Last meeting it was clarified online that event prediction time indicates the time after TTT has passed. Our point with option 3 is that we should analyze the maximum gains we can get from the predictions, i.e. saving HO preparation time and TTT (under the condition that the entering conditions of the event are met and the prediction indicates they will keep on being met until TTT expiry). |
| vivo | Option 3 | The table 2.4-2 is not aligned with the description for option3? The HO CMD is t3=t0+max(HO prep time, t1-t0-TTT) |
| Samsung | Option 2 | First, AI-based prediction should be additional feature that can be used on top of existing HO framework. Thus, HO based on actual measurement event should be baseline. Also, we think HO command without actual MR (based only on prediction) is risky. If UE’s moving speed or direction change after the prediction, when NW make HO command at the predicted timing, UE would not be able to perform HO successfully. Given that prediction accuracy can not be guaranteed in real field, we prefer to be conservative on HO decision and support option 2. For option 3, we can avoid the risk of early HO a little bit more than option 1 by sending predicted MR after A3 entering condition is first met. However, in that case, we understand there should be two different prediction report from UE (One at t0 for early HO preparation and another one at t1-TTT for early HO command), which seems inefficient and has not been discussed until now. |
| Qualcomm | Please see comments | If network receives an event prediction report, the network need not wait for an actual measurement report before transmitting the HO command to the UE, assuming that the event prediction report is transmitted by the UE if accuracy requirements are met, as mentioned in our response to Q7. |
| Mediatek | Neither of the options Option 3 is acceptable purely for simulation purpose. | First, the network behaviour should follow the current legacy behaviour, where the network sends the HO command immediately upon receiving the MR from the UE, regardless of whether the MR is triggered by prediction or actual measurement. Second, as the event prediction is UE-side model, when to send the MR will be decided by the UE, who takes the TTT into account when sending the MR. Taking the following figure as example: The UE starts the prediction when the entering condition is met, and the prediction is to verify that whether the entering condition can be fulfilled in TTT. If it is the case, the UE can send MR immediately. |
| Xiaomi | Option 2 | The predicted result may be incorrect. In option 1, the incorrect prediction would result in too early handover, which further causes additional HOF. However, in option 2, no additional HOF would be caused by AI, since HO is always triggered by real measurement. The AI would definitely bring gain from reducing HO delay point of view. |
| ZTE | Option 1 |  |
| Nokia | Option 2 | Agree with Samsung. |

The rapporteur made a good comment that “the key issue is how much we can trust the predicted measurement event. Option 2 takes least risk but maybe also least performance. Option3 takes more risk but maybe also most performance. And option 1 is in the middle.” In addition, the comment from Samsung, “For option 3, we can avoid the risk of early HO a little bit more than option 1 by sending predicted MR after A3 entering condition is first met. However, in that case, we understand there should be two different prediction report from UE (One at t0 for early HO preparation and another one at t1-TTT for early HO command), which seems inefficient”, is noteworthy. As shown below, the network can send the HO command only after it receives a report of A3 entering conditions are met based on actual measurement results. It may significantly reduce the risk of early HO, but it brings two predicted or actual measurement reports from the UE, which seems inefficient.

Figure 2. case#1: A3 entering conditions are met (left) and case#2: A3 entering conditions are not met (right)

Observation 2: For Option 3, to reduce the risk of early HO, it brings two predicted or actual measurement reports from the UE, which seems inefficient.

From the agreements, the HO models for measurement event prediction for temporal domain case A and temporal domain case B are different. It can increase the simulation workload and complexity.

Observation 3: The HO models for measurement event prediction for temporal domain case A and temporal domain case B are different. It can increase the simulation workload and complexity.

For Option 3, as a compromise between risk and performance, we have the same view as Mediatek’s responses highlighted in yellow. After the generalization, we can redefine Option 3 as below:

Option 3: The UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation.

Proposal 1: RAN2 to consider a revised HO model of Option 3:

Option 3: The UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation.

The two figures below show that:

(left) when A3 entering conditions are met based on actual measurement (@ ta) and A3 event is predicted at tb based on actual measurements and predicted results (@ t1=ta),

“The UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation.”

(right) when A3 entering conditions are met based on actual measurement (@ ta) and A3 event is predicted at tb based on actual measurements and predicted results (@ t1 later than ta), when PW length is shorter than TTT length or TTT is already running based on actual historical measurement results,

“The UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation.”

Figure 3. revised HO model of Option 3 (left) and when PW length is shorter than TTT length or TTT is already running (right)

Observation 4: The operation of the revised HO model of Option 3 is clear even when PW length is shorter than TTT length or TTT is already running based on actual historical measurement results.

With the revised HO model of Option 3, the network can send the HO command only after it receives a report of predicted A3 event which means that A3 entering conditions are met based on actual measurement results. As discussed above, it may significantly reduce the risk of early HO with only one predicted measurement report from the UE.

Observation 5: The revised HO model of Option 3 can reduce the risk of early HO without additional predicted or actual measurement reports from the UE.

With the revised HO model of Option 3, the HO models for measurement event prediction for temporal domain case A and temporal domain case B are the same. It can reduce the simulation workload and complexity.

Observation 6: With the revised HO model of Option 3, the HO models for measurement event prediction for temporal domain case A and temporal domain case B are the same. It can reduce the simulation workload and complexity.

2.2 Benchmarking with Non-AI

According to the Objective of the study item description [3], it says that “The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]”. Therefore, for system level simulation, the evaluation of the AI performance gain in HO performance KPIs comparing the result with non-AI is one of the most important works of this SI.

Observation 7: For system level simulation, the evaluation of the AI performance gain in HO performance KPIs comparing the result with non-AI is one of the most important works of this SI.

In RAN2#128 meeting, with regard to benchmarking with non-AI for RRM measurement prediction, the following agreements/observations were achieved [1]:

Agreements/observations

- Companies can compare results with non-AI approaches.   Temporal domain, sample and hold, and frequency domain, pathloss offset.   Companies can consider other simple models (e.g. ARIMA).

- For FR1 intra-frequency temporal domain case B, when PW is short, the performance between AI and sample-and-hold is not significant. However, when PW becomes larger, AI outperforms sample-and-hold.

- In frequency domain prediction cases (2GHz and 4GHz), cell-based approaches achieve limited gain compared to pathloss offset without the help of neighbor cell measurement. The cluster-based shows better performance compared to pathloss offset.

- For intra-frequency temporal domain case A, AI can provide gains (in terms of L3 cell RSRP difference) compared to sample and hold. The gain improves with UE speed.

Complying with the agreements, for system level simulation, companies can also compare results with non-AI approaches (e.g., sample and hold) and the legacy algorithm.

Proposal 2: RAN2 to consider that for system level simulation, companies can compare results with non-AI approaches (e.g., sample and hold) and the legacy algorithm.

With the revised HO model of Option 3, comparing results with non-AI approaches and the legacy algorithm can be very straightforward. The figures of the revised HO model and non-AI are the same, but they can be different when PW length is shorter than TTT length or TTT is already running (i.e., Figure 3 (right) for the revised HO model and Figure 4 (top right) for non-AI). For simplicity, non-AI can be simulated using the legacy algorithm with TTT of 0 ms.

Figure 4. revised HO model of Option 3 (top left), non-AI (top right), the legacy algorithm (bottom)

Observation 8: With the revised HO model of Option 3, comparing results with non-AI approaches and the legacy algorithm can be very straightforward. For simplicity, non-AI can be simulated using the legacy algorithm with TTT of 0 ms.

3. Conclusion

Observation 1: When PW length is shorter than TTT length or TTT is already running based on actual historical measurement results, the operation of the current HO model of Option 3 may be ambiguous.

Observation 2: For Option 3, to reduce the risk of early HO, it brings two predicted or actual measurement reports from the UE, which seems inefficient.

Observation 3: The HO models for measurement event prediction for temporal domain case A and temporal domain case B are different. It can increase the simulation workload and complexity.

Observation 4: The operation of the revised HO model of Option 3 is clear even when PW length is shorter than TTT length or TTT is already running based on actual historical measurement results.

Observation 5: The revised HO model of Option 3 can reduce the risk of early HO without additional predicted or actual measurement reports from the UE.

Observation 6: With the revised HO model of Option 3, the HO models for measurement event prediction for temporal domain case A and temporal domain case B are the same. It can reduce the simulation workload and complexity.

Observation 7: For system level simulation, the evaluation of the AI performance gain in HO performance KPIs comparing the result with non-AI is one of the most important works of this SI.

Observation 8: With the revised HO model of Option 3, comparing results with non-AI approaches and the legacy algorithm can be very straightforward. For simplicity, non-AI can be simulated using the legacy algorithm with TTT of 0 ms.

Based on the discussion in Section 2, we propose the following:

Proposal 1: RAN2 to consider a revised HO model of Option 3:

Option 3: The UE reports when A3 event is satisfied with actual measurements and predicted results. And handover command will be received after handover preparation.

Proposal 2: RAN2 to consider that for system level simulation, companies can compare results with non-AI approaches (e.g., sample and hold) and the legacy algorithm.

4. References

RAN2#128 Meeting Report

R2-2411175, Summary of [AT128][017][AI mob]Simulation assumptions, OPPO

RP-240082	Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR	OPPO