3GPP TSG-RAN WG2 Meeting #129	R2-2501277

Athens, Greece, 17th – 21st February, 2025

Agenda item:	8.17

Source:	Samsung

Title:	Initial aspects of IoT NTN TDD

WID/SID:	IoT_NTN_TDD

Document for:	Discussion and Decision

# Introduction

In RAN#106 the WID for IoT NTN TDD was agreed [1]:

This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

In this contribution, we provide some initial discussion on IoT NTN TDD.

# Discussion

- The key aspect of the work item is to adapt NB-IoT NTN to a special TDD band, which can be seen in Figure 1. The UL and DL frames are 8.28 ms and the simplex frame is 20.32 ms and in total it comes to 90 ms.

Figure 1.

- The challenging aspects of this specific frame is that the periodicity of the TDD bands has a periodicity that is odd to 3GPP specifications, and that each UL or DL frame being 8.28 ms makes allocating the subframes difficult. Another challenging aspect has to do with the fact that legacy services prevent all of the UL and DL frames to be available for NB-IoT. This means that very few subframes are available, potentially interrupting or delaying procedures. Due to the existing services, it was agreed in the WID that only a single UL and DL frame with 8 subframes each, are available every 90 ms.

Observation 1: One challenging aspect has to do with the fact that very few subframes are available regularly, potentially interrupting or delaying procedures.

- Some issues that we have identified early on are the following:

- Impacts on random access procedures. For instance how the random access response window shall operate when there is a large gap between the UL frame and DL frame.

- Impacts on paging. For the paging procedures, Paging Occasion are defined per SFN, which for FDD LTE assumes that most of the SFNs are available to be used.

- A lot of these may still require RAN1 decisions, such as decision on the downlink subframes and any general way to handle gaps between different frames. RAN1 would also need to indicate how SFNs are counted.

Proposal 1: RAN2 to address:

- Impacts on random access procedures,

- Impacts on paging procedures.

# Conclusion

In this contribution, we provided some initial discussions on IoT NTN TDD:

Observation 1: One challenging aspect has to do with the fact that very few subframes are available regularly, potentially interrupting or delaying procedures.

Proposal 1: RAN2 to address:

- Impacts on random access procedures,

- Impacts on paging procedures.

# Reference

RP-243293, Revised WID for introduction of IoT NTN TDD mode, Iridium Satellite LLC, RAN#106, Madrid, Spain, December 2024.