3GPP TSG RAN WG2 Meeting #129

Athens, Greece, Feb. 17th – 21st, 2025         	                	                          R2-2501278

Agenda Item:	8.1.4

Source:	Rakuten Mobile

Title:	Discussion on UE Side Data collection

Document for:	Discussion and Decision

# Introduction

Based on RAN#106 Guidance we would following issues in this contribution.

- Study RAN aspects related to data transfer over UP - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. - Discuss NG-RAN involvement in the data transfer of UE side data collection(including visibility discussion). - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. - Discuss on the scalability aspects of CP. - RAN2 can Liaise with other WGs if/when needed.

# Discussion

## AI/ML Data Transfer Over UP: Network Adaptability and Traffic Optimization

AI/ML data transfer over the User Plane (UP) introduces an entirely different set of challenges compared to traditional broadband services. AI/ML traffic patterns are bursty, workload-dependent, and highly asynchronous, meaning they cannot be managed using conventional QoS scheduling mechanisms. Unlike voice or video services that maintain a continuous flow of packets, AI/ML transmissions are often triggered by event-driven processes such as model retraining, network anomaly detection, or inference updates, resulting in unpredictable traffic spikes.

A critical concern raised in plenary discussions is that AI/ML data flows could saturate uplink resources, particularly in scenarios where multiple UEs upload large volumes of training data simultaneously. While UP allows for scalable AI/ML data transfer, it lacks mechanisms to prioritize and regulate transmission dynamically at the RAN level. If AI/ML traffic is not managed properly, it could interfere with more latency-sensitive traffic such as VoNR, real-time gaming, or even essential control-plane signaling.

A possible approach to mitigating this is introducing AI/ML-specific dynamic scheduling, where NG-RAN can identify and allocate resources for AI/ML workloads based on network conditions. This would require defining AI/ML traffic classes distinct from traditional GBR/non-GBR services, ensuring AI/ML data does not degrade QoS for other traffic types. However, scheduling AI/ML traffic in real time at NG-RAN requires a careful balance between minimizing control-plane overhead and maintaining flexibility for UE-initiated transmissions.

Another open question is how to preemptively manage AI/ML data bursts rather than react to congestion once it occurs. Predictive scheduling, based on historical AI/ML traffic loads and workload anticipation, could allow NG-RAN to allocate resources before congestion occurs rather than after. However, this requires additional intelligence at the scheduler level, which may not be straightforward to implement within existing RAN architectures.

- AI/ML traffic in UP is inherently unpredictable and does not align with standard 5QI-based scheduling models, requiring new traffic management strategies at NG-RAN.

- Unregulated AI/ML data transfers risk congesting uplink resources, interfering with essential QoS flows such as VoNR .

- Recommend SA2 to Define an AI/ML-specific uplink scheduling framework at NG-RAN, enabling dynamic allocation of AI/ML data resources based on network load.

- Introduce an RRC-based AI/ML congestion mitigation mechanism, allowing NG-RAN to instruct UEs to delay or reschedule AI/ML data uploads when congestion thresholds are exceeded.

## Handling AI/ML Data Aggregation and Compression for UP Transfer

#### One of the inefficiencies we see with AI/ML data transfer today is redundancy in reporting. AI/ML data is not like standard application data—it is often repetitive, contains large volumes of correlated measurements, and in many cases, can be compressed before transmission without loss of usefulness. However, current UPF and RAN mechanisms do not account for this, meaning a large portion of AI/ML data is likely consuming more bandwidth than necessary.

#### From an operator’s perspective, this is an unnecessary burden on the air interface. If AI/ML data could be aggregated and compressed before being sent over UP, we could significantly reduce bandwidth consumption without affecting AI/ML model performance. One possible approach is to allow NG-RAN to configure AI/ML aggregation policies via RRC, where UEs batch AI/ML data before transmission instead of sending individual reports. This would be similar to how we handle periodic CSI reporting, except optimized for AI/ML workloads.

#### Another area worth exploring is AI/ML-specific header compression at the PDCP layer. Unlike conventional application data, AI/ML data often includes metadata that doesn’t need to be fully transmitted every time. If we can strip redundant metadata while maintaining integrity, this could further reduce transmission overhead.

#### Additionally, we should consider filtering redundant AI/ML reports before they reach the UPF. Often, AI/ML models generate highly correlated measurements over time, meaning the network is processing multiple versions of the same data unnecessarily. A mechanism where NG-RAN or UPF can intelligently discard redundant AI/ML reports before transmission could significantly optimize the efficiency of AI/ML data transfers.

- AI/ML model updates frequently contain redundant information, leading to inefficient uplink resource utilization.

- There is no defined mechanism in RAN specifications for aggregating AI/ML data at the UE level before transmission.

- Existing UP mechanisms do not support AI/ML-specific compression or filtering, requiring new optimizations at PDCP or SDAP.

- Introduce RRC-based AI/ML data aggregation parameters, enabling UEs to batch model updates before transmission.

- Define AI/ML-aware header compression at PDCP, allowing redundant metadata to be removed from AI/ML data packets before UP transmission.

- Develop NG-RAN filtering mechanisms to suppress redundant AI/ML model updates, ensuring only meaningful changes are transmitted.

## AI/ML Data Transmission Timing and Synchronization Over UP

Unlike conventional user traffic, AI/ML data collection and transmission occur asynchronously, often dependent on external triggers such as model retraining events, network anomaly detection, or inference feedback requirements. This introduces a new challenge for RAN-level synchronization, as AI/ML data transmission timing must align with network conditions while avoiding congestion.

One possible approach is introducing AI/ML transmission timing control at NG-RAN, where AI/ML data uploads are scheduled in predefined network-adaptive time windows. This would allow RAN to control the timing of AI/ML transmissions, ensuring that AI/ML data uploads occur when network utilization is low.

Additionally, NG-RAN could implement AI/ML-aware timing synchronization policies, allowing coordination between UEs to prevent simultaneous AI/ML uploads that could congest the network. A potential implementation could involve AI/ML timing request signaling, where UEs request transmission slots based on model retraining schedules, allowing NG-RAN to grant or defer transmission windows dynamically.

- AI/ML data transfers are asynchronous, requiring network-adaptive transmission timing controls to optimize UP-based scheduling and reduce network congestion.

- Implement AI/ML transmission timing control at NG-RAN, where AI/ML uploads occur in predefined, congestion-aware time windows.

- Introduce AI/ML-aware synchronization signaling, allowing UEs to request transmission slots for AI/ML data uploads based on network load.

- Investigate whether AI/ML data transmission should follow a scheduled periodic model or a network-initiated adaptive timing approach.

## AI/ML Data Differentiation for UP Traffic Prioritization

AI/ML data is treated as generic UP traffic, meaning it competes for resources without explicit prioritization mechanisms. This presents two problems: first, real-time AI/ML inference updates require lower latency but are not prioritized over background AI/ML model training data. Second, AI/ML training data uploads could be deprioritized by existing QoS mechanisms, leading to excessive delays in scenarios where rapid model updates are required for adaptive network optimizations.

One potential solution is allowing NG-RAN to assign AI/ML service differentiation tags at RRC level, ensuring that real-time AI/ML traffic can be prioritized dynamically based on network state. Another approach is defining dynamic AI/ML traffic reclassification, where NG-RAN can upgrade, or downgrade AI/ML data priority based on real-time network conditions.

- AI/ML data competes with real-time services, leading to potential QoS conflicts and inefficiencies.

- There is no defined mechanism in current 5G QoS frameworks to differentiate between high-priority and low-priority AI/ML traffic.

- Implement AI/ML transmission timing control at NG-RAN, where AI/ML uploads occur in predefined, congestion-aware time windows.

- Introduce AI/ML-aware synchronization signaling, allowing UEs to request transmission slots for AI/ML data uploads based on network load.

- Investigate whether AI/ML data transmission should follow a scheduled periodic model or a network-initiated adaptive timing approach.

## NG-RAN’s Role in the Control and Configuration of UE-Side Data Collection

The discussion around NG-RAN’s role in AI/ML data collection continues to be one of the more debated aspects in RAN2. While it is generally accepted that NG-RAN should configure measurement parameters, the extent of its role in initiating, regulating, and terminating data collection remains unclear. The AI/ML data collection process today typically follows either UE autonomy, where UEs decide when to collect and report data, or network-controlled scheduling via CN or OAM, where higher-layer functions define when data collection should occur. The question that remains unanswered is: Should NG-RAN have a more direct role in managing when and how AI/ML data is collected at the UE?

From an operator’s perspective, it makes sense for NG-RAN to have some degree of control, especially considering that AI/ML data collection can impact uplink scheduling, resource allocation, and network congestion. Unlike traditional measurement reports, AI/ML data is bulkier, periodic, and often generated at unpredictable times based on application-layer triggers. This means that uncontrolled AI/ML data collection could overload network resources, particularly in cells with high traffic loads or where critical services like VoNR require low latency and predictable resource allocation.

A straightforward approach to addressing this issue would be to allow NG-RAN to introduce AI/ML-aware measurement control parameters in RRC signaling, ensuring that AI/ML data collection aligns with network resource availability. However, if NG-RAN takes full control of AI/ML data collection scheduling, this could introduce excessive control-plane signaling overhead, particularly in large networks where thousands of UEs might need collection adjustments.

An alternative approach is to allow NG-RAN to act as a policy enforcer rather than an initiator, meaning it configures AI/ML data collection rules but leaves scheduling and initiation to CN/OAM. This would allow AI/ML data collection to align with higher-layer network policies while ensuring AI/ML collection does not overload RAN resources.

A hybrid model could also be considered, where NG-RAN provides dynamic collection recommendations, but UEs retain flexibility in determining when data is collected within those constraints. This could help balance network efficiency without requiring frequent RRC reconfiguration updates. However, feasibility remains an issue, as such a model would require new signaling interactions that do not currently exist in 5G RRC frameworks.

- NG-RAN currently has no defined role in actively managing AI/ML data collection timing, leading to potential network congestion risks.

- If NG-RAN takes full control, it may introduce excessive RRC signaling overhead, particularly in high-density networks.

- A hybrid approach, where NG-RAN provides collection recommendations while UEs maintain some autonomy, may be a feasible alternative but requires further study.

- Introduce AI/ML-specific measurement control parameters in RRC signaling, allowing NG-RAN to regulate when and how frequently UEs collect AI/ML data.

- Enable NG-RAN to act as a policy enforcer rather than a scheduler, defining AI/ML data collection rules but leaving scheduling to CN/OAM.

- Evaluate a hybrid NG-RAN-driven collection model where UEs can adjust collection timing within NG-RAN-defined constraints to balance network control and flexibility.

## NG-RAN’s Role in AI/ML Data Transfer and Visibility

One of the most critical challenges with AI/ML data transfer over UP is that NG-RAN has little to no visibility into these transfers. Unlike Control Plane (CP) transfers, where RAN can monitor and enforce policy, UP-based AI/ML transfers bypass traditional RAN monitoring mechanisms, meaning AI/ML data is uploaded without explicit network awareness. From an operator’s standpoint, this is a significant problem, as it means AI/ML traffic cannot be prioritized, shaped, or scheduled dynamically.

One of the proposed solutions is metadata tagging, where UEs insert network-recognizable identifiers into AI/ML data packets, allowing NG-RAN or UPF to differentiate AI/ML traffic from other UP flows. However, the technical feasibility of this approach remains uncertain. Unlike CP-based services, where metadata is typically embedded in NAS/RRC messages, UP packets are not processed directly by NG-RAN, making it unclear how NG-RAN would use metadata tagging in real time. This raises a fundamental question about where AI/ML metadata should be introduced—should it be at the PDCP, SDAP, or UPF level? Each option has different trade-offs in terms of network complexity, processing overhead, and compatibility with existing RAN functions.

Another challenge is that AI/ML traffic is unpredictable and may cause uplink congestion when multiple UEs attempt to transfer large datasets at the same time. If NG-RAN has no mechanism to regulate AI/ML data flows, there is no way to ensure that essential traffic is not negatively impacted. A potential solution could be to allow NG-RAN to issue congestion indicators dynamically, instructing UEs to adjust their AI/ML data transfer rates based on real-time network load conditions. However, this raises another issue—how frequently should NG-RAN send congestion updates? Excessive congestion signaling could introduce unnecessary overhead, while infrequent updates might not provide enough control to prevent congestion events before they happen.

Another possible solution is periodic AI/ML data transfer reporting, where UEs provide NG-RAN with status updates about AI/ML data uploads, allowing NG-RAN to track trends and apply preemptive congestion management techniques. However, such an approach would require defining new RRC reporting messages, as current RAN mechanisms do not support AI/ML-specific transfer status reporting.

- UP-based AI/ML data transfers lack NG-RAN visibility, making it difficult for operators to enforce policies and manage congestion.

- While metadata tagging could help classify AI/ML traffic, NG-RAN does not process UP packets, making it unclear where metadata tagging should be applied.

- Dynamic congestion control for AI/ML data remains undefined, requiring a balance between network responsiveness and signaling overhead.

- Introduce AI/ML metadata tagging at the SDAP layer, allowing NG-RAN to identify AI/ML data flows without modifying UP packet handling.

- Enable NG-RAN to issue congestion indicators dynamically, instructing UEs to delay or throttle AI/ML data uploads based on network conditions.

- Develop an RRC-based AI/ML transfer reporting mechanism, allowing UEs to provide periodic updates to NG-RAN about AI/ML data upload status.

## AI/ML Data Transfer to CN or OAM

One of the key challenges in AI/ML data transfer is determining how AI/ML data should be classified and routed within the network. While User Plane Function (UPF) currently handles traffic classification based on service types such as IMS voice, video streaming, and best-effort browsing, there is no standardized mechanism to differentiate AI/ML data from regular user traffic. This creates ambiguity in whether AI/ML data should be treated as application-layer content (requiring aggregation in CN) or network-optimization telemetry (better suited for OAM processing).

From an operator’s perspective, the lack of clear routing policies for AI/ML data is a significant issue, as it can lead to inefficient handling of inference updates and training model transfers. AI/ML workloads vary—some datasets are critical for real-time network optimization (e.g., beam prediction updates), while others are long-term model training updates that do not require immediate processing. If all AI/ML data is treated the same way, networks risk either introducing delays in critical inference operations or over-prioritizing low-priority model updates that could have been scheduled more efficiently.

A possible solution is defining AI/ML-specific traffic classification at the RAN level, enabling NG-RAN to pre-classify AI/ML data before it reaches UPF, rather than expecting UPF alone to handle differentiation. This would allow AI/ML data flows to be categorized based on their urgency, processing destination (CN vs. OAM), and impact on real-time network functions. Additionally, there is a need to standardize AI/ML data formatting indicators, ensuring that CN and OAM functions can efficiently parse and process AI/ML datasets without unnecessary conversions or delays.

Another aspect that requires further study is whether AI/ML routing policies should be static or adaptive. A static routing model would define fixed rules on whether a particular AI/ML dataset should be sent to CN or OAM, ensuring consistency across deployments. In contrast, an adaptive model could allow dynamic rerouting of AI/ML data based on network congestion, available processing capacity, or other real-time factors. While adaptive routing introduces flexibility, it also requires additional network intelligence and coordination between RAN and core functions.

- There is no standardized traffic classification mechanism for AI/ML data at RAN or UPF, leading to inefficient handling of AI/ML model updates and inference data.

- Current transport formats do not account for AI/ML-specific processing needs, resulting in potential delays in model updates and inference optimizations.

- Unclear whether AI/ML routing should follow predefined (static) policies or adaptive mechanisms based on network conditions.

- RAN2 recommend SA2 to Introduce AI/ML traffic classification parameters at NG-RAN, allowing pre-classification of AI/ML data flows before reaching UPF.

- Define AI/ML-specific data formatting indicators to enable efficient AI/ML processing at CN and OAM without requiring unnecessary format conversion.

- Investigate adaptive AI/ML routing mechanisms that allow real-time adjustment of AI/ML data flows based on network congestion and processing resource availability.

## NG-RAN’s Role in AI/ML Data Transfer and Visibility

The lack of visibility into AI/ML data transfers over the User Plane (UP) has been a recurring concern in RAN2 discussions. Unlike Control Plane (CP) transfers, which allow the network to track session parameters and enforce policies, UP-based AI/ML data flows bypass traditional RAN supervision, making it difficult for operators to monitor, prioritize, or manage AI/ML traffic dynamically. This lack of oversight could lead to unregulated AI/ML data bursts, uplink congestion, and an inability to enforce priority-based scheduling mechanisms.

A key issue is that AI/ML data transfers occur without explicit signaling interactions with NG-RAN, meaning that RAN has no way of knowing when an AI/ML dataset is being uploaded unless specific mechanisms are introduced. One potential solution is AI/ML traffic tagging at the UE level, allowing UEs to embed network-recognizable identifiers in AI/ML data packets, enabling NG-RAN or UPF to differentiate AI/ML traffic from standard user traffic. However, there are technical feasibility concerns—NG-RAN does not process UP packets directly, so AI/ML metadata tagging would need to be introduced at an appropriate layer (PDCP, SDAP, or UPF) without excessive overhead.

Another challenge is that AI/ML workloads generate unpredictable traffic bursts, leading to network congestion in uplink-heavy scenarios. While NG-RAN today has mechanisms to manage congestion for traditional uplink services, there is no defined mechanism to regulate AI/ML traffic dynamically based on real-time conditions. A possible enhancement is allowing NG-RAN to issue congestion indicators to UEs, signaling them to adjust AI/ML data transfer rates or delay non-urgent uploads. This would enable dynamic rate control based on network load conditions. However, congestion signaling should be used sparingly, as excessive signaling could introduce unnecessary control-plane overhead, especially in dense network deployments.

Additionally, an AI/ML transfer status reporting mechanism could allow UEs to periodically update NG-RAN on the status of ongoing AI/ML data transfers. This would provide NG-RAN with better forecasting capabilities, ensuring that AI/ML data bursts are managed more proactively instead of reactively. However, such a mechanism must avoid excessive RRC signaling and should be designed in a way that balances network awareness with efficiency.

- UP-based AI/ML data transfers currently lack NG-RAN visibility, making it difficult to enforce policies, manage congestion, or apply priority-based traffic handling.

- Embedding AI/ML traffic metadata at the UE level could improve network awareness but requires careful consideration of where (PDCP, SDAP, UPF) to introduce tagging.

- NG-RAN has no mechanism to dynamically regulate AI/ML data transfer rates based on congestion conditions, requiring new signaling solutions.

- Introduce AI/ML metadata tagging at SDAP to allow NG-RAN to differentiate AI/ML data from regular UP traffic without modifying packet headers.

- Enable NG-RAN to issue congestion indicators dynamically, instructing UEs to adjust AI/ML data transmission rates based on real-time network load.

- Develop an RRC-based AI/ML transfer status reporting mechanism, allowing UEs to periodically update NG-RAN on AI/ML data upload progress to optimize scheduling.

# Conclusion

In summary, we make the following proposals for UE Side Data collection:

AI/ML Data Transfer Over UP: Network Adaptability and Traffic Optimization

- AI/ML traffic in UP is inherently unpredictable and does not align with standard 5QI-based scheduling models, requiring new traffic management strategies at NG-RAN.

- Unregulated AI/ML data transfers risk congesting uplink resources, interfering with essential QoS flows such as VoNR

- Recommend SA2 to Define an AI/ML-specific uplink scheduling framework at NG-RAN, enabling dynamic allocation of AI/ML data resources based on network load.

- Introduce an RRC-based AI/ML congestion mitigation mechanism, allowing NG-RAN to instruct UEs to delay or reschedule AI/ML data uploads when congestion thresholds are exceeded.

Handling AI/ML Data Aggregation and Compression for UP Transfer

- AI/ML model updates frequently contain redundant information, leading to inefficient uplink resource utilization.

- There is no defined mechanism in RAN specifications for aggregating AI/ML data at the UE level before transmission.

- Existing UP mechanisms do not support AI/ML-specific compression or filtering, requiring new optimizations at PDCP or SDAP.

- Introduce RRC-based AI/ML data aggregation parameters, enabling UEs to batch model updates before transmission.

- Define AI/ML-aware header compression at PDCP, allowing redundant metadata to be removed from AI/ML data packets before UP transmission.

- Develop NG-RAN filtering mechanisms to suppress redundant AI/ML model updates, ensuring only meaningful changes are transmitted.

AI/ML Data Transmission Timing and Synchronization Over UP

- AI/ML data transfers are asynchronous, requiring network-adaptive transmission timing controls to optimize UP-based scheduling and reduce network congestion.

- Implement AI/ML transmission timing control at NG-RAN, where AI/ML uploads occur in predefined, congestion-aware time windows.

- Introduce AI/ML-aware synchronization signaling, allowing UEs to request transmission slots for AI/ML data uploads based on network load.

- Investigate whether AI/ML data transmission should follow a scheduled periodic model or a network-initiated adaptive timing approach.

AI/ML Data Differentiation for UP Traffic Prioritization

- AI/ML data competes with real-time services, leading to potential QoS conflicts and inefficiencies.

- There is no defined mechanism in current 5G QoS frameworks to differentiate between high-priority and low-priority AI/ML traffic.

- Implement AI/ML transmission timing control at NG-RAN, where AI/ML uploads occur in predefined, congestion-aware time windows.

- Introduce AI/ML-aware synchronization signaling, allowing UEs to request transmission slots for AI/ML data uploads based on network load.

- Investigate whether AI/ML data transmission should follow a scheduled periodic model or a network-initiated adaptive timing approach.

NG-RAN’s Role in the Control and Configuration of UE-Side Data Collection

- NG-RAN currently has no defined role in actively managing AI/ML data collection timing, leading to potential network congestion risks.

- If NG-RAN takes full control, it may introduce excessive RRC signaling overhead, particularly in high-density networks.

- A hybrid approach, where NG-RAN provides collection recommendations while UEs maintain some autonomy, may be a feasible alternative but requires further study.

- Introduce AI/ML-specific measurement control parameters in RRC signaling, allowing NG-RAN to regulate when and how frequently UEs collect AI/ML data.

- Enable NG-RAN to act as a policy enforcer rather than a scheduler, defining AI/ML data collection rules but leaving scheduling to CN/OAM.

- Evaluate a hybrid NG-RAN-driven collection model where UEs can adjust collection timing within NG-RAN-defined constraints to balance network control and flexibility.

NG-RAN’s Role in AI/ML Data Transfer and Visibility

- UP-based AI/ML data transfers lack NG-RAN visibility, making it difficult for operators to enforce policies and manage congestion.

- While metadata tagging could help classify AI/ML traffic, NG-RAN does not process UP packets, making it unclear where metadata tagging should be applied.

- Dynamic congestion control for AI/ML data remains undefined, requiring a balance between network responsiveness and signaling overhead.

- Introduce AI/ML metadata tagging at the SDAP layer, allowing NG-RAN to identify AI/ML data flows without modifying UP packet handling.

- Enable NG-RAN to issue congestion indicators dynamically, instructing UEs to delay or throttle AI/ML data uploads based on network conditions.

- Develop an RRC-based AI/ML transfer reporting mechanism, allowing UEs to provide periodic updates to NG-RAN about AI/ML data upload status.

AI/ML Data Transfer to CN or OAM

- There is no standardized traffic classification mechanism for AI/ML data at RAN or UPF, leading to inefficient handling of AI/ML model updates and inference data.

- Current transport formats do not account for AI/ML-specific processing needs, resulting in potential delays in model updates and inference optimizations.

- Unclear whether AI/ML routing should follow predefined (static) policies or adaptive mechanisms based on network conditions.

- RAN2 to recommend SA2 to Introduce AI/ML traffic classification parameters at NG-RAN, allowing pre-classification of AI/ML data flows before reaching UPF.

- Define AI/ML-specific data formatting indicators to enable efficient AI/ML processing at CN and OAM without requiring unnecessary format conversion.

- Investigate adaptive AI/ML routing mechanisms that allow real-time adjustment of AI/ML data flows based on network congestion and processing resource availability.

NG-RAN’s Role in AI/ML Data Transfer and Visibility

- UP-based AI/ML data transfers currently lack NG-RAN visibility, making it difficult to enforce policies, manage congestion, or apply priority-based traffic handling.

- Embedding AI/ML traffic metadata at the UE level could improve network awareness but requires careful consideration of where (PDCP, SDAP, UPF) to introduce tagging.

- NG-RAN has no mechanism to dynamically regulate AI/ML data transfer rates based on congestion conditions, requiring new signaling solutions.

- Introduce AI/ML metadata tagging at SDAP to allow NG-RAN to differentiate AI/ML data from regular UP traffic without modifying packet headers.

- Enable NG-RAN to issue congestion indicators dynamically, instructing UEs to adjust AI/ML data transmission rates based on real-time network load.

- Develop an RRC-based AI/ML transfer status reporting mechanism, allowing UEs to periodically update NG-RAN on AI/ML data upload progress to optimize scheduling.

# Reference

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023

- RP-243307, “Way forward on AIML UE sided data collection,” Dec 2024

- RAN2#128 meeting report

- R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1