3GPP TSG-RAN WG2 Meeting #129	R2-2501300

# Athens, Greece, Feb. 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5273 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on number of Rx branches supported by a UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core, NR_redcap_enh-Core, NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | numberOfRxERedCap, numberOfRxRedCap and supportOf2RxXR fields in the UERadioPagingInformation message indicates the number of Rx branches supported by an UE although (e)RedCap and XR UE support for number of Rx branches is band specific. The (e)RedCap and XR UE support for number of Rx branches may be same or different on different bands supported by the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced band specific supported number of Rx branches fields for in the UERadioPagingInformation message. Impact analysis Impacted functionality: Paging of (e)RedCap and XR device. Inter-operability: - No inter-operability issues are foreseen since the CR affects only Radio Access Network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect number of Rx branches may be assumed in the paging for the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 11.2.2	Message definitions

*** Text skipped ***

#### –	UERadioPagingInformation

This message is used to transfer radio paging information, covering both upload to and download from the 5GC, and between gNBs.

Direction: gNB to/ from 5GC and gNB to/from gNB

UERadioPagingInformation message

-- ASN1START

-- TAG-UE-RADIO-PAGING-INFORMATION-START

UERadioPagingInformation ::= SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

ueRadioPagingInformation            UERadioPagingInformation-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

UERadioPagingInformation-IEs ::=    SEQUENCE {

supportedBandListNRForPaging        SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR    OPTIONAL,

nonCriticalExtension                UERadioPagingInformation-v15e0-IEs                      OPTIONAL

}

UERadioPagingInformation-v15e0-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1     ENUMERATED {supported}          OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2     ENUMERATED {supported}          OPTIONAL,

nonCriticalExtension                UERadioPagingInformation-v1700-IEs          OPTIONAL

}

UERadioPagingInformation-v1700-IEs ::= SEQUENCE {

ue-RadioPagingInfo-r17                 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17)     OPTIONAL,

inactiveStatePO-Determination-r17      ENUMERATED {supported}                               OPTIONAL,

numberOfRxRedCap-r17                   ENUMERATED {one, two}                                OPTIONAL,

halfDuplexFDD-TypeA-RedCap-r17         SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL,

nonCriticalExtension                   UERadioPagingInformation-v1800-IEs                   OPTIONAL

}

UERadioPagingInformation-v1800-IEs ::= SEQUENCE {

numberOfRxERedCap-r18                  ENUMERATED {one, two}                                OPTIONAL,

supportOf2RxXR-r18                     ENUMERATED {supported}                               OPTIONAL,

nonCriticalExtension                   UERadioPagingInformation-v1840-IEs                   OPTIONAL

}

UERadioPagingInformation-v1840-IEs ::= SEQUENCE {

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18    ENUMERATED {supported}                   OPTIONAL,

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18    ENUMERATED {supported}                   OPTIONAL,

nonCriticalExtension                               UERadioPagingInformation-v18xx-IEs {}                              OPTIONAL

}

UERadioPagingInformation-v18xx-IEs ::=    SEQUENCE {

numberOfRxRedCapPerBand-r18             SEQUENCE (SIZE (1..maxBands)) OF NumberOfRxRedCap-r18		OPTIONAL,

numberOfRxERedCapPerBand-r18            SEQUENCE (SIZE (1..maxBands)) OF NumberOfRxRedCap-r18		OPTIONAL,

supportOf2RxXRPerBand-r18	            SEQUENCE (SIZE (1..maxBands)) OF NumberOfRxXR-r18 		   OPTIONAL,

nonCriticalExtension                	UERadioPagingInformation-v18xx-IEs                     	OPTIONAL

}

NumberOfRxRedCap-r18 ::= ENUMERATED {one, two, spare}

NumberOfRxXR-r18	 ::= ENUMERATED {two, moreThanTwo, spare}

-- TAG-UE-RADIO-PAGING-INFORMATION-STOP

-- ASN1STOP

UERadioPagingInformation field descriptions

supportedBandListNRForPaging Indicates the UE supported NR frequency bands which are derived by the gNB from UE-NR-Capability.

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A in FDD FR1.

dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A in FDD FR2-NTN.

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A in TDD FR1.

dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A in TDD FR2.

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B in FDD FR1.

dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B in FDD FR2-NTN.

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B in TDD FR1.

dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2 Indicates whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type B in TDD FR2.

halfDuplexFDD-TypeA-RedCap Indicates whether the (e)RedCap UE only supports half-duplex operation for FDD in the indicated band(s).

inactiveStatePO-Determination Indicates whether the UE supports to use the same i_s to determine PO in RRC_INACTIVE state as in RRC_IDLE state.

numberOfRxERedCap Indicates the number of Rx branches supported by an eRedCap UE. The network ignores this field in case numberOfRxERedCapPerBand is included.

numberOfRxRedCap Indicates the number of Rx branches supported by a RedCap UE. The network ignores this field in case numberOfRxRedCapPerBand is included.

supportOf2RxXR Indicates whether the UE is a 2Rx XR UE. The network ignores this field in case supportOf2RxXRPerBand is included.

numberOfRxERedCapPerBand Indicates the number of Rx branches supported by an eRedCap UE for each band which are derived by the gNB from maxNumberMIMO-LayersPDSCH from UE-NR-Capability. When this field is included, the network does not include numberOfRxERedCap unless Rx capabilities are the same across all bands. This field shall contain the same number of entries, listed in the same order as in supportedBandListNRForPaging. The first element in the list corresponds to the first element in supportedBandListNRForPaging. The second element in the list corresponds to the second element in supportedBandListNRForPaging, and so on.

numberOfRxRedCapPerBand Indicates the number of Rx branches supported by a RedCap UE for each band which are derived by the gNB from maxNumberMIMO-LayersPDSCH from UE-NR-Capability. When this field is included, the network does not include numberOfRxRedCap unless Rx capabilities are the same across all bands. This field shall contain the same number of entries, listed in the same order as in supportedBandListNRForPaging. The first element in the list corresponds to the first element in supportedBandListNRForPaging. The second element in the list corresponds to the second element in supportedBandListNRForPaging, and so on.

supportOf2RxXRPerBand Indicates the number of Rx branches supported by a XR UE for each band which are derived by the gNB from maxNumberMIMO-LayersPDSCH from UE-NR-Capability. When this field is included, the network does not include supportOf2RxXR unless Rx capabilities are the same across all bands. This field shall contain the same number of entries, listed in the same order as in supportedBandListNRForPaging. The first element in the list corresponds to the first element in supportedBandListNRForPaging. The second element in the list corresponds to the second element in supportedBandListNRForPaging, and so on.

ue-RadioPagingInfo The field is used to transfer UE capability information used for paging. The gNB generates the ue-RadioPagingInfo and the contained UE capability information is absent when not supported by the UE.

End of changes