3GPP TSG-RAN WG2 Meeting #129	R2-2501305

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.8.6

Source:	Ericsson

Title:	E-UTRAN TN to NR-NTN mobility

Document for:	Discussion

# 1	Introduction

RAN plenary #103 approved a work item [1] on mobility between LTE (EUTRAN) terrestrial networks (TN) and NR non-terrestrial networks (NTN). The work item aims at specifying support for the following objectives:

- RRC_IDLE mode mobility based on cell reselection from E-UTRA TN to NR NTN, where E-UTRA TN provides satellite information for NR NTN neighbour cells in a System Information Block. [RAN2]

In this contribution, we discuss an open issue from last meeting.

# 2	Discussion

### 2.1	RRC Release with redirection

One unresolved issue is determining whether RRC_IDLE mode mobility using RRC Release with redirection from LTE TN to an NR NTN carrier frequency is supported. After extensive discussion during RAN2#128, the following agreement was reached:

RAN2 understands that the UE behaviour is currently not clear if the network includes a NR NTN frequency in redirectedCarrierInfo in the RRCConnectionRelease message in a TN cell. We can further discuss whether we want to clarify the UE behaviour and in case whether we need to introduce a new UE capability.

We understand that a UE may support both LTE TN and NR NTN since the introduction of NTN in Release 17. However, RRC_IDLE mode mobility between LTE TN and NR NTN is considered a new feature introduced in Release 19. This feature requires LTE TN UEs to interpret new signaling to measure a NR NTN neighbor cell served by a satellite. Therefore, legacy LTE TN UEs (earlier than Release 19) are not expected to support this feature and are outside the scope of this work item. Additionally, the capabilities agreed upon in this work item so far are capabilities without signaling [2]. Although the network may be aware that the UE supports NR NTN, it does not know whether the UE can read the necessary SIB information and perform the mobility event. Given that the network needs certainty whether an NR NTN frequency can be configured in the redirectedCarrierInfo, we propose that RAN2 introduces a new capability with signaling.

Introduce a new capability with signalling to support RRC Release with redirection from LTE TN to NR NTN.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce a new capability with signalling to support RRC Release with redirection from LTE TN to NR NTN.

# 4	References

RP-240846, Inter RAT mobility support from E-UTRAN TN to NR-NTN, RAN#103, Mar. 2024

R2-2409536, Introduction of LTE TN to NR NTN Mobility UE Capability, vivo, RAN2#128, Orlando, USA, November 2024.