3GPP TSG-RAN WG2 Meeting #129	R2-2501311

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.4.2

Source: 	Meta

Title:  	Discussion on DSR Enhancements

Document for:	Discussion and Decision

# Introduction

In the Revised WID on XR (eXtended Reality) for NR Phase 3, one of the objectives is to specify enhancements for scheduling [1]:

- Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105

In RAN2 #126-bis meeting, the following agreements were made on DSR enhancements:

Agreements on DSR enhancements - We do not change the definition of delay-critical data. - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s). - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds.

In RAN2 #127 meeting, the following agreements were made on DSR enhancements:

- Let the network configure the triggering and reporting thresholds without constraints. - RAN2's understanding is that the data that has been already reported in the DSR should not trigger another DSR. - The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. - The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In this paper, we discuss our views on the DSR enhancements.

# Discussion

In RAN2#126-bis, there is an FFS on whether old and new DSR can be configured/used at the same time if there is at least one LCG configured with multiple reporting thresholds. It is preferred that Rel-19 enhanced DSR is used.

Proposal 1:  When at least one LCG is configured with multiple reporting thresholds, Rel-19 DSR should be used.

Different from Rel-18 DSR, in Rel-19 enhanced DSR, triggering and reporting thresholds should be configured separately.

Proposal 2:  For Rel-19 enhanced DSR, configure triggering and reporting thresholds separately.

# Conclusion

In this paper, we discuss our views on the DSR enhancements.

Proposal 1:  When at least one LCG is configured with multiple reporting thresholds, Rel-19 DSR should be used.

Proposal 2:  For Rel-19 enhanced DSR, configure triggering and reporting thresholds separately.

# References

[1] RP-241771, “Revised WID on XR (eXtended Reality) for NR Phase 3” Melbourne, Australia, September 9-12, 2024