# 3GPP TSG-RAN WG2 Meeting #129		                  									R2-2501316

Athen, Greece, Feb. 17th – 22nd, 2025

Agenda item:	8.2.2

Source:	Robert Bosch GmbH

Title:	Discussions on ambient IoT paging

Document for:	Discussion and Decision

# Introduction

The ambient IoT WI description for RAN2 [1] mentions that:

Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

•	A-IoT Paging, including subsequent paging for the same service. Support at least the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

And also in the last RAN2 meeting, it was agreed that:

Agreements on paging

- An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.

- Capture the scenario that different readers may page the same device for same service request.  We will discuss this in WI phase (if in scope of WI) and consider progress from all the WGs

- Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

# Ambient IoT Paging Message

In the context of Ambient IoT (AIoT) devices, the paging message may contain additional information fields beyond the existing ones in the NR paging message. One such field is the multiple reader paging flag, which serves to alert AIoT devices that the paging message may be sent by multiple readers. This is crucial for the devices to understand that they may receive a paging message from various readers and avoid duplicated responses to the same paging message.

Observation 1: An ambient IoT device may receive a paging message from several readers.

Proposal 1: The AIoT paging message should have at least a flag to indicate to AIoT device(s) that more than one reader may send a paging message.

In this case, a paging message from the core network can trigger several readers to receive a paging message and forward it the relevant devices. Therefore, another important field is the reader group ID, which allows the network to prompt all readers belonging to a specific group to send the paging message. The additional information field ensures that readers can properly interpret and respond to incoming messages.

Proposal 2: AIoT paging may include a reader group ID which triggers a group of readers to page AIoT device(s).

Since ambient IoT devices located in specific geographical area can be paged to identify the existing devices in the given area, it is beneficial for the ambient IoT devices to be configured to keep a zone ID and if a paging message contains the same zone ID, all devices in that zone should respond to the paging message. However, this can be also done by group ID but a zone is a location dependent concept in comparison with  “group”. A zone may include several groups.

Observation 2: In many use cases, there is a need to page ambient IoT devices which are located in a pre-defined area.

Proposal 3: AIoT paging message can include information to page devices which are in a pre-defined geographical area.

Besides, in many use cases the R2D message should be transmitted with a high reliability and in case if the device fails to receive an R2D message, the reader must retransmit the failed message or any other data unit for e.g., segment. To achieve this goal, a paging message can activate/deactivate acknowledgement by a device for R2D messages.

Proposal 4: AIoT paging message can include at least a flag to activate/deactivate acknowledgement by a device for R2D message(s).

# Conclusions

Based on the discussion in the contribution, we have the following observation and proposals:

Observation 1: An ambient IoT device may receive a paging message from several readers.

Observation 2: In many use cases, there is a need to page ambient IoT devices which are located in a pre-defined area.

Proposal 1: The AIoT paging message should have at least a flag to indicate to AIoT device(s) that more than one reader may send the paging message.

Proposal 2: AIoT paging may include a reader group ID which triggers a group of readers to page AIoT device(s).

Proposal 3: AIoT paging message can include information to page devices which are in a pre-defined geographical area.

Proposal 4: AIoT paging message can include at least a flag to activate/deactivate acknowledgement by a device for R2D message(s).

# References

# [1] RP-243312: Solutions for Ambient IoT (Internet of Things) in NR