3GPP TSG-RAN WG2 Meeting #129	R2-2501322

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.9.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On PWS support for NB-IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The R19 WID [1] for IoT NTN has been updated in RAN plenary#105 meeting to cover the objective for support of PWS (Public Warning System). In details, the support of PWS messages for NB-IoT is expected to reuse the LTE mechanisms. Furthermore, the support of geo-fencing function for ETWS will be considered based on NR NTN outcome.

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

Following agreements have been reached in RAN2-127bis meeting and RAN2-128 meeting.

RAN2-127bis Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

RAN2-128 Agreements: 1. We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. 2. We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed 3. Add the following PWS indication in Paging-NB: - etws-Indication; - cmas-Indication.

In this contribution, we elaborate more on remaining issues related to PWS for NB-IoT NTN.

# 2	Discussion

## 2.1	Geofencing for ETWS

As part of the PWS objective for IoT NTN, RAN2 will consider supporting indication of intended service area for ETWS in IoT NTN based on NR NTN outcome. For Rel-19 NR NTN, RAN2 discussed whether the geofencing for an area smaller than an NTN cell should be supported. As a result, RAN2 sent LS to SA1 [2] and asked for feedback whether the capability to geofence emergency messages to an area should be supported for ETWS in an NTN cell

SA1 has attempted to address the issue described in the LS [2] and agreed a corresponding CR [4], providing the following new requirement:

## 5.6 Additional PWS Requirements Specific to ETWS Subject to local/regional regulatory requirements, in addition to ETWS Primary and Secondary Notifications, ETWS shall be able to support broadcasting Warning Notification Area(s). This requirement applies to Satellite E-UTRAN and Satellite NG-RAN. Subject to local/regional regulatory requirements, an ETWS-capable UE shall present the received ETWS Notification(s) (Primary or Secondary) if there is a corresponding Warning Notification Area(s) and if the ETWS-capable UE is able to determine that its location is within the corresponding Warning Notification Area(s). Otherwise, if an ETWS-capable UE is unable to or fails to determine its location, the ETWS-capable UE shall present the ETWS Notification(s) (Primary or Secondary) to the user.

According to the additional PWS requirement applicable to ETWS, defined recently by SA1, the UE capable of ETWS shall be able to present the ETWS notification if the UE is able to determine its location is within the Warning Notification Area. If the UE is unable to determine its location (i.e. within or outside the Warning Notification Area) it shall also present the ETWS notification.

Observation 1: According to SA1, ETWS-capable UE shall present the ETWS notification if the UE is able to determine it is within the Warning Notification Area or when the UE is unable to determine its location.

Furthermore, in the SA1 CR [4], the above requirement can be applied to not only Satellite NG-RAN but also Satellite E-UTRAN. In other words, the geofencing to the intended area should be supported for ETWS over IoT NTN.

Proposal 1: RAN2 to confirm that, the geofencing information of the intended area should be supported for ETWS over IoT NTN.

As it is expected the ETWS area may be smaller than a NTN cell footprint, it may be worth to find the means to avoid alerting UEs outside of the ETWS area but located within the same NTN cell (i.e. receiving the same System Information).

Observation 2: ETWS area is likely to cover a smaller area than NTN cell’s footprint.

Observation 3: It is beneficial if UEs outside of the ETWS area but located within the same NTN cell (i.e. receiving the same System Information) are not alerted.

To achieve such goal, the UE should be told where the intended ETWS area actually is. In the simplest approach the reference location + radius, known from Rel-18 non-terrestrial coverage area signaling, can be reused. Alternatively, the warningAreaCoordinatesSegments defined for CMAS may also be reused to indicate the geographical area (e.g., for SIB11). However, since the discussion in NR NTN is still ongoing, to prevent parallel discussions, IoT NTN discussion and conclusion can wait for the conclusion from NR NTN and then discuss the specific solution accordingly.

Proposal 2: Wait for NR NTN's conclusion to determine the solution for indicating the intended service area for ETWS over IoT NTN.

## 2.2	Inter-cell PWS reception

According to current TS36.331 specification, the PWS message may be segmented to accomodate a subset of the message information within the SIB (which has size limitations). However, for the UE receiving the PWS message, the segment feature is only supported for intra-cell case. i.e., if the RRC idle UE enters a new cell, the UE shall discard any previously buffered warning message segment(s).

#### 36.331: 5.2.2.4 System information acquisition by the UE … 1> if the UE is ETWS capable: 2> upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment: 3> discard any previously buffered warningMessageSegment; 3> clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11;

Observation 4: In legacy, the PWS message segmentation feature is only supported for the intra-cell scenario.

Different from TN, in NTN, the cell (satellite) is moving hence the UE may experience cell change frequently. For Earth-Moving cell (EMC), a UE may experience a cell change every ~6 seconds as indicated in TR38.821 Table 7.3.2.1.4-1. Therefore, the NW may not manage to broadcast all segments of a PWS message in the current NTN serving cell to a UE due to the large PWS message size (and corresponding huge number of segments) and limited cell availability time (~6 seconds). This may cause the UE cannot receive all the segments for the PWS message if the serving cell is changed before the UE receives all the segments.

Furthermore, whether UE can receive all the segments of a PWS message also depends on when the NW starts to broadcast the segments and the UE’s location. As illustrated in Figure 1, when the satellite/eNB starts to broadcast PWS segments at current moment, the UE located in the cell coverage edge cannot receive all the segments from current serving cell since its cell coverage will be moved out very soon. Similarly, in an Earth Fixed Cell scenario, the UE may also start the PWS message segment acquisition in one cell, but then experience a cell change (according to t-service).

Figure 1: The UE may not receive all segments from current (earth moving) serving cell

If the legacy mechanism of discarding buffered PWS segments upon cell change is followed, there might be the risk of non-succesful delivery of PWS message due to frequent cell change in NTN scenario.

Observation 5: In NTN, all the segments of PWS message may not be delivered to UE successfully within a single cell in moving cell scenario.

Therefore, how to support PWS segment reception in inter-cell scenario should be discussed for NTN. The PWS information (SIB11 and SIB12) already includes a message identifier, serial number and segment number.

SystemInformationBlockType11 information element -- ASN1START SystemInformationBlockType11 ::= SEQUENCE { messageIdentifier BIT STRING (SIZE (16)), serialNumber BIT STRING (SIZE (16)), warningMessageSegmentType ENUMERATED {notLastSegment, lastSegment}, warningMessageSegmentNumber INTEGER (0..63), warningMessageSegment OCTET STRING, dataCodingScheme OCTET STRING (SIZE (1)) OPTIONAL, -- Cond Segment1 ..., lateNonCriticalExtension OCTET STRING OPTIONAL } -- ASN1STOP

Based on this it may be possible for the UE to combine segments received on two different cells, but there is an inherent risk of the cells not having coordinated transmissions. Therefore, we think RAN2 can discuss whether the cells shall indicate to the UE that inter-cell PWS message segment reception is possible.

Proposal 3: RAN2 to discuss how to support continued reception of PWS segmentation of message in moving cell scenario.

## 2.3	Support PWS in S&F operation

When the cell capable of store-and-forward operates in normal mode the PWS transmission can be supported for the areas that is under the coverage of the cell at the time of normal mode operation. When the cell switches to S&F mode the time of delivery of PWS depends on the actual service area and the time the cell takes to start serving the new service area.  This impacts the delivery time requirements of PWS.

Observation 6: PWS delivery when cell operates in S&F mode may have impacts on meeting delay requirements of PWS services.

From RAN2 perspective the UE capability for S&F and PWS support are independent features and it is up to network implementation to configure these services together. However whether such support is needed for PWS needs to be decided in SA2 based on service requirements and impacts.

Observation 7: RAN2 assumes no additional impacts in supporting PWS functionality in S&F operation.

Based on the above analysis we propose that support of PWS in S&F operation is decided by SA2. RAN2 can indicate that this can be supported without additional signalling impacts

Proposal 4: Support for PWS in SF operation is decided at SA2. RAN2 to decide whether to send LS on this or wait for SA2 decision.

## 2.4	PWS delivery during eDRX

eDRX is a key feature for NB-IoT devices that enhances their power efficiency and extends battery life. This makes it as a key enabler for NB-IoT in massive IoT deployments. To further reduce the energy consumption for paging monitoring towards a UE configured with eDRX, the wake-up signals (WUS) feature can be used. The typical scenario is that NW configure a short eDRX cycle configurations to UE to support the case where the network command expects a UE response within specific duration.

Observation 7: WUS configuration helps to minimize downlink monitoring related energy consumption for UE configured with shorter eDRX cycle.

However, if PWS is supported and the transmission is indicated via paging messages (NB-IoT paging) or direct indication information message, it may wake-up all the UEs in the satellite coverage area if the current WUS is used. For PWS scheduling in resource and energy efficient manner use of separate WUS for PWS services can be considered.

Observation 8: When WUS is configured in NB-IoT cell, the PWS transmission may false wake-up all the UE outside the PWS service area resulting in reducing the overall efficiency of WUS benefits.

The false wake-up problem of PWS for NB-IoT UE is specific problem for NTN where the cell coverage will be larger than PWS service area. Hence we propose RAN2 to investigate the impacts of PWS on WUS efficiency and identify need for changes.

Proposal 5: RAN2 to investigate the false wake-up issue for PWS transmission for WUS configuration in NB-IoT-NTN.

# 3	Conclusion

ETWS Changes for area notification in NB-IoT NTN

Observation 1: According to SA1, ETWS-capable UE shall present the ETWS notification if the UE is able to determine it is within the Warning Notification Area or when the UE is unable to determine its location.

Proposal 1: RAN2 to confirm that, the geofencing information of the intended area should be supported for ETWS over IoT NTN.

Observation 2: ETWS area is likely to cover a smaller area than NTN cell’s footprint.

Observation 3: It is beneficial if UEs outside of the ETWS area but located within the same NTN cell (i.e. receiving the same System Information) are not alerted.

Proposal 2: Wait for NR NTN's conclusion to determine the solution for indicating the intended service area for ETWS over IoT NTN

PWS reception in moving cell scenario

Observation 4: In legacy, the PWS message segmentation feature is only supported for the intra-cell scenario.

Observation 5: In NTN, all the segments of PWS message may not be delivered to UE successfully within a single cell in moving cell scenario.

Proposal 3: RAN2 to discuss how to support continued reception of PWS segmentation of message in moving cell scenario.

PWS Support in SF mode

Observation 6: PWS delivery when cell operates in S&F mode may have impacts on meeting delay requirements of PWS services.

Observation 7: RAN2 assumes no additional impacts in supporting PWS functionality in S&F operation.

Proposal 4: Support for PWS in SF operation is decided at SA2. RAN2 to decide whether to send LS on this or wait for SA2 decision.

WUS Efficiency Impacts for PWS Transmission

Observation 7: WUS configuration helps to minimize downlink monitoring related energy consumption for UE configured with shorter eDRX cycle.

Observation 8: When WUS is configured in NB-IoT cell, the PWS transmission may false wake-up all the UE outside the PWS service area resulting in reducing the overall efficiency of WUS benefits.

Proposal 5: RAN2 to investigate the false wake-up issue for PWS transmission for WUS configuration in NB-IoT-NTN.