3GPP TSG-RAN WG2 Meeting #129	R2-2501491

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:		8.7.5

Source:		Futurewei

Title:		Combined approach to avoid unnecessary RLC retransmissions

Document for:		Discussion and Decision

# Introduction

In meeting #127, RAN2 agreed on the following regarding how to avoid unnecessary RLC retransmissions [1]:

In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where

    - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach

    - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

In this contribution, we provide the details of the combined approach.

# Discussions

## Details on RX side

In one of our previous papers [2], we have shown that even for a video stream with 150Mbps data rate (corresponding to AVC with 8k resolution [3]) and 12-bit RLC SN, it takes about 156 msec before the receiving window is stalled by a missing RLC PDU, comparing to the 10 msec PDB/PSDB for DL XR video. Although the PDB/PSDB for UL XR video traffic may be longer than 10 msec, the video data rate for UL XR video traffic is typically much lower than 150Mbps. Therefore, the receiving side of the AM RLC entity has a quite amount of time to detect that a missing RLC PDU has become obsolete before the receiving window can be stalled by it.

The local timer needs to be started by a local event detected at the receiving side of the AM RLC entity, e.g., receiving an RLC PDU with an SN after an SN gap created by the missing RLC PDU(s). A first thought is that if an RLC PDU belonging to a next PDU Set has been received for more than (PDB/PSDB – traffic periodicity), any RLC PDUs belonging to the previous video frame should have become obsolete. However, because the PDU Set Information is not provided to the receiving side of the AM RLC entity, there may be an ambiguity regarding whether the received RLC PDU and the missing RLC PDU(s) belong to the same PDU Set or not. Therefore, to be safe, the initial timer value should be longer than (PDB/PSDB – traffic periodicity), e.g., is set to be equal to PDB/PSDB. In this way, if the received RLC PDU just becomes obsolete, any RLC PDUs in the SN gap should be obsolete as well.

Therefore, we proposal that when detecting RX_Next_Highest > RX_Next +1 for the first time, a SN gap timer (t-SNGap) is started by the receiving side of the AM RLC entity and a new state variable, e.g., referred to as RX_Gap_Trigger, is set to be equal to RX_Next_Highest. When t-SNGap is running, no new t-SNGap should be started by the same RLC entity due to the reception of more subsequent RLC PDUs, i.e., t-SNGap is per RLC entity. The initial value of t-SNGap can be RRC-configured but should be greater than or equal to PDB/PSDB to prevent premature detection.

When t-SNGap is running, if an RLC SDU with an SN equal to RX_Next is received, RX_Next is updated in accordance with the current RX operations, and the t-SNGap is stopped and reset. After RX_Next is updated, if RX_Next_Highest > RX_Next +1, t-SNGap is started again and RX_Gap_Trigger is updated to be equal to RX_Next_Highest.

When t-SNGap expires, any RLC SDUs with an SN between RX_Next and RX_Gap_Trigger are considered as being obsolete, RX_Next is updated to the SN of the first RLC SDU with SN > RX_Gap_Trigger for which not all bytes have been received. After RX_Next is updated, if RX_Next_Highest > RX_Next +1, t-SNGap is started again and RX_Gap_Trigger is updated to be equal to RX_Next_Highest.

Proposal 1. Introduce a local timer, referred to as t-SNGap, and a new state variable, referred to as RX_Gap_Trigger, at the receiving side of AM RLC entity. When t-SNGap is running, no new t-SNGap can be started by the same RLC entity, i.e., t-SNGap is per RLC entity.

Proposal 2. The initial value of t-SNGap can be RRC-configured but should be greater than or equal to PDB/PSDB.

Proposal 3. When detecting RX_Next_Highest > RX_Next +1 while t-SNGap is not running, start t-SNGap and set RX_Gap_Trigger to be equal to RX_Next_Highest.

Proposal 4. When t-SNGap is running, if an RLC SDU with an SN equal to RX_Next is received, RX_Next is updated in accordance with the current RX operations, and the t-SNGap is stopped and reset. After RX_Next is updated, if RX_Next_Highest > RX_Next +1, t-SNGap is started again and RX_Gap_Trigger is updated to be equal to RX_Next_Highest.

Proposal 5. When t-SNGap expires, any RLC SDUs with an SN between RX_Next and RX_Gap_Trigger are considered as being obsolete, RX_Next is updated to the SN of the first RLC SDU with SN > RX_Gap_Trigger for which not all bytes have been received. After RX_Next is updated, if RX_Next_Highest > RX_Next +1, t-SNGap is started again and RX_Gap_Trigger is updated to be equal to RX_Next_Highest.

## Details on TX side

Since the transmitting side of the AM RLC entity receives the SDU discard indication from upper layer, it can stop all pending transmission and retransmission of all RLC PDUs generated from the discarded RLC SDUs immediately. However, for those discard RLC SDUs of which an RLC PDU has already been transmitted but has not been positively acknowledged yet, care needs to be taken to prevent them from stalling the transmitting window.

Today, the transmitting window will be stalled when an RLC SDU with an RLC SN equal to TX_Next_Ack has not been positively acknowledged in an RLC Status Report yet and the next to-be-transmitted RLC SDU has an RLC SDU equal to (TX_Next_Ack + AM_Window_Size), after considering the possibility of SN wrap-around. We suggest not to have the receiving side of the peer AM RLC entity to report ACK for those RLC SDUs that they have deemed as obsolete by using the local timer t-SNGap, as described above, when addressing the issue of potential TX-window stalling due to SDU discarding.

Because the decision to discard the obsolete RLC SDUs (and associated RLC data PDU(s)), for which an RLC transmission has begun, is made by the transmitting side, the transmitting side does not need to rely on an ACK indication for this RLC SDU in the RLC Status Report to push its transmitting window forward. Instead, the transmitting side of the AM RLC entity can compare the RLC SNs of the discarded RLC SDUs, for which an RLC transmission has begun, with TX_Next_Ack, and if any of these RLC SNs is greater than TX_Next_Ack after considering the possibility of SN wrap-around, set TX_Next_Ack to be equal to the sum of one and the last RLC SN among the discarded RLC SDUs. To ensure that the TX-window always advances after the RX-window advances, the transmitting side of the AM RLC entity can start a local timer when receiving the SDU discard indication from upper layer with the initial timer value set to be greater than or equal to PDB/PSDB and advances its TX-window (by updating TX_Next_ACK, as described before) only after the local timer expires. In this way, the transmitting window will not be stalled by the discarding of the obsolete RLC SDU, for which an RLC transmission has begun.

Similar to the RX-window stalling, as we have analyzed in [2], it takes a much longer time than the PDB/PSDB before the SN of a discarded SDU can stall the TX-window. Therefore, implementations may decide on the initial value of the local timer on the transmitting side of the AM RLC entity with a wide margin.

Proposal 6. The receiving side of the peer AM RLC entity does not report ACK for those RLC SDUs deemed as obsolete in the RLC Status Report.

Proposal 7. When the transmitting side of the AM RLC entity receives SDU discard indication from upper layer (i.e., PDCP), the AM RLC entity performs the following steps:

- Step 1. Identify each RLC SDU, if any, which is indicated as being discarded by upper layer, and either 1) an RLC PDU containing the RLC SDU or a segment thereof has been transmitted but for which a positive acknowledgement has not been received yet or 2) a segment of the RLC SDU has been transmitted but another segment of the RLC SDU has not been transmitted yet.

- Step 2. Determine whether TX_Next_Ack is among the RLC SNs of the RLC SDUs, if any, identified in Step 1.

- Step 3. If the answer is Yes in Step 2, update TX_Next_Ack to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next, for which a positive acknowledgment has not been received yet, and the RLC SN is not among the RLC SDUs identified in Step 1.

Proposal 8. To ensure that the TX-window advances after the RX-window advances, the transmitting side of the AM RLC entity can start a local timer when receiving the SDU discard indication from upper layer with the initial timer value set to be greater than or equal to PDB/PSDB and performs Step 3, as in Proposal 6, only after the local timer expires. FFS if the initial timer value of the local timer can be decided by implementation.

# Conclusions

We propose the following:

Proposal 1. Introduce a local timer, referred to as t-SNGap, and a new state variable, referred to as RX_Gap_Trigger, at the receiving side of AM RLC entity. When t-SNGap is running, no new t-SNGap can be started by the same RLC entity, i.e., t-SNGap is per RLC entity.

Proposal 2. The initial value of t-SNGap can be RRC-configured but should be greater than or equal to PDB/PSDB.

Proposal 3. When detecting RX_Next_Highest > RX_Next +1 while t-SNGap is not running, start t-SNGap and set RX_Gap_Trigger to be equal to RX_Next_Highest.

Proposal 4. When t-SNGap is running, if an RLC SDU with an SN equal to RX_Next is received, RX_Next is updated in accordance with the current RX operations, and the t-SNGap is stopped and reset. After RX_Next is updated, if RX_Next_Highest > RX_Next +1, t-SNGap is started again and RX_Gap_Trigger is updated to be equal to RX_Next_Highest.

Proposal 5. When t-SNGap expires, any RLC SDUs with an SN between RX_Next and RX_Gap_Trigger are considered as being obsolete, RX_Next is updated to the SN of the first RLC SDU with SN > RX_Gap_Trigger for which not all bytes have been received. After RX_Next is updated, if RX_Next_Highest > RX_Next +1, t-SNGap is started again and RX_Gap_Trigger is updated to be equal to RX_Next_Highest.

Proposal 6. The receiving side of the peer AM RLC entity does not report ACK for those RLC SDUs deemed as obsolete in the RLC Status Report.

Proposal 7. When the transmitting side of the AM RLC entity receives SDU discard indication from upper layer (i.e., PDCP), the AM RLC entity performs the following steps:

- Step 1. Identify each RLC SDU, if any, which is indicated as being discarded by upper layer, and either 1) an RLC PDU containing the RLC SDU or a segment thereof has been transmitted but for which a positive acknowledgement has not been received yet or 2) a segment of the RLC SDU has been transmitted but another segment of the RLC SDU has not been transmitted yet.

- Step 2. Determine whether TX_Next_Ack is among the RLC SNs of the RLC SDUs, if any, identified in Step 1.

- Step 3. If the answer is Yes in Step 2, update TX_Next_Ack to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next, for which a positive acknowledgment has not been received yet, and the RLC SN is not among the RLC SDUs identified in Step 1.

Proposal 8. To ensure that the TX-window advances after the RX-window advances, the transmitting side of the AM RLC entity can start a local timer when receiving the SDU discard indication from upper layer with the initial timer value set to be greater than or equal to PDB/PSDB and performs Step 3, as in Proposal 6, only after the local timer expires. FFS if the initial timer value of the local timer can be decided by implementation.

# References

R2-2407575, Report from session on R18 MBS, R18 QoE and R19 XR, Session Chair (Huawei).

R2-2407511, Discussions on RLC enhancements for Rel-19 XR, Futurewei.

TR 26.928 V17.0.0.