3GPP TSG-RAN WG2 Meeting #129	R2-2501512

Athens, Greece, Feb. 17th – 21st, 2025

Title:	[Draft] Reply LS on the STx2P UE Features

Response to:	R1-2410665 (R2-2500006)

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL

Source:	ZTE [to be RAN2]

To:	RAN1

Cc:

Contact person:	Wenting Li

Li.wenting@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN2 thanks RAN1 for the LS in R1-2410665 on RAN1 UE features list for NR.

Regarding the following two RAN1 FGs on STx2P, RAN2 discussed whether both FGs are also applicable for FR1 and whether a value needs to be signaled for each SCS:

| 40-6-3a-1 | 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG. | Per FSPC | FR2 only | candidate values: UE reports candidate value independently for each SCS in unit of symbols For 15kHz SCS: {1,2,4} For 30kHz SCS: {1,2,4,8} For 60kHz SCS: {2,4,8,16} For 120kHz SCS: {4,8,16,32} For 480kHz SCS: {16,32,64,128} For 960kHz SCS: {32,64,128,256} Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. |
| --- | --- | --- | --- | --- |
| 40-6-3b-2 | 1. Require additional timeline to process multiple TBs for non-codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG. | Per FSPC | FR2 only | candidate values: UE reports candidate value independently for each SCS in unit of symbols For 15kHz SCS: {1,2,4} For 30kHz SCS: {1,2,4,8} For 60kHz SCS: {2,4,8,16} For 120kHz SCS: {4,8,16,32} For 480kHz SCS: {16,32,64,128} For 960kHz SCS: {32,64,128,256} Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. |

RAN2 considers, if both FGs are not applicable for FR1 and if the cross-carrier scheduling is not supported, UE capability values for 15/30kHz SCS may not need to be defined in the UE capability signaling.

In order to specify the signaling for both FGs, RAN2 respectfully asks RAN1 to confirm whether the combination of the cross-carrier scheduling and MDCI MTRP PDSCH/PUSCH operation is supported in R18.  If not supported, RAN2 would like to ask RAN1 to answer the following 2 questions:

- Q1: Are the FGs 40-6-3a-1 and 40-6-3b-2 also applicable for FR1?

- Q2: For both FGs 40-6-3a-1 and 40-6-3b-2 does a UE capability value need to be signaled for each SCS in a given Component Carrier?

Related to Q2, RAN2 would like to inform RAN1 that in the current per FSPC UE capability reporting, the UE can determine the minimum SCS based on the existing two capabilities (i.e. UL: FeatureSetUplinkPerCC->supportedSubcarrierSpacingUL/DL: FeatureSetDownlinkPerCC-> supportedSubcarrierSpacingDL).

2	Actions

To RAN1 group

ACTION: 	RAN2 respectfully asks RAN1 to confirm whether the combination of the cross-carrier scheduling and MDCI MTRP PDSCH/PUSCH operation is supported in R18 and also answer the above questions.

3	Dates of next TSG-RAN WG2 meetings

TSG-RAN2 Meeting #129bis             07 - 11 April 2025	                             Wuhan, CN

TSG-RAN2 Meeting #130	19 - 23 May 2025	Malta, MT