3GPP TSG-RAN2 Meeting #129	R2-2501539

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1244 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Optional feature to support RAN-based Notification Area Update for Multimedia Priority Service (MPS) [MPS_RNAU_ResumeCause] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, FirstNet, AT&T, Verizon, Samsung, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The purpose of this CR is to add an optional feature without UE radio access capability parameters to address the handling of the MPS-subscribed UE during RAN-based Notification Area Updates (RNAUs). In RRC_INACTIVE state, a UE remains CM-CONNECTED and can move within a notification area previously-configured by NG-RAN. A RNA update is triggered when the UE moves out of its notification area by receipt of a SIB1. An update may also be triggered periodically. In 3GPP TS 38.300, the gNB handles access attempts with resume cause "mps-PriorityAccess" with high priority. However, the resume cause assigned to MPS-subscribed UEs during RNAU is “rna-Update”, not “mps-PriorityAccess”. As such, the RRCResumeRequest sent by the MPS-subscribed UE during the RNAU procedure may be dropped when the gNB is congested. Furthermore, if the gNB is congested and sends a RRCReject to the UE, the UE starts timer T302 (waitTime: 1-16 seconds) and is barred until T302 expires. Only access categories 0 and 2 (i.e., paging response and emergency) are exempt from UAC while timer T302 is running. However, access category 8 is used during RNAU. Therefore, all resume procedures in INACTIVE mode are blocked (except for paging response and emergency), including mobile origination and MPS resumes, until timer T302 expires. This issue is solved by adding an optional feature to support using the mps-PriorityAccess resume cause for RNAU for MPS subscribed UEs such that the gNB does not reject the RNAU procedure triggered by an MPS-subscribed UE when congested and the MPS-subscribed UE does not need to start timer T302. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an optional feature to exempt MPS-subscribed UEs from congestion control measures at the gNB during RNAU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPS-subscribed UEs will be blocked at the gNB during RNAU if the gNB is congested. A RRCReject will trigger the MPS-subscribed UE to block all resume procedures, except paging response and emergency, until timer T302 expires. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.306 CR 1906 TS 36.331 CR 5090 TS 38.331 CR 5207 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

## 5.3	RRC connection

Definitions for feature

RRC connection release with deprioritisation It is optional for UE to support RRCRelease with deprioritisationReq as specified in TS 38.331 [9].

RRC connection establishment failure with temporary offset It is optional for UE to support RRC connection establishment failure with temporary offset (Qoffsettemp) as specified in TS 38.331 [9].

Selection of acceptable E-UTRA cell upon HO failure during EPS fallback for emergency call It is optional for UE to support selecting an acceptable E-UTRA cell supporting emergency call if no suitable E-UTRA cell is available upon handover failure during EPS fallback when the UE has an ongoing emergency call as specified in TS 38.331 [9].

E-UTRA cell selection upon HO failure during EPS services fallback It is optional for UE to support selecting a suitable E-UTRA cell, and support selecting an acceptable E-UTRA cell supporting emergency call if no suitable E-UTRA cell is available upon handover failure when the UE is performing emergency services fallback as specified in TS 38.331 [9].

RAN-based Notification Area Update for high priority UEs. It is optional for a UE configured with Access Identities equal to 1, 2, or 11-15 to use a high priority access resume cause if the resumption of the RRC connection is triggered due to an RAN-based Notification Area Update, as specified in TS 38.331 [9].

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>