3GPP TSG-RAN2 Meeting #129-bis	R2-2502912

Wuhan, China, 7 - 11 April 2025 	Revision of R2-2501095

Agenda Item:	8.4.4 Procedures for LP-WUS in RRC_CONNECTED

Source: 	Ericsson

Title:  	LP-WUS in Connected

Document for:	Discussion and Decision

# Introduction

In this contribution the open issues for LP-WUS in Connected are discussed further.

For the RAN1 and RAN2 agreements in previous meetings the reader is referred to the Annex.

# Discussion

## Option 1-2

Both RAN1 and RAN2 have agreed that with option 1-2:

- a new timer is started when LP-WUS is detected

- option 1-1 and option 1-2 are not configured together

- other DRX parameters are not affected, e.g. when there is new UL/DL transmission the UE starts drx-InactivityTimer.

In this contribution the new timer is denoted lpwus-PDCCHMonitoringTimer . Similar as with the legacy drx-onDurationTimer when lpwus-PDCCHMonitoringTimer  is started the UE enters Active Time as already agreed earlier.

Following RAN1 discussion, RAN2 can confirm that LP-WUS is enabled/disabled using RRC signalling:

RAN2 confirms that option 1-2 is enabled/disabled using RRC signalling.

RAN1 agreed that with option 1-1 and 1-2 there is no impact on RRM/RLM/BFD measurements, and that similar as for DCP, the UE can be configured to report periodic CSI/L1-RSRP when LP-WUS is not detected. This means that while there is no traffic, i.e. the UE is outside Active Time and LP-WUS is not detected, then the UE still has to measure RLM/BFD every C-DRX cycle using MR. This is less power consuming than when periodic CSI/L1-RSRP reporting is configured, and the UE has to transmit in UL. Periodic CSI/L1-RSRP reporting significantly reduces the power saving gains of LP-WUS. Instead of periodic reporting every C-DRX cycle, using LP-WUS the NW can request aperiodic CSI/L1-RSRP less often:

LP-WUS provides limited power saving gains in connected, i.e. even when there is no traffic, the MR still has to measure RLM/BFD every C-DRX and, if configured, report periodic CSI/L1-RSRP.

Option 1-1 can provide some power saving gains and option 1-2 does not provide further power savings when the same C-DRX cycle is configured as with option 1-1. But option 1-2 can provide significant DL latency reduction for the initial DL packets when a short LP-WUS periodicity is configured. With option 1-1 the latency can also be reduced by configuring a short C-DRX cycle. But configuration of a short LP-WUS periodicity with option 1-2 is less power consuming compared to configuration of a short C-DRX cycle with option 1-1. In the latter case the UE has to measure RLM/BFD more frequently which increases the power consumption.

The specification impact is reduced when option 1-1 is considered as a special configuration/case of option 1-2, i.e. when the LP-WUS periodicity equals the C-DRX cycle length, then option 1-2 equals option 1-1 (apart from the RLF/BFD measurements):

The LP-WUS periodicity with option 1-2 can be configured to be equal to the C-DRX cycle length.

During Rel-16 the support of DCP with short DRX was discussed in both RAN1 and RAN2. RAN1 did not have time to finish their work and DCP is not supported with short DRX. There is no RAN2 impact to support LP-WUS with short DRX:

From RAN2 perspective LP-WUS can be configured with short DRX.

With DCP the UE is required to start the drx-OnDurationTimer when the UE is not able to monitor the DCP, e.g. when the time between the end of the Active Time is shorter than the DCP time offset. Or when the UE is not able monitor DCP because it has to perform other activities (e.g. during BWP switching, measurement gaps or when monitoring CFRA response during beam failure recovery). The same principle should be applied with both option 1-1 and 1-2:

In Option 1-1, when the UE is not able to monitor the LP-WUS occasion the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

## Minimum time gap and preferred time offset

RAN1 is still discussing the details of minimum time gap and preferred time offset but on a high level the agreements are

- UE signals one value per SCS for the minimum time gap in UE capability.

- UE can indicate a preferred time offset via UAI signalling

- There is a single time offset for the UE at any time.

With DCP the UE can signal a minimum time gap (in slots) per SCS supported in FR1, FR2-1 and FR2-2 in licensed and unlicensed spectrum (as configured by drx-Adaptation-r16 and drx-Adaptation-r17).

With LP-WUS the time offset refers to the time between LP-WUS reception by the LR and start of PDCCH monitoring by the MR.

If the UE is configured with multiple time offsets and it is left to UE implementation which time offset to use, the network resources are wasted. Therefore, we propose:

The gNB configures a single time offset which is equal or longer than the minimum time gap signaled in the LP-WUS UE capability.

The motivation to signal a preferred time offset via UAI is to enable a different sleep state in the UE e.g. light or deep sleep:

|  | Relative Power (unit) | Transition time (ms) | Transition energy (ms) |
| --- | --- | --- | --- |
| Micro sleep | 45 | 0 | 0 |
| Light Sleep | 20 | 6 | 100 |
| Deep Sleep | 1 | 20 | 450 |

If configured, the UE can report a preferred time offset, i.e. try to change the configured offset:

If configured, the UE can signal a preferred time offset via UAI signalling.

RAN1 indicated that for Idle and Inactive the UE may not support LP-WUS on all the supported bands. The DCP capability is per UE (see drx-Adaptation-r16 and drx-Adaptation-r17). RAN2 should discuss and clarify whether the UE in connected mode may not support LP-WUS on all supported bands:

If the UE does not support LP-WUS in connected mode on all supported bands, then the LP-WUS UE capability for connected mode includes a supportedBandListNR IE.

## CA, secondary DRX and DC

RAN1 agreed that LP-WUS is supported with both CA and DC. It is FFS for which cells LP-WUS is applicable. RAN plenary confirmed that FR2 is within the scope of the LP-WUS WID [1,2]. This then also implies that LP-WUS with secondary DRX can be evaluated.

The UE is typically configured with CA to provide the highest throughput and shortest latency. Based on RRM measurement reporting and load balancing the CA configuration may change using RRCReconfiguration, i.e. carriers can be added or removed. Based on traffic demand and to save power SCells may be deactivated or enter dormant state (dormant BWP is used).

The DRX configuration applies to all the cells of the Cell Group (CA/DC) or DRX group (secondary DRX), i.e. the UE is in the same DRX state (e.g. Active Time) in all the cells in the Cell Group or DRX group. The same DRX parameter applies to all the cells of the same Cell/DRX Group, e.g. one drx-OnDurationTimer is running for each Cell/DRX Group. The scheduler uses the full capacity provided by the Cell/DRX groups to schedule traffic.

With DC and secondary DRX the Cell groups and DRX groups can be in different DRX states, e.g. MCG is inside Active Time and SCG is outside Active Time. With DC the timing is not aligned, i.e. the DRX cycles and drx-OnDurationTimer may start/stop at different times.

With secondary DRX a shorter drx-OnDurationTimer and drx-InactivityTimer for FR2 can be configured to enable the secondary DRX group on FR2 to go into sleep more quickly and save power. Thus, the DRX groups may be in different DRX states.

Cross carrier scheduling is not supported with secondary DRX. The trigger for aperiodic CSI and PUSCH reporting is on the same carrier e.g. in PCell of the default DRX group, but the cell for which CSI is reported may be in the same or other DRX group (frequency range). The network does not trigger aperiodic reporting for the other DRX group, when the UE is outside Active Time in the other DRX group, i.e. the UE is only required to measure during Active Time.

DCP can be configured with CA or DC, i.e. the primary cell in the cell group can be configured with DCP. When DCP is triggered then the drx-OnDurationTimer is started and the UE starts monitoring PDCCH in all the serving cells of the cell group and the cell group enters Active Time. This is not applicable for cells that are deactivated or in dormant state, i.e. the UE does not monitor PDCCH on those cells. DCP is not supported with secondary DRX, because secondary DRX was introduced under TEI and it was agreed to keep it simple.

In case the UE does not support LP-WUS on some of the cells/bands in connected mode then the UE can be reconfigured such that LP-WUS is supported on PCell/PSCell. The UE may support LP-WUS using one or multiple LRs e.g. each LR supports a limited number of bands/frequencies to simplify the implementation. Or the UE may support LP-WUS only in one DRX group (e.g. FR1 only) or only in one Cell group.

In the following it is assumed that option 1-1 is a special case/configuration of option 1-2, i.e. LP-WUS periodicity equals the C-DRX cycle length.

LP-WUS should be supported with CA and DC on the primary cell of the cell group, similar as with DCP:

RAN2 to discuss if/how LP-WUS can be configured with CA and DC on the primary cell of the cell group. When LP-WUS is detected the drx-OnDurationTimer in Option 1-1, or lpwus-PDCCHMonitoringTimer in Option 1-2, is started and the all the serving cells of the cell group enter Active Time.

LP-WUS should also be supported with secondary DRX and configured per DRX group. As stated before secondary DRX was treated in TEI and it was agreed to keep it simple. There are not technical reasons to not support LP-WUS with secondary DRX, similar as with CA/DC.

RAN2 to discuss if/how LP-WUS can be configured with secondary DRX, i.e. one LP-WUS is configured on the PCell of the default DRX group and another LP-WUS is configured on a serving cell of the secondary DRX group. When LP-WUS is detected  the  drx-OnDurationTimer in Option 1-1, or lpwus-PDCCHMonitoringTimer in Option 1-2, of the DRX group is started and the DRX group enters Active Time.

## Activation/deactivation

RAN1 and RAN2 agreed that LP-WUS monitoring is enabled/disabled via RRC signalling. It is FFS whether there is additional L1/L2 signalling, timers or conditions to activate/deactivate LP-WUS monitoring when LP-WUS is configured.

Using existing RRM/CSI measurement reporting the gNB can configure or de-configure LP-WUS via RRCReconfiguration, e.g. when the UE is inside/outside LP-WUS coverage. When LP-WUS is configured the gNB uses LP-WUS to wake-up the UE irrespective whether the UE is inside or outside LP-WUS coverage. To avoid ping-pong, i.e. frequent reconfigurations, the gNB can decide to configure LP-WUS after a certain time period when the UE has reported good coverage and immediately de-configure LP-WUS when the quality drops below a certain threshold. This can be left to gNB implementation.

It is important that LP-WUS is reliable and robust, i.e. risk for the error case that the UE is using LP-WUS but the NW is not should be avoided since the UE becomes unreachable in the DL. The opposite, i.e., when the NW is using LP-WUS but the UE directly monitors PDCCH does not cause the UE to become unreachable in the D issue, i.e. it only wastes resources from unnecessary LP-WUS transmission.

For example, when the UE is outside LP-WUS coverage and cannot receive LP-WUS then the UE should monitor PDCCH directly. It is possible that the UE suddenly loses LP-WUS coverage and there is delay in the RRM/CSI measurement reporting, then the UE may become temporarily unreachable until LP-WUS is de-configured. Typically, RRM and CSI reporting is not configured periodically to avoid high signalling load, but event based (RRM) or aperiodic (CSI). Independent on how the reporting is configured, the UE performs RRM/RLM/BFD measurements much more often than the reporting, and a local decision by the UE on current measurements can take much quicker effect.

To resolve the problem that the UE suddenly may lose LP-WUS coverage before LP-WUS is de-configured by the gNB an LR exit condition can be configured, similar when LP-WUS is configured in SIB for UEs in idle/inactive. In connected mode the UE is not required to measure LP-SS, i.e. an exit condition based on LP-SS measurements is not possible. But the exit condition can be based on RRM/RLM/BFD measurements, and RLM/BFD measurements are preferred because the UE is also required to measure RLM/BFD outside Active Time at least every C-DRX cycle, i.e. exit condition is also detected outside Active Time when there is no traffic:

The LP-WUS configuration can include an exit condition based on RLM measurements. When the exit condition is fulfilled the UE stops monitoring LP-WUS and monitors PDCCH directly.

## Paging PDCCH

In connected mode the UE does not receive Paging PDSCH, but the UE is required to monitor Paging PDCCH, e.g. to receive ETWS/CMAS warning messages. If LP-WUS is configured in SIB, then the UE in connected mode can use the idle/inactive LP-WUS to monitor Paging PDCCH:

If LP-WUS in idle/inactive mode is configured in SIB then the UE in connected mode may use LP-WUS monitoring as in idle/inactive mode to receive Paging PDCCH e.g. SI-change and ETWS/CMAS indication in connected mode.

# Summary

RAN2 is kindly asked to discuss LP-WUS in connected mode:

Observation 1	LP-WUS provides limited power saving gains in connected, i.e. even when there is no traffic, the MR still has to measure RLM/BFD every C-DRX and, if configured, report periodic CSI/L1-RSRP.

Proposal 1	RAN2 confirms that option 1-2 is enabled/disabled using RRC signalling.

Proposal 2	The LP-WUS periodicity with option 1-2 can be configured to be equal to the C-DRX cycle length.

Proposal 3	From RAN2 perspective LP-WUS can be configured with short DRX.

Proposal 4	In Option 1-1, when the UE is not able to monitor the LP-WUS occasion the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

Proposal 5	The gNB configures a single time offset which is equal or longer than the minimum time gap signaled in the LP-WUS UE capability.

Proposal 6	If configured, the UE can signal a preferred time offset via UAI signalling.

Proposal 7	If the UE does not support LP-WUS in connected mode on all supported bands, then the LP-WUS UE capability for connected mode includes a supportedBandListNR IE.

Proposal 8	RAN2 to discuss if/how LP-WUS can be configured with CA and DC on the primary cell of the cell group. When LP-WUS is detected the drx-OnDurationTimer in Option 1-1, or lpwus-PDCCHMonitoringTimer in Option 1-2, is started and the all the serving cells of the cell group enter Active Time.

Proposal 9	RAN2 to discuss if/how LP-WUS can be configured with secondary DRX, i.e. one LP-WUS is configured on the PCell of the default DRX group and another LP-WUS is configured on a serving cell of the secondary DRX group. When LP-WUS is detected  the  drx-OnDurationTimer in Option 1-1, or lpwus-PDCCHMonitoringTimer in Option 1-2, of the DRX group is started and the DRX group enters Active Time.

Proposal 10	The LP-WUS configuration can include an exit condition based on RLM measurements. When the exit condition is fulfilled the UE stops monitoring LP-WUS and monitors PDCCH directly.

Proposal 11	If LP-WUS in idle/inactive mode is configured in SIB then the UE in connected mode may use LP-WUS monitoring as in idle/inactive mode to receive Paging PDCCH e.g. SI-change and ETWS/CMAS indication in connected mode.

# References

- RP-242361, Way forward on LPWUS for FR2, vivo, DISC, RAN#105

- RP-241824, Revised WID Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC, RAN#105

# Annex: Agreements

## RAN1 agreements (selection)

Agreement

For LP-WUS CONNECTED mode operation, the following case is not supported:

- Both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell

Agreement

- For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer.

  - Approach 2:

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle.

- For LP-WUS MOs in connected mode for Option 1-2, following is considered further

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset.

  - Approach 2:

    - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters.

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start.

Agreement

For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution

- Use existing DRX Command MAC CE and Long DRX Command MAC CE

- When UE is not in C-DRX active time, UE monitors LP-WUS

Send LS to RAN2 to ask the feasibility of the above potential solution. LS in R1-2410909.

Conclusion

For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group

- When UE is not in C-DRX active time, UE monitors LP-WUS

Send LS to RAN2 to ask whether RAN2 has same understanding or not. LS in R1-2410909.

Agreement

For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

- If the parameter is NOT configured:

  - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

  - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

- If the parameter is configured:

  - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time.

  - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

- FFS: Whether RAN1 spec impact is required.

For future meetings:

Companies are encouraged to consider [Fri]Combined Proposal 3.2-1/3.2-2c of R1-2410906 for further discussions in future meetings.

Consider the following alternatives for UE capability report on the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring:

- Alt 1: For the X = [2, 3, 4] candidate values for the UE capability report on the minimum time gap, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20}ms as starting point

  - Note: Other values are not precluded

  - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR

  - FFS whether/how to consider the case of CA

Alt 2: For the UE capability report on the minimum time gap, UE reports a pair of values for main radio ramp up and the number of SSBs/TRSs needed for synchronization (D, NSSB/NTRS). This may require RAN4 coordination

Alt 3: UE capability report on the minimum time gap includes the main radio ramp up time. The number of SSBs/TRSs needed for synchronization is defined in specs, and different number of SSBs/TRSs can be defined for different UE capability on the main radio ramp up time. This may require RAN4 coordination

Alt 4: UE capability report on the minimum time gap includes the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

FFS: For the case of UAI

## RAN2 agreements (selection)

On DRX command and short DRX

For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used.

RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer.

RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.