3GPP TSG-RAN WG2 Meeting#130		R2-2503372

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	vivo

Title:	Configuration and report of inference input for NW-sided model

Agenda Item:	8.3.3

Document for:	Discussion and Decision

- Introduction

At the RAN2#129-bis meeting, the following agreement was initially reached regarding the inference configuration and reporting:

Agreements

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

In this contribution, focusing on the NW-sided model of AI/ML mobility, we will further discuss the configuration and report of inference input thereinafter.

- Discussion

  - General principle

For the progress on AI/ML based beam management, RAN2 has concluded that the existing L1 measurement configuration and reporting framework can be used to configure and report the input data for inference of the NW-sided model [1][2].

Agreements@RAN2#125bis meeting

- For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference. FFS if further enhancements are needed

Agreements@RAN2#126 meeting

- For the network-side model, required network side additional condition is left up to the network implementation.

- RAN2 will wait for RAN1 for any required UE side additional conditions.

- For network-sided model for BM use case, RAN2 confirms that UE inputs for inference at network-sided model will rely on L1 signaling, RAN2 will not further spend time on this aspect.

Inspired by the above conclusions, we believe that it is similar to reusing the existing RRM measurement configuration and reporting framework to configure and report the input data for the inference of NW-sided model.

- For NW-sided model of RRM measurement prediction, the existing RRM measurement framework can be reused as a baseline for configuration and reporting of the inference input.

As for the reporting scheme, the current RRM measurement framework can support both periodic and event-triggered reporting. When it comes to the inference input of NW-sided model, NW may still intend to have the flexibility on the timing to obtain the measurement predictions. For example, when NW would like to obtain the prediction periodically, it can configure UE with periodic reporting. Or when NW only wants the prediction at specific event occurrence, the event-triggered reporting can be employed. Therefore, both reporting types are useful in certain scenarios and need to be supported.

- Both periodic and event-triggered reporting schemes can be configured for UE to report inference input for NW-sided model of RRM measurement prediction.

  - Specific analysis

Table 2.2-1 shows all the use cases to be further studied and specified. According to the agreed use cases with NW-sided model, we further analyze the details to be enhanced.

Table 2.2-1: Use cases for specification study

|  | UE-sided model | Network-sided model |
| --- | --- | --- |
| Measurement event prediction | Yes | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes , all sub cases | Yes |
| Frequency domain | Yes , all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes | Yes |

2.2.1 Different sub-use cases

In our previous simulation [3], for temporal domain prediction Case B, if the model input is L1 samples (without filtering), the prediction accuracy is quite close to the non-AI approach; if the model input is L1-filtered RSRP, the prediction accuracy is better than the non-AI approach. Further, the L1 filtering is up to UE implementation. Therefore, we propose:

- Consider L1-filtered RSRP as a candidate inference input for the NW-sided model, while the L1 sample (without filtering) is not considered.

According to the understanding of model input for different use cases, we further illustrate all types of inputs if all sub-use cases are to be supported.

Table 2.2-2: Inference input of sub-use cases

| Use case | Sub-use case | Input of Temporal domain prediction | Input of Frequency domain prediction | Input of Spatial domain prediction |
| --- | --- | --- | --- | --- |
| Cell-level prediction | 1 | Multiple instances of L1-filtered beam-level RSRP | L1-filtered beam-level RSRP of Frequency 1 | Set B of L1-filtered beam-level RSRP |
|  | 2 | Multiple instances of L3 Cell-level RSRP | L3 Cell-level RSRP of Frequency 1 | - |
|  | 3 | Multiple instances of L1-filtered beam-level RSRP | L1-filtered beam-level RSRP of Frequency 1 | Set B of L1-filtered beam-level RSRP |
| Beam-level prediction | 4 | Multiple instances of L1-filtered beam-level RSRP | L1-filtered beam-level RSRP of Frequency 1 | Set B of L1-filtered beam-level RSRP |
|  | 5 | Multiple instances of L3 beam-level RSRP | L3 beam-level RSRP of Frequency 1 | - |
|  | 6 | Multiple instances of L1-filtered beam-level RSRP | L1-filtered beam-level RSRP of Frequency 1 | Set B of L1-filtered beam-level RSRP |

- UE can be configured to report the following objects as the contents of inference inputs for NW-sided model:

- For temporal domain prediction: a list of L1-filtered beam-level RSRP, L3 beam-level RSRP, and L3 cell-level RSRP of one cell at multiple time instances;

- For spatial domain prediction: a list of L1-filtered beam-level RSRP of one cell;

- For frequency domain prediction: a list of L1-filtered beam-level RSRP, L3 beam-level RSRP and L3 cell-level RSRP of one or more cells on specific frequency(s).

In reference to TS 38.331, NR supports measurement result of L3 beam-level and cell-level, which corresponds to the blue font part of the above Table 2.2-2. That is, to support sub-use cases 2 and 5 for NW-sided model, UE can be configured to report L3 beam-level per cell and L3 cell-level measurement results.

physCellId PhysCellId OPTIONAL, measResult SEQUENCE { cellResults SEQUENCE{ resultsSSB-Cell MeasQuantityResults OPTIONAL, resultsCSI-RS-Cell MeasQuantityResults OPTIONAL }, rsIndexResults SEQUENCE{ resultsSSB-Indexes ResultsPerSSB-IndexList OPTIONAL, resultsCSI-RS-Indexes ResultsPerCSI-RS-IndexList OPTIONAL } OPTIONAL },

In this sense, to support sub-use cases 1, 3, 4 or 6 for NW-sided model, it would require additional measurement reporting types in RRM measurement report, i.e., L1-filtered beam-level measurement result.

- To support sub-use cases 2 and 5 for NW-sided model, UE can be configured to report L3 beam-level and L3 cell-level measurement results per cell.

- To support sub-use cases 1, 3, 4 or 6 for NW-sided model, UE can be configured to report L1-filtered beam-level measurement results.

2.2.2 Different prediction domains

Specifically speaking, in the current NR RRM measurement results, only one measurement instance is reported at one time from UE to NW. It can be assumed that the reasonable UE behavior is to report the measurement result at the moment when it obtains the result according to the measurement report configuration.

- In the current RRM measurement report, one measurement result corresponds to the results measured at one time instance.

As mentioned above, for NW-sided model, the inference input can be multiple instances of RRM measurement results, which needs to be imported by the model and to process with the final inference output. Furthermore, these instances must be of certain relationships to satisfy the AI functionality. On the contrary, if UE is not able to report the inference inputs of multiple instances in one measurement report, it strictly restricts NW to configure the report periodicity according to the measured RS periodicity. Only in this way, the network ensures the exact time information of each input instances and therefore obtain the corresponding future output with it corresponding time. To make NW configuration more flexible, reasonable for deployment and time-saving, we think it is an optimal enhancement to enable multiple instances of RRM measurement in one measurement report.

- At least for temporal domain prediction, support reporting RRM measurement results of one cell at multiple time instances in one measurement report as the inference input of NW-sided model.

Furthermore, based on the prediction toward temporal, frequency or spatial domain, the inference input can be further categorized to include multiple instances of different measuring occasions, of a specific frequency, of specific beams.

Since the current measurement result is reported per MeasID, which is associated with one measurement object configured on a specific frequency, leading the report per frequency has been realized. For the report of specific beam combinations, it can be supported by NW configuring specific RS indexes in the corresponding measurement object. That is to say, from UE point of view, it cannot know if the measurement reporting is related to AI/ML prediction at NW side. It only sees the corresponding configuration for inference input as any other measurement configuration. And the network can configure the wanted inference input as what the UE should measure.

- NW configures the specific inference inputs as measurement object, where UE is agnostic about the NW-sided model.

In this sense, the left issue is to support multiple instances of different measuring occasions in one measurement report. Regarding how to determine these measuring occasions, and the exact format of reporting of these multiple instances, we believe this can be discussed and concluded in WI phase.

- For the inference input of NW-sided model, RAN2 to discuss how to configure the UE to report measurement results at multiple time instances in one report in WI Phase.

- Conclusion

In the contribution, we have the following observations and proposals:

General principle

- For NW-sided model of RRM measurement prediction, the existing RRM measurement framework can be reused as a baseline for configuration and reporting of the inference input.

- Both periodic and event-triggered reporting schemes can be configured for UE to report inference input for NW-sided model of RRM measurement prediction.

Use cases

- Consider L1-filtered RSRP as a candidate inference input for the NW-sided model, while the L1 sample (without filtering) is not considered.

- UE can be configured to report the following objects as the contents of inference inputs for NW-sided model:

- For temporal domain prediction: a list of L1-filtered beam-level RSRP, L3 beam-level RSRP, and L3 cell-level RSRP of one cell at multiple time instances;

- For spatial domain prediction: a list of L1-filtered beam-level RSRP of one cell;

- For frequency domain prediction: a list of L1-filtered beam-level RSRP, L3 beam-level RSRP and L3 cell-level RSRP of one or more cells on specific frequency(s).

- To support sub-use cases 2 and 5 for NW-sided model, UE can be configured to report L3 beam-level and L3 cell-level measurement results per cell.

- To support sub-use cases 1, 3, 4 or 6 for NW-sided model, UE can be configured to report L1-filtered beam-level measurement results.

Prediction domains

- In the current RRM measurement report, one measurement result corresponds to the results measured at one time instance.

- NW configures the specific inference inputs as measurement object, where UE is agnostic about the NW-sided model.

- At least for temporal domain prediction, support reporting RRM measurement results of one cell at multiple time instances in one measurement report as the inference input of NW-sided model.

- For the inference input of NW-sided model, RAN2 to discuss how to configure the UE to report measurement results at multiple time instances in one report in WI Phase.

- Reference

- Report of 3GPP TSG RAN WG2 meeting#125bis

- Report of 3GPP TSG RAN WG2 meeting#126

- R2-2409667	Updated simulation results for RRM measurement prediction