3GPP TSG RAN WG2#130                                                                   R2-2503524

St Julian’s, Malta, 19th - 23rd May 2025

Agenda Item:	8.12.2

Source:	Ofinno

Title:	Enhancements for Asymmetric DL sTRP and UL mTRP

Document for:	Discussion and Decision

# Introduction

The WID [1] of NR MIMO evolution Phase 5 was agreed in RAN#102 meeting. The objectives related to this agenda item are listed below:

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this contribution, we present our view on the enhancement of asymmetric DL sTRP and UL mTRP scenario.

# Open issues

## MAC-3: Asymmetric UL mTRP in LTM cell switch

RAN1 and RAN2 are considering the use of two TAs for asymmetric DL sTRP/UL mTRP in both cases i) with and ii) without PL offset. When the PL offset is not configured, the UL TRP(s) may transmit SSBs.

In the absence of PL offset configuration, all Rel-18 2TA procedures remain applicable, and the current LTM cell switch procedure can also be reused. There is no need to introduce PL offset signaling in the LTM cell

switch MAC CE, as the UE can default to transmitting to and receiving from the anchor TRP. Support for two TAs in Rel-19 asymmetric DL sTRP/UL mTRP should therefore be enabled for LTM in the same manner as it is for Rel-18 multi-DCI mTRP.

- (MAC-3) Support asymmetric DL sTRP and UL mTRP with no PL offset (i.e. the UL TRP may transmit SSB) for the LTM target cell (i.e., similar to Rel-18 multi-DCI mTRP and no MAC spec impact).

Rel-18 2TA operation with mDCI mTRP is supported for LTM cell switch, where the TA included in the LTM cell switch command is applied to the TAG configured for the TCI state indicated in LTM cell switch command. Rel-19 2TA with sDCI mTRP operation can be applied in the same manner.

- (MAC-3) Support sDCI mTRP 2TA operation for intra-DU LTM, in the same way as Rel-18 mDCI mTRP 2TA, i.e., the TA included in the LTM cell switch command is applied to the TAG configured for the indicated TCI state in LTM cell switch command.

Asymmetric UL mTRP is scoped as intra-DU in the Work Item. In our view, the PL offset can be obtained and preconfigured in advance for intra-DU scenarios, particularly when the UL-only TRP is associated with an additional PCI.

For RACH-less LTM, the UE accesses the target cell using a configured grant. The UE selects the configured grant occasion associated with the TCI state indicated in the LTM cell switch command. If the indicated TCI state is configured with a PL offset, the UE applies it for the first PUSCH transmission in the RACH-less LTM cell switch.

In addition, in Rel-19 asymmetric DL sTRP/UL mTRP scenarios, the PL offset configured for a TCI state can be updated using MAC CE (e.g., Pathloss Offset Update MAC CE). If a PL offset is not pre-configured in the LTM configuration for the target cell, or if one is configured and the network intends to update it, the new PL offset can be included in the LTM cell switch command.

- (MAC-3) Regarding PL offset in LTM, the PL offset configured in the LTM configuration for the indicated TCI state in the LTM cell switch command is applied for the first PUSCH transmission in RACH-less LTM cell switch.

- The PL offset can be indicated in the LTM cell switch command, e.g., under two cases below:

  - Case 1: When PL offset is not provided in LTM configuration for the target cell.

  - Case 2: When PL offset is provided in LTM configuration for the target cell and the NW wants to update PL offset.

## MAC-4: TCI state activation MAC CE for asym. DL sTRP/UL mTRP

For the asymmetric DL sTRP/UL mTRP deployment scenario, RAN1 agreed to reuse the Rel-18 unified TCI framework:

Agreement - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

The Rel-18 activation/deactivation MAC CE supports configurations with up to two DL and two UL TCI states. This configuration is sufficient to cover the asymmetric DL sTRP/UL mTRP scenario, where only one DL TCI state and up to two UL TCI states are used. As a result, the existing Rel-18 MAC CE format can be reused for both joint and separate TCI modes.

In joint TCI mode, there is no issue. However, in separate TCI mode, especially for UL-only TRPs (where no DL TCI state exists), the corresponding Fi,j , j = 2 field can be a reserved field (e.g., set to be 0), and the UE should ignore this Fi,j field.

- (MAC-4) Reuse the Rel-18 TCI State activation/deactivation MAC CE for joint/separate modes for the asymmetric mTRP scenario.

- When separate TCI mode is used in the asymmetric mTRP scenario, the Fi,j fields, j = 2 in the TCI activation/deactivation MAC CE are reserved (e.g., set to 0) and the UE ignores these Fi,j fields.

# Remaining issues

## Expiry of Time alignment timer

In RAN2#123b, the following agreement has been reached regarding the clearing of configured uplink grants and PUSCH resources for semi-persistent CSI reporting upon expiry of TAT associated with one of the two TAGs of a serving cell (e.g., SCell, SpCell).

RAN2#123 Working assumption: We will use the 2-PTAG model, i.e., both TAGs of SpCell are PTAGs; - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired, - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 7 are applied to the TRP with TAT expired, FFS whether 2-6 are applied to the TRP with TAT expired. 1. not perform any uplink transmission except the Random Access Preamble and MSGA transmission; 2. flush all HARQ buffers; 3. notify RRC to release PUCCH, if configured; 4. notify RRC to release SRS, if configured; 5. clear any configured downlink assignments and configured uplink grants; 6. clear any PUSCH resource for semi-persistent CSI reporting; 7. maintain NTA (defined in TS 38.211 [8]) of this TAG; 8. consider all running timeAlignmentTimers as expired. RAN2#123bis The following are taken as baseline - When the TAT for STAG is expired and the other TAT is running for a serving cell (i.e., SCell), no impact to the TRP with running TAT; 1 and 3-7 are applied to the TRP with TAT expired, i.e., 2 is not applied. - when the TAT for PTAG is expired and the other TAT is running for a serving cell (SpCell or SCell), no impact to the TRP with running TAT; 1 and 3-7 are applied to the TRP with TAT expired, i.e., 2 is not applied.

In our view, RAN1 does not restrict the application of Rel-18 multi-TRP uplink schemes, such as Multi-TRP PUSCH/PUCCH repetitions and simultaneous multiple-panel PUSCH/PUCCH transmissions, in asymmetric DL sTRP/UL mTRP scenarios, which are based on single-DCI multi-TRP configurations. That is, the UE may transmit repetitions of a PUSCH or PUCCH transmission to both the anchor TRP and the UL-only TRP, or it may perform simultaneous PUSCH/PUCCH transmission using two panels (e.g., via multipanelSchemeSDM or multipanelSchemeSFN) targeting both TRPs (shown in Figure 1 below).

- : STxMP schemes

According to the RAN2#123b agreement, under the multi-DCI multi-TRP scheme, if the TAT of one TAG expires while another TAG's TAT is running, the UE clears any configured uplink grant and any PUSCH resource for semi-persistent CSI reporting associated with the expired TAG. However, in the single-DCI multi-TRP scheme, a configured uplink grant and a PUSCH resource for semi-persistent CSI reporting may be associated with both TAGs (e.g., when applyIndicatedTCI-State is set to ‘both’, or when the SRS resource set indicator in the DCI activating the semi-persistent CSI reporting is set to ‘10’ or ‘11’).

In such cases, it may be inefficient or unnecessary for the UE to clear the configured uplink grant or PUSCH reporting resources associated with the expired TAG, particularly if the other TAG remains valid. The UE should clear the configured uplink grant or PUSCH reporting resources if TAT of both TAGs are expired.

- When the TAT for a TAG is expired and the other TAT is running for a serving cell (i.e., SCell, SpCell), the UE does not clear any configured uplink grant or PUSCH resource for semi-persistent CSI reporting which is associated with both TAGs.

# Conclusion

In this contribution, we make the following proposals.

- (MAC-3) Support asymmetric DL sTRP and UL mTRP with no PL offset (i.e. the UL TRP may transmit SSB) for the LTM target cell (i.e., similar to Rel-18 multi-DCI mTRP and no MAC spec impact).

- (MAC-3) Support sDCI mTRP 2TA operation for intra-DU LTM, in the same way as Rel-18 mDCI mTRP 2TA, i.e., the TA included in the LTM cell switch command is applied to the TAG configured for the indicated TCI state in LTM cell switch command.

- (MAC-3) Regarding PL offset in LTM, the PL offset configured in the LTM configuration for the indicated TCI state in the LTM cell switch command is applied for the first PUSCH transmission in RACH-less LTM cell switch.

- The PL offset can be indicated in the LTM cell switch command, e.g., under two cases below:

  - Case 1: When PL offset is not provided in LTM configuration for the target cell.

  - Case 2: When PL offset is provided in LTM configuration for the target cell and the NW wants to update PL offset.

- (MAC-4) Reuse the Rel-18 TCI State activation/deactivation MAC CE for joint/separate modes for the asymmetric mTRP scenario.

- When separate TCI mode is used in the asymmetric mTRP scenario, the Fi,j fields, j = 2 in the TCI activation/deactivation MAC CE are reserved (e.g., set to 0) and the UE ignores these Fi,j fields.

- When the TAT for a TAG is expired and the other TAT is running for a serving cell (i.e., SCell, SpCell), the UE does not clear any configured uplink grant or PUSCH resource for semi-persistent CSI reporting which is associated with both TAGs.

# References

- RP-242394, WID revision: NR MIMO Phase 5, Samsung, RAN#105, Sept. 2024