3GPP TSG-RAN WG2 Meeting #130	R2-2503555

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.6.4

Source: 	Fujitsu

Title:	Discussion on conditional Intra-CU LTM

Document for:	Discussion and Decision

- Introduction

At previous RAN2 meetings, some agreements on conditional Intra-CU LTM are reached. However, there are still some essential remaining open issues.

This contribution will discuss the following issues:

- Coexistence with network triggered LTM cell switch

- L3-based CLTM

- CLTM cell switch execution when both L1 and L3 conditions are fulfilled

- Discussions

Coexistence with network triggered LTM cell switch

RAN2 has agreed that the network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration [1]. So, TA value for a CLTM candidate cell can be provided to the UE via early TAC MAC CE, the LTM Cell Switch Command MAC CE and measured by the UE.

Regarding the TA value, the following cases need to be considered:

- Case 1: early TAC MAC CE is received before LTM Cell Switch Command MAC CE

  - Case 1-1: TA provided by the early TAC MAC CE is still valid while valid TA is included in the LTM Cell Switch Command MAC CE

  - Case 1-2: TA provided by the early TAC MAC CE is still valid, but invalid TA (TA = FFF) is included in the LTM Cell Switch Command MAC CE

  - Case 1-3: TA provided by the early TAC MAC CE is invalid while valid TA is included in the LTM Cell Switch Command MAC CE

  - Case 1-4: TA provided by the early TAC MAC CE is invalid while invalid TA (TA = FFF) is included in the LTM Cell Switch Command MAC CE

- Case 2: UE has no stored TA, and preamble is transmitted but no early TAC MAC CE is received before LTM Cell Switch Command MAC CE

  - For this case, there is no impact to the UE, because the network will not transmit the early TAC MAC CE. For example, source DU can wait for the TA value from target DU and include the valid TA in the LTM Cell Switch Command MAC CE rather than early TAC MAC CE. If invalid TA is included in the LTM Cell Switch Command MAC CE, it is not expected for the UE to receive TA value by early TAC MAC CE.

For Case 1, more discussions are needed. We think that valid TA included in the LTM Cell Switch Command MAC CE should be applied for the target cell considering the network may include the updated TA value. This means that TA in the LTM Cell Switch Command MAC CE is applied for Case 1-1 and Case 1-3. For Case 1-4, both TAs provided by the network are invalid and thus UE measured TA is used or RACH-based LTM is performed.

Case 1-2 is possible. For example, the UE received the TA included in the early TAC MAC CE before receiving LTM Cell Switch Command MAC CE. Considering the TA timer maintained at the network for the target cell, the network may not include the same TA value in the LTM Cell Switch Command MAC CE to avoid potential UL interference. According to Rel-18 mechanism, if invalid TA is provided in the LTM Cell Switch Command MAC CE, UE measured TA is used or RACH-based LTM is performed. However, if UE measured TA is used, then it means that UE measured TA is prioritized over the TA provided by the network, which is not aligned with Rel-18 principles. If RACH-based LTM is performed, larger interruption is incurred.

Based on this, we propose to use valid TA provided in the early TAC MAC CE if invalid TA is indicated in the LTM Cell Switch Command MAC CE. Whether valid TA in the early TAC MAC CE is prioritized over UE measure TA needs more discussion.

Proposal 1: When the LTM CSC MAC CE indicating a CLTM candidate and its TA to FFF, the stored TA provided by the early TAC MAC CE can be used if the corresponding early TAT is running.

RAN2 has agreed that the UE maintains the TA value and keeps the early TA timer running for other candidate cells when CLTM cell switch is executed and CLTM TAT for the target cell is not stopped if it is running. The UE starts the PTAT using the remaining time for the CLTM timer. If valid TA provided in the early TAC MAC CE is used for LTM cell switch execution triggered by the network, same UE behaviours will apply.

Proposal 2: Same UE behaviours will apply to the PTAT, early TA timers for the target cell and candidate cells if the TA provided by the early TAC MAC CE is used for the LTM cell switch triggered by the network.

L3-based CLTM

For L3-based CLTM, if RACH-based LTM is performed, legacy mechanism can be used to determine the target beam. If CLTM cell switch is triggered by L3 measurement and there is valid TA for the target cell, RACH-less LTM is performed. Valid TA can be provided by the early TAC MAC CE. In this case, it is better to include the beam information in the early TAC MAC CE as well and thus the UE can use the indicated beam as the target beam.

Proposal 3: Beam information is indicated in the early TAC MAC CE to support L3 based RACH-less CLTM.

Proposal 3bis: Beam information in the early TAC MAC CE takes LTM CSC MAC CE as baseline.

After beam determination, the next issue is to determine CG occasion. One possibility is to provide CG information in the early TAC MAC CE, e.g. if less CG occasion is configured to reduce reserved resources.

Proposal 4: RAN2 to discuss whether to include CG information in the early TAC MAC CE.

CLTM cell switch execution when both L1 and L3 conditions are fulfilled

From a given source cell, the network may configure either L1 execution conditions or L3 execution conditions for a CLTM candidate cell. Also, there can be the case where conditions for CLTM cell switch towards multiple candidates are fulfilled. If L1 conditions for CLTM are fulfilled, it is up to MAC entity to select one candidate beam satisfying the L1 condition and to trigger CLTM, e.g. associated LTM configuration ID is indicated to upper layers to perform LTM cell switch execution. If L3 conditions are fulfilled, the legacy prioritization applies. However, there could be one case where L1 execution condition for one CLTM candidate cell is fulfilled and L3 execution conditions for another CLTM candidate cell are fulfilled at the same time. So, it needs to be further discussed how to handle the case where both L1 and L3 conditions are fulfilled when both conditions are configured.

When L1 condition is fulfilled, the MAC entity indicates the corresponding LTM configuration ID to upper layers and waits for RRC request. So, RRC has knowledge of CLTM trigger at RRC and CLTM trigger from MAC entity and thus RRC can prioritize CLTM triggered by L1 conditions or CLTM triggered by L3 conditions. Alternatively, the prioritization is up to UE implementation. It requires RAN2 discussions.

Proposal 5: RAN2 to discuss whether CLTM triggered by L1 conditions or by L3 conditions should be prioritized or it is up to UE implementation when L1 execution condition for one CLTM candidate cell is fulfilled and L3 execution conditions for another CLTM candidate cell are fulfilled at the same time.

- Conclusion

This document discussed the remaining open issues for Intra-CU CLTM and made the following proposals:

Coexistence with network triggered LTM cell switch

Proposal 1: When the LTM CSC MAC CE indicating a CLTM candidate and its TA to FFF, the stored TA provided by the early TAC MAC CE can be used if the corresponding early TAT is running.

Proposal 2: Same UE behaviours will apply to the PTAT, early TA timers for the target cell and candidate cells if the TA provided by the early TAC MAC CE is used for the LTM cell switch triggered by the network.

L3-based CLTM

Proposal 3: Beam information is indicated in the early TAC MAC CE to support L3 based RACH-less CLTM.

Proposal 3bis: Beam information in the early TAC MAC CE takes LTM CSC MAC CE as baseline.

Proposal 4: RAN2 to discuss whether to include CG information in the early TAC MAC CE.

CLTM cell switch execution when both L1 and L3 conditions are fulfilled

Proposal 5: RAN2 to discuss whether CLTM triggered by L1 conditions or by L3 conditions should be prioritized or it is up to UE implementation when L1 execution condition for one CLTM candidate cell is fulfilled and L3 execution conditions for another CLTM candidate cell are fulfilled at the same time.

# Reference

- R2-2502981, “Report from session on V2X/SL, R19 NES and MOB”, Vice Chairman (Samsung), RAN2#129bis