3GPP TSG-RAN WG2 Meeting #130	R2-2503598

St Julian’s, Malta, 19th – 23th May, 2025

Agenda Item:	8.9.2

Source: 	TCL

Title:  	Discussion on Store & Forward satellite operation

Document for:	Discussion and Decision

- Introduction

In Rel-19 IoT NTN WID [1], Store & Forward satellite operation is one of the features to be supported.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

This contribution would like to provide our considerations on this objective of Rel-19 IoT NTN.

- Discussion on Store & Forward satellite operation

During R2#129b, a cell reselection issue triggered by discussions on S&F Mode operation has been discussed [2]. This issue originated from SA2 agreed that the MME-configured satellite list is used by the UE assist with saving power and determining which satellites to receive MT data/signalling. According to the latest TS 23.401 which already implemented the agreed SA2 CR, although it says that how the UE behaves upon reception of this MME-configured satellite list, it was also captured that if the UE access a satellite that is not belonging to the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. However, based on the current cell (re)selection mechanism, the UE shall select the cell relying on frequency priority and RSRP/RSRQ. This means that UE has to camp/reselect to the corresponding cells meeting the RSRP/RSRQ conditions, irrespective of whether the cells are provided by satellites included in the MME-configured satellite list. That is to say, current cell reselection procedure leaves no such room for UE implementation to further consider the MME-configured satellite list as specified in SA2 procedure. The consequence, is the increasing chance for UE access failure, if the current cell reselection procedure is followed without considering the MME-configured satellite list.

Therefore, some companies propose that during cell reselection procedure, UE may prioritize frequency(ies)/cell(s) corresponding to the satellite(s) indicated in the MME-configured satellite list. We acknowledge the motivation behind this proposal. The existing cell reselection procedure relies solely on channel quality and frequency priority, with no additional factors considered. If an out-of-list satellite has higher frequency priority and superior channel quality, the UE may prioritize connecting to it, which could increase the probability of communication failures. However, we approach this modification with caution for the following reasons.

First, we believe SA2’s proposal targets optimizations specific to the Store-and-Forward (S&F) mode rather than a general-case enhancement. The required change to the cell reselection procedure—replacing frequency-priority-based rules with cell-priority-based criteria—would fundamentally alter a general-purpose process.

Second, while SA2’s intent in introducing the S&F monitoring list is to guide UE implementations toward minimizing connection failures, the relationship between out-of-list satellite connections and failure probability remains probabilistic and heavily dependent on implementation algorithms at both the MME and UE. For this mitigation to work effectively, the MME must ensure accurate and timely configuration of the S&F monitoring list and its delivery to UEs. Even with such configurations, connections to out-of-list satellites may not always result in failures; this remains a probability-based outcome.

Furthermore, the S&F monitoring list introduced by SA2 is an optimization specific to S&F mode, which remains functional even without the list. We believe SA2’s original intent was not to modify the existing cell reselection mechanism. Any changes to this mechanism would impact not only S&F mode but also general cell reselction procedures. Therefore, we do not recommend making significant alterations to the cell reselection process solely to address a corner-case scenario. At this stage, there is no justification for introducing cell-priority-related mechanisms into the cell reselection procedure unless additional scenarios are identified that necessitate such an approach.

- At this stage, no need to introduce any cell-priority-related mechanisms into the cell reselection procedure.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- At this stage, no need to introduce any cell-priority-related mechanisms into the cell reselection procedure.

4. Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek

[2] R2-2502053, Discussion on RAN2 impacts for the support of S&F operation, CATT