3GPP TSG-RAN WG2 Meeting #130	R2-2503621

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Discussion on RRM measurement gaps enhancements

Agenda Item:	8.7.3

Document for:	Discussion and Decision

- Introduction

In this paper, we will first discuss the UAI reporting mechanism related to the measurement gap occasions and then analyze the RRC-based measurement gap cancellation solution.

- Discussion

  - (Capability Open issue 1) UAI report of measurement gap occasions

According to the LS [1], RAN4 has agreed on including in UAI the information related to cancellable measurement gap occasions ratio.

- The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%

In the XR UE capability email discussion [2], open issue about whether RAN2 to define the UE capability for UAI for a ratio of gap occasions is proposed:

Company contributions can focus on following open issues:

- Open issue 1: the pre-requisite of enhanced delay status report

- Open issue 2: whether to have separate or joint UE capability for UE Tx and Rx operations for unnecessary retransmission avoidance.

- Open issue 3: whether RAN2 to define the UE capability for UAI for a ratio of gap occasions.

In our view, as descripted in the LS, RAN4 will not define UE behavior and the corresponding requirements based on UAI. Therefore, RAN2 should work to define the UE capability for this feature. The capability of UAI for a ratio of gap occasions can be defined as an optional per-UE capability. Additionally, if a UE supports UAI reporting of measurement gap occasions ratio, it should also indicate support of DCI-based measurement gap cancellation.

Proposal 1：(Capability Open issue 1) An optional per-UE capability with signaling (e.g. measOccasionRatio-r19)  is introduced to indicate the support of UAI reports related to measurement occasions ratio for measurement gap cancellation. A UE supporting this feature shall also indicate support for DCI-based measurement gap cancellation.

  - (RRC-5/RRC-6) UAI report of measurement gap occasions

The current RRC running CR has already captured the content of MG occasions in the UAI. However, when and how to trigger the UAI reporting is still FFS [3].

UEAssistanceInformation message

UEAssistanceInformation-v19xy-IEs ::= SEQUENCE {

measOccasionAssistance-r19             MeasOccasionAssistance-r19                                 OPTIONAL,

nonCriticalExtension                 SEQUENCE {}                                      OPTIONAL

}

MeasOccasionAssistance-r19 ::=                      SEQUENCE{

measOccasionRatio-r19                 ENUMERATED {0, 20, 40, 60}                                               OPTIONAL,

...

}

Issue5: FFS when the UE should trigger UAI for assistance information for measurement occasion

Issue6: FFS what are the configurations for controlling UAI reporting assistance information for measurement occasion ratio.

In our view, the existing UAI framework can be reused for the MG occasions assistance information reporting. The network can configure whether UE is allowed to report UAI and the network can configure a prohibit timer to avoid frequent reporting. As for the triggering condition, UAI with MG occasion ratio information is triggered if the UE did not transmit a UAI with MG occasion ratio information since it was configured, or if the recommended MG ratio occasion information has changed since the last transmission and when the prohibit timer is not running.

Proposal 2: (RRC-5/RRC-6) The existing UAI framework is reused for the MG occasions ratio assistance information reporting, including:

- Network can configure whether UE is allowed to report UAI and network can configure prohibit timer for the UAI reporting;

- UAI with MG occasion ratio information is triggered if the UE did not transmit a UAI with MG occasion ratio information since it was configured, or if the recommended MG ratio occasion information has changed since the last transmission when the prohibit timer is not running.

  - RRC-based MG skipping solution

From RAN2 perspective, we believe that the RRC-based solution offers advantages over the DCI-based solution in at least the following two scenarios:

- Scenario 1: When MG occasions overlap with data whose arrival can be predicted.

- Scenario 2: When CG scheduling is configured.

For scenario 1, given that XR traffic is quasi-periodic (e.g., with periodicity of 16.67ms with jitter), it is possible to predict the overlap of MG occasions with a significant portion of packets based on the traffic period, jitter range, and MG configuration. Therefore, it is more appropriate to employ an RRC-based MG skipping solution (i.e., RRC configures the MG skipping patterns) rather than a DCI-based solution for these packets. The RRC-based solution addresses situations where the overlap of MG occasions and data can be anticipated, whereas the DCI-based solution addresses situations where such overlapping cannot be predicted due to packet arrival jitter and changes in data burst size. The RRC and DCI-based solutions are complementary and could be jointly utilized for measurement gap skipping.

As for scenario 2, the CG scheduling method is a kind of main scheduling method for UL XR traffic, if CG is configured and DCI-based MG skipping solution is used, the network may need to indicate UE before the CG occasions to skip the MG every time with DCI which have no data to schedule, which is inefficient.

In RAN2#129 meeting, it was agreed that RAN2 will not work on semi-static MG skipping solutions unless requested by RAN4/RAN1.

- RAN2 will not work on semi-static MG skipping solutions unless requested by RAN4/RAN1

Though RAN4 has not reached any conclusion on the semi-static MG skipping solutions so far, considering the advantages of RRC-based solutions over the DCI-based solution from the above analysis, we believe that the RRC-based solution should be discussed in RAN2.

Proposal 3: RRC-based solution indicating the occasions where measurement gaps/restrictions should be canceled is supported.

- Conclusion

In this paper, we have discussed the UAI report mechanism of the measurement gap occasions and the RRC-based measurement cancellation solution. The following observations and proposals are made:

Proposal 1: (Capability Open issue 1) An optional per-UE capability with signaling (e.g. measOccasionRatio-r19)  is introduced to indicate the support of UAI reports related to measurement occasions ratio for measurement gap cancellation. A UE supporting this feature shall also indicate support for DCI-based measurement gap cancellation.

Proposal 2: (RRC-5/RRC-6) The existing UAI framework is reused for the MG occasions ratio assistance information reporting, including:

- Network can configure whether UE is allowed to report UAI and network can configure prohibit timer for the UAI reporting;

- UAI with MG occasion ratio information is triggered if the UE did not transmit a UAI with MG occasion ratio information since it was configured, or if the recommended MG ratio occasion information has changed since the last transmission when the prohibit timer is not running.

Proposal 3: RRC-based solution indicating the occasions where measurement gaps/restrictions should be canceled is supported.

- References

- R4-2504972, Reply LS on UE assistance information, RAN4#114bis.

- R2-2503438, Open issues of Rel-19 XR UE capabilities, Xiaomi.

- R2-2503788, Summary of [POST129bis][503][XR] RRC running CR (Huawei), Huawei, HiSilicon.