3GPP TSG-RAN WG2 Meeting #130                                                                    R2-2503659

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	CATT

Title:	Remaining issues on LP-WUS in IDLE and INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our considerations on some open issues of LP-WUS in IDLE and INACTIVE.

Discussion

(RRC-12) whether/how to enable/disable LP-WUS, e.g. by RRC/NAS

It is FFS whether/how to enable/disable LP-WUS, e.g. by RRC/NAS [1]. Some companies pointed that LP-WUS introduces additional latency. Indeed, LP-WUS introduces larger additional latency than PEI. Regarding the existing agreements on wake-up delay in RAN1, 900ms additional latency may be introduced when UE performs LP-WUS monitoring. However, paging cycle the UE monitors (i.e., i-DRX cycle of the UE) maybe 320ms. The additional latency introduced by LP-WUS is larger than i-DRX cycle of the UE in this case. Considering latency requirements vary among different UEs it is useful to enable/disable LP-WUS monitoring in IDLE/INACTIVE per UE according to latency requirements per UE. Therefore, it is proposed that:

Proposal 1: (RRC-12) Enabling/disabling LP-WUS monitoring in IDLE/INACTIVE per UE is supported.

We think there are three possible options to enable/disable LP-WUS monitoring in IDLE/INACTIVE per UE, including:

- Option 1: NW can enable/disable LP-WUS monitoring in IDLE/INACTIVE per UE via dedicated RRC message, e.g. RRCRelease message.

- Option 2: NW can enable/disable LP-WUS monitoring in IDLE/INACTIVE per UE via NAS message.

- Option 3: UE can determine whether to enable/disable LP-WUS monitoring in IDLE/INACTIVE by itself based on latency requirement.

Option 1 brings impact on RAN3, e.g. RAN node needs to notify CN whether to enable/disable LP-WUS monitoring for the UE when it releases the UE, and CN paging message or RAN paging message between NW nodes need to indicate whether to enable/disable LP-WUS monitoring for the UE.

With Option 2, NAS message needs to be enhanced which brings impact on SA2, and like Option 1 which brings impact on RAN3, CN paging message or RAN paging message between NW nodes need to indicate whether to enable/disable LP-WUS monitoring for one UE.

With option 3, considering the UE can be aware of its latency requirement based on UE specific i-DRX configured by NAS or RRC, it is possible for the UE to decide whether to enable/disable LP-WUS monitoring by itself. For example, if the offset between LO and PO for the UE is larger than i-DRX cycle of the UE, it means paging latency requirement cannot be fulfilled with LP-WUS monitoring. Then the UE will not use LP-WUS. Correspondingly, NW will follow the same principle to decide whether to use LP-WUS for one UE based on i-DRX cycle of the UE and the offset between LO and PO. Considering Option 3 doesn’t introduce additional impacts on other groups, it is preferred.

Proposal 2: (RRC-12) RAN2 consider UE to determine whether to enable/disable LP-WUS monitoring in IDLE/INACTIVE based on latency requirement (i.e., i-DRX cycle of the UE).

X in UE_ID based subgrouping

In RAN2#129bis meeting, the following agreements were achieved:

LP-WUS is supported with eDRX, FFS on exact impact if any Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X. X is based on 32 subgrouping number. Details can be discussed in the running CR.

Therefore, the possible options for X include:

- Option 1: X is dependent on if eDRX is applied and different cases of Np.

If eDRX is not applied and Np is the number of subgroupsNumForUEID for PEI, X=262144 (256*4*8*32);

If eDRX is not applied and Np is 1, X=32768 (256*4*32);

If eDRX is applied and Np is the number of subgroupsNumForUEID for PEI, X=1048576(1024*4*8*32);

If eDRX is applied and Np is 1, X=131072(1024*4*32).

- Option 2: X is dependent on if eDRX is applied without considering different cases of Np

If eDRX is not applied, X=262144 (256*4*8*32);

If eDRX is applied, X= 1048576 (1024*4*8*32).

- Option 3: X is 1048576, i.e., the largest UE ID range in all LP_WUS cases is be used for all LP-WUS monitoring cases.

In our understanding, all above options are feasible. CN can be aware of whether eDRX is configured or not for a UE. But CN doesn’t have knowledge of UE_ID based subgrouping PEI in gNB. Hence, CN needs to provide at least 18bits Extended UE Identity Index Value to gNB for LP-WUS for non-eDRX case. Therefore, it is not necessary to distinguish whether PEI is configured in the cell. Obviously, additional complexity is introduced in Option 1.

For both paging and PEI, different values for eDRX and non-eDRX case are used for UE_ID. The same principle can be reused for LP-WUS. Hence, we slightly prefer option 2.

Proposal 3: X is dependent on if eDRX is applied without considering different cases of Np

- If eDRX is not applied, X=262144 (256*4*8*32).

- If eDRX is applied, X= 1048576 (1024*4*8*32).

Conclusion

According to the analysis in Section 2, we propose:

Proposal 1: (RRC-12) Enabling/disabling LP-WUS monitoring in IDLE/INACTIVE per UE is supported.

Proposal 2: (RRC-12) RAN2 consider UE to determine whether to enable/disable LP-WUS monitoring in IDLE/INACTIVE based on latency requirement (i.e., i-DRX cycle of the UE).

Proposal 3: X is dependent on if eDRX is applied without considering different cases of Np

- If eDRX is not applied, X=262144 (256*4*8*32).

- If eDRX is applied, X= 1048576 (1024*4*8*32).

Reference

- R2-2503765, Discussion summary and list of RRC open issue for LP-WUS WUR