3GPP TSG-RAN WG2 Meeting #130	           R2-2503669

St.Julians, Malta, May 19th – May 23rd, 2025

Agenda item:	8.10.3

Source:	China Telecom

Title:	MRO for intra-NTN mobility

Document for:	Discussion

- Introduction

In this paper, we provide our suggestion on the MRO enhancement option for location-based CHO in NTN in the LS R2-2503317 sent from RAN3.

- Discussion

RAN3 want to check with RAN2 which MRO enhancement option for location-based CHO in NTN for earth moving scenario to be adopted in the RLF report:

-	Option 1: to introduce the absolute UTC time when RLF/HOF happens, based on which the network computes the distance.

-	Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens.

RAN3 agreed to select one from option 1 and option 2 to resolve the accuracy and complexity issue of the network-based solution for earth moving case. From our point of view, for the UE configured with condEvent D2 (i.e. Distance between UE and the serving cell moving reference location is above threshold1 and distance between UE and a moving reference location is below threshold2), it is natural that the UE need to measure the distance from the UE to moving reference location of serving cell and the distance from the UE to moving reference location of candidate cell. Thus it is straightforward to include measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF occurred in the RLF report (i.e. option 2), without additional effort. On the other hand, the network can obtain the distance without additional complexity. Thus we suggest option 2.

Proposal 1: Regarding MRO for CHO with a location-based trigger condition, include measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF occurred in the RLF report (i.e. option 2).

3	Conclusions

Based on the discussion above, we have the following proposal.

Proposal 1: Regarding MRO for CHO with a location-based trigger condition, include measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF occurred in the RLF report (i.e. option 2).