3GPP TSG-RAN WG2 #130	R2-2503686

St Julian, Malta, May 19th – 23rd, 2025

Agenda item:	   8.3.2

Source:		Lenovo

Title:                             Functionality Management for AI/ML Mobility

Document for:		Discussion and Decision

# Introduction

As instructed by the SID, the AI/ML for mobility SI will study the RRM measurement prediction and measurement event prediction. In this paper, we discuss essential functionality management aspects for AI/ML mobility, considering these use cases and the respective scenarios of spatial and temporal domain predictions.

# Discussion

In R19 AI/ML for NR air interface work item, the fundamental framework of AI/ML functionality has been discussed considering the following modules for the beam management and positioning use cases:

- Life Cycle Management (LCM)

  - LCM for NW-sided model

  - LCM for UE-sided model

- Data collection

  - Data collection for NW-sided model

  - Data collection for UE-sided model

The general aspects of LCM have focused on the procedures to enable AI/ML functionality and eventually provide an inference outcome for an intended use case or a sub-use case.  The procedures that were addressed include network configuration for AI/ML-related functionalities, determination of applicability of functionality, signaling mechanisms for applicability report, inference reporting from the UE, etc. The agreed AI/ML framework can be extended to other use cases, such as AI/ML for mobility: RRM measurement prediction and RRM event prediction.

The following definitions of supported functionality, applicable functionality, and activated functionality from the AI for Air interface work item serve as the baseline.

Agreements -  AI/ML for Air Interface (RAN2_127)

      - Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling).

      - Applicable functionalities refer to functionalities that the UE is ready to apply for inference.

      - Activated functionalities refer to functionalities already enabled for performing inference.

To apply AI/ML functionalities for inference, the respective configuration and thus the functionalities must be applicable from both the network and UE perspectives. Therefore, determining applicability is a crucial step for the smooth functioning of all AI/ML features. Upon receiving the set of parameters for inference or the full inference configuration from the network, the UE is expected to report the applicability of the configured functionalities. It has been agreed in the last RAN2 meeting that, for AI/ML Mobility, UE reports the applicability via RRCReconfigurationComplete and/or UAI message.

Agreements – AI/ML for Mobility (RAN2_129bis)

      - For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

This agreement can be further detailed as follows (aligned with AI for Air Interface):

Proposal 1: UE can report explicit applicability/inapplicability in the initial applicability report via RRCReconfigurationComplete and subsequently only report the change in applicability via UAI.

How UE can determine the applicability of an AI/ML functionality has been intensively discussed in RAN2, and it has been concluded that the applicability status depends on the configuration, network-side additional conditions, UE-side additional conditions, and the model availability at the UE. The network-side additional conditions and UE-side additional conditions are essential to maintain the consistency in training and inference of a functionality. The UE-side additional conditions can be internally known to the UE, which do not require specification impact. Whereas, network-side additional conditions, if available, must be considered necessary to decide the applicability status of the functionalities/configurations by the UE. We believe this criterion for applicability determination should be common to AI/ML framework and is not limited to certain use cases. It is beneficial for different scenarios and conditions experienced by the network, which can also be extended beyond the serving cell. Following the progress of AI/ML for Air interface, additional conditions are represented by associated ID(s). Specially for the case of mobility, it is important to discuss the scope of associated ID(s) and their impact on the applicability of functionality when network-side additional conditions differ during UE mobility. Associated ID(s) beyond cell-specific ID(s) will be useful for an uninterrupted transition in inference during/after handovers, and for efficient LCM processes, such as monitoring model performance.

Proposal 2: Associated ID(s) (if available) are provided to UE for applicability determination. RAN2 should discuss the scope of associated ID(s) based on the following options:

- Per cell associated ID(s)

- Per area/group of cells associated ID(s)

## 2.1. RRM Measurement Prediction

The high-level categorization of RRM measurement prediction is done in two ways: prediction in the spatial domain and prediction in the temporal domain. At the same time, pure temporal prediction is studied as the baseline for measurement predictions.

The AI/ML for mobility SID suggests studying both UE-sided and NW-sided models for RRM measurement prediction. The two possible approaches that were studied in this SI are beam-level and cell-level predictions. As cell-level measurements are the focus of this study, they can be either generated or predicted based on the beam-level or cell-level inputs. RAN2 agreement lists three possible cases for cell-level RRM measurement prediction. It is important to discuss how LCM aspects apply to these different types of measurement prediction cases for NW-sided and UE-sided models.

For intra-cell measurement predictions with different cases, the final output will be predicted cell-level measurements, while the inputs will be different for types of measurement predictions (temporal/spatial). RAN2 is suggested to discuss whether temporal domain and spatial domain RRM measurement prediction can be applied and generate inference at the same time at the UE, as it can affect how the UE determines and reports applicability. This will be important to define the granularity of functionality.

Proposal 3: Temporal, spatial, and frequency domain predictions can be configured simultaneously.

The different cases of temporal and spatial domain prediction for inter/intra-cell scenarios have also been studied. The assumption is that a configuration from the network is always required, which is used for or in the process of measurement inference. However, the measurement configuration may be different for each case and depends on whether the cell-level measurements are predicted based on predicted beam-level results, cell-level results, or beam-level results of the same cell or a different cell. In some cases, it may be a configuration of a set of beams, as agreed in the R19 AI/ML air interface for BM. Based on the progress of LCM in AI/ML air interface and agreements from the last meeting, the legacy measurement configuration framework can be used as a baseline for inference configuration and reporting, which can be extended further, considering different cases of measurement prediction.

The network configuration will differ for the different cases of spatial and temporal domain RRM measurement prediction. UE can be configured with different periodic configurations for RRM measurement predictions, for example, periodic temporal domain inference configuration and periodic spatial domain inference configuration. Upon receiving inference configurations, UE will determine their applicability individually for each case, and thus, one or more periodic inference configurations can be applicable. For certain applicable configurations, UE can start inference immediately upon receiving the configuration and sending the applicability report; whereas, UE can be configured to trigger inference upon an event being fulfilled, e.g., based on radio conditions thresholds. It would be beneficial for RAN2 to discuss the UE behaviour in this case and activation for inference of such applicable inference configurations, and how multiple applicable configurations can be handled. Each set of inference configuration for the sub-cases of measurement prediction can be associated with an individual identifier (e.g., measId) such that independent measurement configurations for the respective cases/sub-cases can be activated and reported, if applicable.

Proposal 4: After receiving the inference configuration for RRM measurement prediction and reporting the applicable AI/ML functionalities, the inference can be triggered by UE immediately or upon a certain event.

Proposal 5: RAN2 clarifies if multiple periodic RRM measurement predictions (e.g., temporal/spatial predictions) can be activated at the same time.

Proposal 6: For RRM measurement prediction, one set of inference configuration can be identified with a measId.

In legacy, s-measureconfig is configured to UE, which controls when UE to performs measurement for a non-serving cell. This is intended for power savings since a UE located at the cell centre will not be handed over to a neighbour cell. RAN2 agreed to support intra-frequency, intra and inter-cell spatial domain measurement predictions for beam and cell level measurements. Similarly, when UE stays at the centre of one cell, UE is not expected to predict Inter-cell measurement results. One option is that legacy s-measureconfig can be used to control when the UE should perform measurement prediction for non-serving cells. Alternatively, one separate threshold can be configured for the UE to determine when the measurement predictions should be performed for the non-serving cell. For example, if the quality of the serving cell is less than the configured threshold, the measurement prediction for non-serving cells can be triggered.

Proposal 7: UE may perform measurement prediction for non-serving cells if the channel quality of the serving cell is worse than a configured threshold. FFS if the threshold is from legacy s-measureconfig IE or a separate threshold.

## 2.2. Measurement Event Prediction

The events A1-A5, which are utilized to evaluate the signal quality of serving and neighbour cells located at intra/inter frequency, are designed for mobility purposes. When one of the events is met, UE is triggered to report the measurement results. Once the network receives the measurement results, the network will decide whether to prepare a handover and which neighbour cell can be selected as the target cell. Event A3 was agreed as a baseline since it is most relevant for mobility.

In current specifications, the entering condition and leaving condition are specified for each event. When the entry condition of an event is met during TTT, the event is determined as satisfied. This implies UE enters the cell edge and consequently, the measurement report is triggered. When the leaving condition of an event is met during TTT, it means UE returns to the cell centre from the cell edge. UE is triggered to report the measurement result if reportOnLeave is configured. Similarly, the network can configure UE to predict whether the entering condition or the leaving condition is fulfilled. The prediction of the leaving condition could be helpful for the network to decide a suitable time instance for handover execution.

Proposal 8: After receiving the inference configuration for RRM event prediction, the inference can be triggered by UE immediately or upon a certain event.

Proposal 9: Network can configure UE to predict whether the entering condition or the leaving condition of a concerned RRM event will be met.

UE receives a configuration for event prediction. Meanwhile, a prediction window may be provided together with the configuration. If a prediction window is provided, UE is expected to predict whether an event occurs within the prediction window.

Proposal 10: The prediction window is configured for UE. If a prediction window is provided, UE is expected to predict whether an event occurs within the prediction window.

Similar to the case of RRM measurement prediction, network configuration (s-measureconfig) controls when UE performs measurement for a non-serving cell. It is also beneficial to consider when UE performs event prediction depending on UE position in the cell, the cell centre, or the cell edge. For this purpose, a condition can be configured to UE by the serving cell. For example, when the channel quality of the serving cell, e.g., PCell, is not good enough, UE starts measurement event prediction.

Proposal 11: UE may perform measurement event prediction based on a trigger condition, e.g., the channel quality of the serving cell becomes bad.

# 3	Conclusion

Based on the discussion above, we request RAN2 to discuss and agree on the following proposals:

Proposal 1: UE can report explicit applicability/inapplicability in the initial applicability report via RRCReconfigurationComplete and subsequently only report the change in applicability via UAI.

Proposal 2: Associated ID(s) (if available) are provided to UE for applicability determination. RAN2 should discuss the scope of associated ID(s) based on the following options:

- Per cell associated ID(s)

- Per area/group of cells associated ID(s)

RRM Measurement prediction

Proposal 3: Temporal, spatial, and frequency domain predictions can be configured simultaneously.

Proposal 4: After receiving the inference configuration for RRM measurement prediction and reporting the applicable AI/ML functionalities, the inference can be triggered by UE immediately or upon a certain event.

Proposal 5: RAN2 clarifies if multiple periodic RRM measurement predictions (e.g., temporal/spatial predictions) can be activated at the same time.

Proposal 6: For RRM measurement prediction, one set of inference configuration can be identified with a measId.

Proposal 7: UE may perform measurement prediction for non-serving cells if the channel quality of the serving cell is worse than a configured threshold. FFS, if the threshold is from legacy s-measureconfig IE or a separate threshold.

Measurement event prediction

Proposal 8: After receiving the inference configuration for RRM event prediction, the inference can be triggered by UE immediately or upon a certain event.

Proposal 9: Network can configure UE to predict whether the entering condition or the leaving condition of a concerned RRM event will be met.

Proposal 10: The prediction window is configured for UE. If a prediction window is provided, UE is expected to predict whether an event occurs within the prediction window.

Proposal 11: UE may perform measurement event prediction based on a trigger condition, e.g., the channel quality of the serving cell becomes bad.

# Reference

[1] RP-234055, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, RAN#102.

[2] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)