3GPP TSG-RAN WG2 Meeting #130	R2-2503694

Malta, May 19th – 23rd, 2025

Agenda Item:	8.3.4

Source:	InterDigital Inc.

Title:	Data collection for RRM measurement prediction

Document for:	Discussion

# 1. Introduction

In RAN2-129bis, initial discussion on LCM and the spec impact for the AI/ML for mobility use case were made and some agreements were made regarding data collection [6]:

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

In this contribution, we discussed some of the open issues regarding data collection for AI/ML mobility.

# 2. Discussion

### 2.1 Network side data collection

As shown in Annex 1, RAN2 has made several agreements regarding network side data collection for the BM case. In our understanding, apart from the content of the data being collected, the signaling aspect is quite similar in the AIML mobility use case as well (i.e., UE logs measurements according to a data collection configuration from the gNB and sends them on demand).

The content of the collected data depends on the RRM sub-use case, which are listed below:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated

- Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

- Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

Thus, based on the sub-use case, the data to be collected for the training of the network side model (i.e., input and output/ground-truth) can be one of the following:

  - L3 cell level measurements

  - L1 beam level measurements

  - L3 beam level measurements

Considering the agreements regarding the signaling for the BM case and the content of the collected data discussed above, we propose the following for the collection of data by the UE for the training of a network side model:

Proposal. 1: For the collection of data for the training of a network-sided model for RRM measurement prediction:

- UE can be configured to log, at a certain logging periodicity, one or more of the following:

  - L3 cell level measurements,

  - L3 beam level measurements,

  - L1 beam level measurements

- The UE configuration is provided via the RRM measurement framework. Required enhancements are FFS.

- The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity.

- The UE sends availability indication of collected data via UAI or RRCReconfigurationComplete message.

- The availability indication can be triggered due to:

  - full buffer being reached (if configured),

  - buffer threshold being reached (if configured), or

  - low power (if configured)

- Upon sending the availability indication, UE indicates:

  - Data is available

  - Reason for the triggering of the availability indication (full buffer. Threshold)

  - Low power indication

- The UE sends the collected data upon explicit/on-demand request from the network (using UEInformationRequest/Response signaling)

- The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., before/during HO).

- The UE releases the collected data upon transitioning to IDLE/INACTIVE.

### 2.2 UE side data collection

As can be seen in Annex 2, RAN2 has made some progress regarding the configuration aspects of the data collection for the training of the UE sided model for the BM use case. Specifically, RAN2 has agreed that UE can request the data collection configuration and may indicate the preferred configurations (E.g., among possible configurations provided by the network).

Proposal 2: For the collection of data for the training of a UE-sided model for RRM measurement prediction:

- the UE can request the network for a data collection configuration (e.g., via UAI)

- the network can provide data collection configuration to the UE with or without UE request.

- the details of the data collection configuration (e.g., any enhancements to MeasConfig) are FFS.

# 3. Conclusion

In this contribution, we discussed data collection for Network and UE sided models for RRM measurement prediction, and the following were proposed:

Proposal. 1: For the collection of data for the training of a network-sided model for RRM measurement prediction:

- UE can be configured to log, at a certain logging periodicity, one or more of the following:

  - L3 cell level measurements,

  - L3 beam level measurements,

  - L1 beam level measurements

- The UE configuration is provided via the RRM measurement framework. Required enhancements are FFS.

- The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity.

- The UE sends availability indication of collected data via UAI or RRCReconfigurationComplete message.

- The availability indication can be triggered due to:

  - full buffer being reached (if configured),

  - buffer threshold being reached (if configured), or

  - low power (if configured)

- Upon sending the availability indication, UE indicates:

  - Data is available

  - Reason for the triggering of the availability indication (full buffer. Threshold)

  - Low power indication

- The UE sends the collected data upon explicit/on-demand request from the network (using UEInformationRequest/Response signaling)

- The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., before/during HO).

- The UE releases the collected data upon transitioning to IDLE/INACTIVE.

Proposal 2: For the collection of data for the training of a UE-sided model for RRM measurement prediction:

- the UE can request the network for a data collection configuration (e.g., via UAI)

- The network can provide data collection configuration to the UE with or without UE request.

- The details of the data collection configuration (e.g., any enhancements to MeasConfig) are FFS.

# 4. References

[1]	RAN2-126 Chairman Notes, Fukuoka, Japan, May 2024

[2]	RAN2-127 Chairman Notes, Maastricht, The Netherlands, August 2024

[3]	RAN2-127bis Chairman Notes, Hefei, China, October 2024

[4]	RAN2-128 Chairman Notes, Orlando, USA, November 2024

[5]	RAN2-129 Chairman Notes, Athens, Greece, February 2025

[6]	RAN2-129bis Chairman Notes, Wuhan, China, April 2025

# 5. Annex1 (Agreements regarding network side data collection)

RAN2-126 agreements [1]:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting.

RAN2-127 agreements [2]:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network.

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event-based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

RAN2-127bis agreements [3]:

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicate the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

RAN2-128 agreements [4]:

Agreements on NW side data collection: - Focus on the following three radio condition event-based logging: - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

RAN2-129 agreements [5]:

Agreements on NW side data collection: - Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported.FFS what to log - Low power bit indication is supported - Data availability indication is supported. FFS when this would be triggered - As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT. - UE retains logged data during handover (HO). FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed - UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE). - FFS how to handle idle/inactive and RLF cases and whether we have a unified.

RAN2-129bis agreements [6]:

Agreements on NW side data collection: - Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. Source gNB decides whether the data should be kept. The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell). FFS signaling details. - Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS - NOTE: it is up to UE Implementation how buffer threshold reached and low power is - The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc) determined - For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit - New SRB can be configured for NW-side data collection (with lower priority)

# 6. Annex2 (Agreements regarding UE side data collection)

RAN2-129 agreements [5]:

Agreements on UE side data collection: - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - a. The network can decide when to start/stop the data collection and send configuration. - b. The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication)

RAN2-129bis agreements [6]:

Agreements on data collection configuration: - The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: - An indication on start/stop of data collection - Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.